3GPP TSG-RAN WG4 Meeting # 101-e 			R4-2119233

# Electronic Meeting, November 1-12, 2021

Source: 		Nokia, Nokia Shanghai Bell

Title: 			On 1900MHz RMR RAN4 BS RF requirements impact due to ECC Decision (20)02

Agenda Item: 	7.4.3

Document for:	Approval

# Introduction

During RAN4#100e meeting, a WF on BS RF requirements for RMR was agreed [1]. This document addresses the following part of the WF, in order to progress the work on BS RF requirements from ECC Decision (20)02:

Way Forward 1 (Sub topic 1-1: EIRP requirements conversion to conducted requirements):

For the next meeting, provided pros and cons analyses for the following options for the EIRP-based ECC requirements conversion to conducted requirements:

- Manufacturer declaration based approach,

- Fixed antenna gain approach.

Other options are not precluded for the analysis next meeting

# Discussion

As requested in the WF, below is the summary of pros and cons for manufacturer declaration vs. fixed antenna gain:

| Approach | Pros | Cons |
| --- | --- | --- |
| Manufacturer declaration (option A) | Allows for deployment flexibility, optimized for all considered scenarios while keeping conducted requirements not too restrictive for some (lower antenna gain) deployments. | Not preferred option for ETSI harmonized standard. |
| Fixed antenna gain (option B) | Preferred option for ETSI harmonized standard. | Reduced deployment flexibility, defined requirements may be too restrictive since fixed antenna gain needs to be defined as maximum antenna gain used for all scenarios. |

Furthermore, below is the summary of how relevant ECC Decision 20(02) requirements would need to be addressed in 3GPP specifications for both option A and B.

Table 1: General in-block requirement
mandatory for uncoordinated deployment

| RMR channel BW | Maximum e.i.r.p. |
| --- | --- |
| 10 MHz | = 65 dBm/10MHz (Note 1) |
| Note 1: In case an administration wishes to allow higher e.i.r.p., coordination or other mitigation measures are required. |  |

Option A: new declaration parameters would need to be introduced in 3GPP specifications

Option B: new conducted requirement would need to be added, taking into account 18dBi antenna gain

Table 2: Baseline requirement

| Frequency range | e.i.r.p. limit |
| --- | --- |
| 1920-1980 MHz | -43 dBm/5MHz |

Option A: new declaration parameters would need to be introduced in 3GPP specifications

Option B: new conducted requirement would need to be added, taking into account 18dBi antenna gain

Table 3: Requirements on wideband RMR BS receiver characteristics

| Parameter | Value |
| --- | --- |
| Level of the wanted signal | RefSens + 3 dB |
| Maximum 5 MHz LTE interfering signal in 1805-1880 MHz | -20 dBm |
| The antenna connector of the BS receiver is the reference point. The reference sensitivity (RefSens) is the minimum mean power received at the antenna connector at which a specified minimum performance shall be met. These requirements cover both blocking and third-order intermodulation. |  |

Option A: new blocking requirement would need to be added

Option B: new blocking requirement would need to be added

# Conclusion

This document provides the summary of pros and cons for manufacturer declaration vs. fixed antenna gain as well as how relevant ECC Decision 20(02) requirements would need to be addressed in 3GPP specifications for both options. These shall be taken into account for final 3GPP decision on which option is preferred for these requirements.

# References

[1] R4-2115637	WF on BS RF requirements for RMR900 and RMR1900; Huawei; RAN4#100-e