3GPP TSG-RAN WG4 Meeting #101-e	R4-2119954

Electronic Meeting, Nov 01-12, 2021

Source: 	OPPO

Title: 	WF on PC2 intra-band contiguous UL CA with UL MIMO

Agenda Item:	[101-e][119] NR_RF_FR1_enh_IntraHPUE

Document for:	Approval

Issue 3-1-2: MPR for PC2 intra-band UL CA with UL MIMO

Background: 1st round status in 101-e is as below.

| MPR | 1Tx MPR | 1Tx MPR + delta | considered as PC1.5 |
| --- | --- | --- | --- |
| 23+23 |  | delta op1: Sky, Intel, vivo delta op2: HW, LGE, Intel, vivo |  |
| 23+26 | Skyworks | Same as 23+23: vivo, LGE, Intel, HW, [OPPO] |  |
| 26+26 | Sky, LGE, Intel, HW | Same as 23+23: OPPO | vivo, HW, [QC] |

For 23+23dBm

- Option 1: A delta MPR compared to 1Tx MPR is specified as follows: - +0.5dB for CP-OFDM for: - contiguous inner and outer allocations - non-contiguous inner and outer 1 allocations - non-contiguous outer2 allocations MPR the same than for 1Tx - +1dB for DFT-s-OFDM for: - contiguous inner and outer allocations - non-contiguous inner and outer 1 allocations - non-contiguous outer2 allocations MPR the same than for 1Tx - Option 2: A delta MPR compared to 1Tx MPR is specified as follows: - +0.5dB for - contiguous outer allocations - non-contiguous inner allocations - +1dB for - non-contiguous outer 1 and outer 2 allocations - non-contiguous outer2 allocations MPR the same than for 1Tx

Agreement: for 23+23dBm delta MPR

|  | CP-OFDM | DFT-s-OFDM |
| --- | --- | --- |
| contiguous inner | +0.5 | +1 |
| contiguous outer | +0.5 | +1 |
| non-contiguous inner | +0.5 | +1 |
| non-contiguous outer 1 | +1 | +1 |
| non-contiguous outer 2 | +1 | +1 |

For 26+26dBm

- Option 1: - In release 17 the 1Tx MPR can be reused without additional MPR - In release 18 the MPR can be revised based on PC1.5 study or dedicated MPR - Option 2: One set of MPR requirements for all PC2 PA configurations. A delta MPR compared to 1Tx MPR is the same as 23+23

Agreed WF for 26+26dBm MPR:

- In release 17 the 1Tx MPR can be reused without additional MPR for both 1Tx and 2Tx operations, i.e., rank-1 and rank-2 operations.

For 23+26dBm:

- Option 1: One set of MPR requirements for all PC2 PA configurations. A delta MPR compared to 1Tx MPR is the same as 23+23 - Option 2: Different from 26+26 and 23+23

Agreed WF for 23+26dBm MPR:

- Depending on the capability signaling for 23+26dBm PC2 intra-band UL CA+UL-MIMO, either 23+23dBm MPR or 1Tx PC2 MPR will be applied

  - FFS the capability signaling

    - Option 1: reusing TxD

    - Option 2: new signaling

Issue 3-1-1: whether UE capability is needed to distinguish different PA configurations

Background: 1st round status in 101-e is as below.

| Signaling | TxD indication | None (1 set requirements) | New signaling |
| --- | --- | --- | --- |
| 23+23 | - TxD for 23+23: QC, HW, Intel | HW, LGE, vivo, [Ericsson] | Skyworks, OPPO, Intel |
| 23+26 |  |  |  |
| 26+26 | - New signaling: QC - Considered as PC1.5: HW, [QC] |  |  |

UE capability options

Below options are updated by moderator based on 1st round feedback.

- Option 1: TxD is used to distinguish 23+23 and 23+26 (QC, HW, Intel) - Option 2: New signaling to distinguish different PA configurations - Option 2a: new signaling to distinguish 23+23, 23+26, 26+26 (Skyworks, OPPO, Intel) - Option 2b: new signaling only to distinguish 26+26 and others (QC) - Option 3: One set of requirements for different PA configurations (HW, LGE, vivo, [Ericsson]) - [0.5-1]dB more back off based on 1Tx UL contiguous CA MPR

WF:

- FFS after the conclusion of 23+26 MPR and considering the following cases:

  - Case1: 23+26 follows 1Tx PC2 MPR

  - Case 2: 23+26 follows 23+23 MPR

Suffix H for CA with UL MIMO

Issue 3-2-1 correction of clauses of suffix H for CA with UL MIMO

- Proposal: To change suffix H general to CA with UL MIMO, which could include inter-band CA with UL MIMO, intra-band CA with UL MIMO, rather than for intra-band UL contiguous CA with UL MIMO only as in current spec.

Note: All feedback companies agree the proposal in 1st round.

WF:

- Agree the proposed correction of suffix H clauses.

Issue 3-2-2 Applicability of requirements in suffix H for CA with UL MIMO

Proposal: : If a UE supports the ULCA+MIMO feature, in addition to meeting the feature’s requirements (suffix ‘H’), it must meet the general requirements as well as the requirements for ULCA (suffix ‘A’) and the requirements for ULMIMO (suffix ‘D’).

WF:

- Agree the proposal while FFS on how to reduce test burden.