3GPP TSG-RAN4 Meeting #101-e	R4-2120269

# Electronic Meeting, 1 – 12 November, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | 1 | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to update positioning measurement requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2021-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - It is unclear if the definition of PRS resource overlapping with MG should account for the search window given by expected RSTD and expected RSTD uncertainty. - It was agreed in RAN4#100-e to “clarify that the requirements do not apply if PRS configuration exceeds any of the UE capabilities in NR-DL-PRS-ResourcesCapability, and it would be up to UE implementation which PRS resources are measured, subject to UE capabilities”, but this was not captured in the spec yet. - The wording “the least common multiple of … is used to derive the measurement period” is ambiguous because the LCM is used to derive Tprs,i instead of the measurement period. - The definition of Nmuting factor is conflicting with the latest RAN1 spec.. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that the definition of PRS resource overlapping with MG should account for the search window given by expected RSTD and expected RSTD uncertainty. - Capture the agreement that “the requirements do not apply if PRS configuration exceeds any of the UE capabilities in NR-DL-PRS-ResourcesCapability, and it would be up to UE implementation which PRS resources are measured, subject to UE capabilities”. - Clarify that “the least common multiple” is used to derive Tprs,i - Update the definition of parameter Nmuting by removing the upper bound, to align with RAN1 38.214. - Some wording and format correction to improve the readability. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE positioning measurement requirements are incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9.1, 9.9.2, 9.9.3, 9.9.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

9.9.1	Introduction

This clause contains requirements for UE capable of performing NR positioning measurements defined in TS 38.215 [4], including RSTD, PRS-RSRP, UE Rx-Tx time difference, and NR E-CID measurements.

For RSTD, PRS-RSRP and UE Rx-Tx time difference measurements, the requirements in clauses 9.9.2, 9.9.3 and 9.9.4 apply provided:

-	UE is configured with per-UE measurement gaps

-	No active BWP switching occurs during the measurement gaps for PRS measurement, and

All measurement requirements specified in clause 9.9.2, 9.9.3 and 9.9.4 shall apply without DRX as well as for any DRX configuration specified in TS 38.331 [2].

UE is not required to perform additional SSB measurement for the SSB configured as QCL source of PRS resources.

UE is only required to measure PRS resources that are fully or partially overlapped with measurement gaps, and the requirements in clause 9.9.2, 9.9.3 and 9.9.4 are applicable to PRS resources that are fully or partially overlapped with measurement gaps.

A PRS resource is considered to be fully (partially) overlapped with measurement gaps if all (some) of its instances are overlapped with a measurement gap occasion. A PRS resource instance is considered to be overlapped with measurement gap occasion if the minimum number of unmuted repetitions of the instance taking into account nr-DL-PRS-ExpectedRSTD-Uncertainty and nr-DL-PRS-ExpectedRSTD is fully covered by the MGL excluding RF switching time, where the minimum number is given in the accuracy requirements in clause 10.1.23, 10.1.24 and 10.1.25 for RSTD, PRS-RSRP and UE Rx-Tx time difference, respectively.

When UE is configured with measurement for more than one positioning requests, the measurement period for each request may be longer than measurement period when UE is configured with measurement for single positioning request.

9.9.2	RSTD measurements

9.9.2.1	Introduction

The requirements in clause 9.9.2 shall apply provided the UE has received NR-DL-TDOA-RequestLocationInformation message from LMF via LPP [34] requesting the UE to measure and report DL RSTD measurements defined in TS 38.215 [4].

9.9.2.2	Requirements Applicability

The requirements in clause 9.9.2 apply for periodic and triggered RSTD measurements, provided:

-	PRS-RSTD related side conditions given in clause 10.1.23 for FR1 and FR2 are fulfilled, for a corresponding Band.

9.9.2.3	Measurement Capability

UE PRS RSTD measurement capability is as indicated by the UE in NR-DL-TDOA-ProvideCapabilities, according to TS 37.355[34].

9.9.2.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the periodic measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other LPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report.

The reported RSTD measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clauses 10.1.23.3.

The RSTD measurements performed and reported according to this section shall meet the RSTD measurement accuracy requirements in clause 10.1.23, for each measured DL PRS resource.

9.9.2.4.1	Void

9.9.2.4.2	Void

9.9.2.4.3	Void

9.9.2.5	Measurements Period Requirements

When physical layer receives last of NR-TDOA-ProvideAssistanceData message and NR-TDOA-RequestLocationInformation message from LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 9.9.2.3) DL RSTD measurements, defined in TS 38.215 [4], during the measurement period TRSTD,Total defined as:

TRSTD,Total=i=1LTRSTD,i+ L-1*maxTeffect,i

Where ,

i is the index of positioning frequency layer,

L is total number of positioning frequency layers, and

Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i

TRSTD,i is the measurement period for PRS RSTD measurement in positioning frequency layer i as specified below:

TRSTD,i=CSSFPRS,i*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast,i ,

where:

NRxBeam,i is the UE Rx beam sweeping factor. In FR1, NRxBeam,i = 1; and in FR2,  NRxBeam,i = 8.

CSSFPRS,i is the carrier-specific scaling factor for NR PRS-based positioning measurements in positioning frequency layer i as defined in clause 9.1.5.2.

NPRS,islot is the maximum number of DL PRS resources in positioning frequency layer i configured in a slot.

Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer i to be measured during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only the PRS resources unmuted and fully or partially overlapped with MG are considered.

Nsample is the number of PRS RSTD samples and Nsample= 4.

Tlast,i is the measurement duration for the last PRS RSTD sample in positioning frequency layer i, including the sampling time and processing time, Tlast,i = Ti + Tavailable_PRS,i ,

Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i defined as:

Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i

Where,

Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34],

Tavailable_PRS,i= LCMTPRS,i,MGRPi, the least common multiple between TPRS,i and MGRPi.

MGRPi is the repetition periodicity of the measurement gap applicable for measurement in the PRS frequency layer i.

TPRS,i is the periodicity of DL PRS resource with muting on positioning frequency layer i.

If more than one PRS periodicities are configured in positioning frequency layer i, the least common multiple of PRS periodicities TperPRS with muting among all DL PRS resource sets in the positioning frequency layer is used to derive TPRS,i, where,

TperPRS with muting=Nmuting*TperPRS, is the PRS periodicity with muting per PRS resource,

TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where b1TperPRS*TmutingPRS ≤10240msNmuting=TmutingPRS*min(L,10240TperPRS*TmutingPRS)b1TperPRS*TmutingPRS>10240msNmuting=1

TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

- Note: For the purpose of calculating TPRS,i, only the PRS resources fully or partially covered by the MG are considered.

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSysmbols in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34].

N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfDL-PRS-ResProcessedPerSlot specified in TS 37.355 [34].

The time TRSTD,Total starts from the first MG instance aligned with a DL PRS resource(s) in the assistance data after both the NR-TDOA-ProvideAssistanceData message and NR-TDOA-RequestLocationInformation message are delivered from LMF to the physical layer of UE via LPP [34].

Note: 	No per-positioning frequency layer requirement is applied in scenarios when multiple positioning frequency layers are configured.

If during the measurement period of one or more positioning frequency layers, the MG pattern is reconfigured, the measurement period can be longer. When PRS-RSRP is configured for DL-TDOA, RSTD and RSRP are performed over the same measurement period.

The measurement requirements in this clause apply, provided no PRS symbols are dropped during the measurement period TRSTD,Total within measurement gaps due to collisions with other signals; otherwise, the measurement period can be longer.

The measurement requirements do not apply for a PRS resource, if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i.

The measurement requirements do not apply for a PRS resource, if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

The requirements in clause 9.9.2 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-TDOA-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If handover occurs while RSTD measurements are being performed, then the UE shall continue and complete the on-going RSTD measurements. The UE shall also meet the RSTD measurement requirements in this clause and measurement accuracy requirements in clause 10.1.23. However, in this case the RSTD measurement period TRSTD,total.HO shall be as follows:

TRSTD, total,HO=TRSTD, Total+K*Teffect+THO

Where,

-	K is the number of times handover occurs during TRSTD,total.HO;

-	Teffect is the largest Teffect,i among all positioning frequency layers;

-	THO is the time during which the RSTD measurement may not be possible due to handover; it can be up to Tinterrupt as defined in clause 6.1.

9.9.2.6	Void

9.9.3	PRS-RSRP measurements

9.9.3.1	Introduction

The requirements in clause 9.9.3.5 shall apply provided the UE has received a message from LMF via LPP [34] requesting the UE to measure and report PRS-RSRP measurements defined in TS 38.215 [4].

9.9.3.2	Requirements applicability

The requirements in clause 9.9.3 apply for periodic and triggered PRS-RSRP measurements, provided:

-	PRS-RSRP related side conditions given in clause 10.1.24 are met for a corresponding Band.

9.9.3.3	Measurement Capability

UE PRS-RSRP measurement capability is as indicated by the UE in NR-DL-AoD-ProvideCapabilities according to TS 37.355 [34].

9.9.3.4	Measurement Reporting Requirements

This requirement assumes that the measurement report is not delayed by other LPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report.

The reported PRS-RSRP measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clauses 10.1.24.3.

The PRS-RSRP measurement accuracy for all measured PRS resources shall be fulfilled according to the accuracy requriements specified in the clauses 10.1.24.

9.9.3.5	Measurement Period Requirements

When the physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 9.9.3.3) PRS-RSRP measurements, defined in TS 38.215 [4], from configured PRS resources for configured TRPs on configured positioning frequency layers, within TPRS-RSRP,total ms.

TPRS-RSRP, total=i=1LTPRS-RSRP,i+L-1*maxTeffect,i

where

i is the index of positioning frequency layer,

L is total number of positioning frequency layers,

Teffect,i is the periodicity of the PRS-RSRP measurement in positioning frequency layer i.

TPRS-RSRP,i=CSSFi*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast

where

CSSFi is the carrier specific scaling factor for PRS-RSRP measurements specified in clause 9.1.5.2,

NRxBeam,i is the scaling factor for Rx beam sweeping, and NRxBeam,i=1 if positioning frequency layer i is in FR1 and NRxBeam,i=8 if positioning frequency layer i is in FR2,

Lavailable_PRS,i is the time duration of available PRS to be measured in the positioning frequency layer i to be measured during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only the PRS resources unmuted and fully or partially overlapped with MG are considered.

NPRS,islot is the maximum number of DL PRS resources of positioning frequency layer i configured in a slot,

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSysmbols in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34],

N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfDL-PRS-ResProcessedPerSlot  in clause 6.4.3 of TS 37.355 [34],

Nsample is the number of PRS-RSRP measurement samples and Nsample= 4,

Tlast = Ti +Tavailable_PRS,i is the measurement duration for the last PRS-RSRP sample, including the sampling time and processing time,

Teffect,i= TiTavailable_PRS,i*Tavailable_PRS,i is the periodicity of PRS-RSRP measurement in positioning frequency layer i,

Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34],

Tavailable_PRS,i= LCMTPRS,i,MGRPi is the least common multiple between TPRS,i and MGRPi ,

TPRS,i is the maximum PRS resource periodicity among all PRS resources in positioning frequency layer i,

MGRPi is the measurement gap repetition period in positioning frequency layer i.

If positioning frequency layer i has more than one DL PRS resource set with different PRS periodicities with muting,  TperPRS with muting=Nmuting*TperPRS, the least common multiple of  TperPRS with muting among the DL PRS resource sets is used to derive TPRS,i, where:

TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where b1TperPRS*TmutingPRS ≤10240msNmuting=TmutingPRS*min(L,10240TperPRS*TmutingPRS)b1TperPRS*TmutingPRS>10240msNmuting=1TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

Note:	For the purpose of calculating TPRS,i, only the PRS resources fully or partially covered by the MG are considered.

When PRS-RSRP measurements are configured for DL-AoD, the time TPRS-RSRP,total starts from the first MG instance aligned with DL PRS resources in the assistance data after both the NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message from LMF via LPP [34] are delivered to the physical layer of UE.

Note:	No per-positioning frequency layer requirement is applied in scenarios when multiple positioning frequency layers are configured.

When the PRS-RSRP measurement is configured together with RSTD measurement then the PRS-RSRP measurement shall meet the RSTD measurement requirements defined in clause 9.9.2.

When the PRS-RSRP measurement is configured together with UE Rx-Tx time difference measurement then the PRS-RSRP measurement shall meet the UE Rx-Tx time difference measurement requirements defined in clause 9.9.4.

The measurement requirements do not apply for a PRS resource:

-	if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i or

-	if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

If during the measurement period of one or more positioning frequency layers, the MG pattern is reconfigured either per UE request or not per UE request, the measurement period can be longer.

The requirements in this section apply, provided no PRS symbols are dropped during the measurement period TPRS-RSRP, total within measurement gaps due to collisions with other signals; otherwise, a longer measurement period may be used.

The requirements in clause 9.9.3 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-AoD-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If handover occurs while PRS-RSRP measurements are being performed then the UE shall complete the ongoing PRS-RSRP measurements session. The UE shall also meet the PRS-RSRP measurement requirements in this clause and measurement accuracy requirements in clause 10.1.24. However, in this case the PRS-RSRP measurement period TPRS-RSRP,total.HO shall be as follows:

TPRS-RSRP, total,HO=TPRS-RSRP, total+K*Teffect+THO   ms

where

K is the number of times handover occurs during TPRS-RSRP,total.HO;

Teffect is the largest Teffect,i among all positioning frequency layers;

THO is the time during which the PRS-RSRP measurement may not be possible due to handover; it can be up to Tinterrupt as defined in clause 6.1.

9.9.4	UE Rx-Tx time difference measurements

9.9.4.1 Introduction

The requirements in this clause shall apply, provided the UE has received nr-Multi-RTT-RequestLocationInformation message from LMF via LPP [34] requesting the UE to measure and report one or more UE Rx-Tx time difference measurements defined in TS 38.215 [4].

9.9.4.2 Requirements Applicability

The requirements in clause 9.9.4 apply for periodic and triggered UE Rx-Tx time difference measurements, provided:

-	UE Rx-Tx time difference measurement related side conditions given in clause 10.1.25 are met for a corresponding band.

-	SRS is configured on at least one of the PCell, PSCell and SCell.

9.9.4.3 Measurement Capability

UE Rx-Tx time difference measurement capability is as indicated by the UE in NR-Multi-RTT-ProvideCapabilities, according to TS 37.355 [34].

9.9.4.4 Measurement Reporting Requirements

This requirement assumes that the measurement report is not delayed by other LPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report.

The UE Rx-Tx time difference measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clause 10.1.25.

The UE Rx-Tx time difference measurement accuracy for all measured DL PRS resources shall be fulfilled according to the accuracy requirements specified in clause 10.1.25.

9.9.4.5 Measurement Period Requirements

When physical layer receives last of NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message from LMF via LPP [34], UE shall be able to measure multiple (up to the UE capability specified in clause 9.9.4.3) UE Rx-Tx time difference measurements as defined in TS 38.215 [4] in configured positioning frequency layers within the measurement period TUERxTx,Total ms.

TUERxTx, Total=i=1LTUERxTx,i+L-1*maxTeffect,i.

where i is the index of positioning frequency layer,

TUERxTx,i is the measurement period for UE Rx-Tx time difference measurements in positioning frequency layer i as further defined in this clause,

L is total number of positioning frequency layers, and

Teffect,i is the periodicity of the UE Rx-Tx time difference measurement in positioning frequency layer i as defined further in this clause.

TUERxTx,i=CSSFi*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast,i

Where

CSSFi is the carrier-specific scaling factor for NR PRS-based measurement in the positioning frequency layer i as defined in clause 9.1.5.2,

NRxBeam,i is the scaling factor for Rx beam sweeping, and NRxBeam,i=1 if positioning frequency layer i is in FR1 and NRxBeam,i=8 if positioning frequency layer i is in FR2,

Lavailable_PRS,i is the time duration of available PRS resources in the positioning frequency layer i, to be measured during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only the PRS resources unmuted and fully or partially overlapped with MG are considered.

NPRS,islot is the maximum number of DL PRS resources of positioning frequency layer i configured in a slot,

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSysmbols in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in clause 4.2.7.2 of TS 37.355 [34],

N’ is UE capability for number of DL PRS resources that it can process in a slot corresponding to maxNumOfDL-PRS-ResProcessedPerSlot as specified in clause 6.4.3  of TS 37.355 [34],

Nsample is the number of UE Rx-Tx time difference measurement samples and Nsample= 4,

Tlast,i is the measurement duration for the last UE Rx-Tx time difference measurement sample in the positioning layer i, including the sampling time and processing time, Tlast,i = Ti + Tavailable_PRS,i ,

Teffect,i is periodicity of UE Rx-Tx time difference measurement in positioning frequency layer i:

Teffect,i= TiTavailable_PRS,i*Tavailable_PRS,i

where

Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34],

Tavailable_PRS,i= LCMTPRS,i,MGRPi, the least common multiple between TPRS,i and MGRPi

MGRPi is the measurement gap repetition periodicity in positioning frequency layer i.

TPRS,i is the PRS resource periodicity in positioning frequency layer i. If the positioning frequency layer i has more than one DL PRS resource sets with different PRS periodicities with muting,  TperPRS with muting=Nmuting*TperPRS, the least common multiple of TperPRS with muting among DL PRS resource sets is used to derive TPRS,i, where

TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where b1TperPRS*TmutingPRS ≤10240msNmuting=TmutingPRS*min(L,10240TperPRS*TmutingPRS)b1TperPRS*TmutingPRS>10240msNmuting=1TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

Note:	For the purpose of calculating TPRS,i, only the PRS resources fully or partially covered by the MG are considered.

The time TUERxTx,Total starts from the first MG instance aligned with DL PRS resources in the assistance data after both the NR-Multi-RTT-RequestLocationInformation message and NR-Multi-RTT-ProvideAssistanceData message from LMF via LPP [34] are delivered to the physical layer of UE.

Note:	No per-positioning frequency layer requirement is applied in scenarios when multiple positioning frequency layers are configured.

The UE Rx-Tx time difference measurement period is restarted if HO occurs during the measurement period and after SRS reconfiguration on the target cell is complete.

The measurement requirements do not apply for a PRS resource:

-	if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i or

-	if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

If during the measurement period of one or more positioning frequency layers, the MG pattern is reconfigured either per UE request or not per UE request, the measurement period can be longer.

The requirements in this section apply, provided no PRS symbols are dropped during the measurement period TUERxTx,Total within measurement gaps due to collisions with other signals; otherwise, a longer measurement period may be used.

When PRS-RSRP is configured for multi-RTT, the UE Rx-Tx time difference measurements and PRS-RSRP measurements are performed over the same measurement period.

The requirements in clause 9.9.4 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-Multi-RTT-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

Editor’s note: FFS: Measurement period requirements when cell change does not impact SRS configuration

Editor’s note: FFS: Measurement period requirements when cell change does impact SRS configuration

If UE uplink transmission timing changes due to the network-configured Timing Advance command during the UE Rx-Tx measurement period, then the UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

If UE uplink transmission timing changes due to the change in the NTA_offset defined in Table 7.1.2-2 during the UE Rx-Tx measurement period, then the UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

<End of Change 1>