3GPP TSG-RAN WG4 Meeting # 101-e 						R4-2120321

Electronic Meeting, 1st – 12th Nov, 2021

Agenda item:			8.19.3

Source:	Moderator (Samsung)

Title:	WF on FeMIMO RRM requirements for inter-cell beam management

Document for:	Approval

Introduction

This contribution is to capture the agreements for the email discussion on inter-cell beam management for Rel-17 FeMIMO RRM in RAN4 #101 meeting

Way-forward

- Define known/unknown cell condition for non-serving cell configured for L1-RSRP measurements

- Measurement accuracy for serving cell L1-RSRP can be reused for L1-RSRP measurement for non-serving cell for inter-cell beam management

- The L1-RSRP measurement requirements for non-serving cells are only applicable to SSB withing the active DL BWP of the UE.

- RAN4 develops UE requirements for at least Nmax = 1, where Nmax is the maximum number of RRC configured PCIs different from the serving cell for measurement/reporting.

  - RAN4 will further study to specify the requirements for other Nmax value taking RAN1 decision into account

- RAN4 will further study timing offset assumptions between non-serving cell and serving cell for L1-RSRP measurement in future RAN4 meetings

  - Where non-serving cell refers to all the TRPs configured by gNB for inter-cell BM and serving cell(s) is the cell(s)/TRP(s) from which UE receive PDCCH/PDSCH.

- RAN4 will further study the FFT implementation assumption for L1-RSRP measurement

- RAN4 will further study the Rx beam assumption for L1 RSRP measurement

- RAN4 will further study the measurement behaviour for L1-RSRP measurement for non-serving cell whether to perform the non-serving cell L1-RSRP measurement outside SMTC or within SMTC