3GPP TSG-RAN WG4 Meeting # 101-e 												R4-2120335

Electronic Meeting, November 1st – 12th 2021

Agenda item:			8.23

Source:	Nokia, Nokia Shanghai Bell, …

Title:	WF on NR_IIOT_URLLC_enh_RRM

Document for:	Approval

# Background

During RAN4#101

agenda item in the RAN4#101 e-meeting:

8.23	Enhanced IIoT and URLLC support	[NR_IIOT_URLLC_enh]

* There is incoming LS from RAN1, i.e., R1-2108635 LS on TA-based propagation delay compensation

8.23.1	General	[NR_IIOT_URLLC_enh-Core]

8.23.2	RRM core requirements	[NR_IIOT_URLLC_enh-Core]

8.23.2.1	Propagation delay compensation enhancements	[NR_IIOT_URLLC_enh-Core]

8.23.2.2	Reference point for Te requirements	[NR_IIOT_URLLC_enh-Core]

8.23.2.3	Others	[NR_IIOT_URLLC_enh-Core]

Email discussion: [101-e][238] NR_IIOT_URLLC_enh_RRM

R4-2120233	Email discussion summary for [101-e][238] NR_IIOT_URLLC_enh

Type: other		For: Information

Source: Moderator (Nokia)

Abstract:

Discussion:

Decision:		Return to.

R4-2110380	Email discussion summary: [101-e][238] NR_IIOT_URLLC_enh_RRM

Type: other		For: Information
					Source: Moderator (Nokia)

Abstract:

Discussion:

Decision:		Return to.

Earlier agreed documents

RAN4#100bis:

- R4-2115371, WF on NR_IIOT_URLLC_enh_RRM

RAN4#99:

- R4-2108368, WF on RRM for NR IIoT and URLLC,

# WF on topic#1 Propagation delay compensation enhancements

RAN4 had GTW on NR_IIOT_URLLC_enh_RRM Monday November 1st during which following was agreed:

Sub-topic 1-1, Issue 1-1:

- Agreements

  - There is no consensus in RAN4 whether it is feasible to support a smaller value than the current Te assuming the existing conditions in TS 38.133 for Te requirement

Sub-topic 1-2, Issue 1-2:

- Agreements

  - It is feasible to support a smaller Te value than the current Te for the use of propagation delay compensation under assumption of using TRS (or other RS used for Te estimation) instead of SSB

    - Smaller Te can be achieved when TRS (or other RS) bandwidth is larger than SSB bandwidth

    - Smaller Te can be achieved for UE is operating in RRC_CONNECTED mode

    - FFS whether a smaller Te can be achieved for the first transmission in the DRX cycle

      - Further check with RAN1 if presence of TRS (or other RS) can be guaranteed during the DRX OFF

    - Support of Smaller Te can be defined as an optional Rel-17 capability and should not apply to all UEs

Topic#1 Propagation delay compensation enhancements:

Following has been proposed as tentative agreement after 1st round (Sub-Topic 4):

Agree that it is it feasible to introduce enhanced TA command indication granularity.

Additionally, following side conditions are agreed:

- Applies only to UE supporting PDC

- It is understood that granularity and accuracy are not the same. Hence, this agreement does not impact UL timing requirement.

Please raise any concern if the tentative agreement above is not agreeable.

Topic above was discussed in GTW (Wednesday session) and following was agreed:

- Agreements

  - It is feasible to introduce enhanced TA command indication granularity

  - It is RAN4 understanding that

    - Enhanced granularity will apply for UE supporting PDC

    - This agreement does not impact UL timing accuracy requirement

  - TA command indication granularity can be reduced to 64Tc for both 15kHz SCS and 30kHz SCS.

    - RAN4 will further discuss whether there are any specific conditions to use improved TA command indication granularity

Topic #2 Reference point for Te requirements:

During the first round discussion RAN4 discussed Issue 2-3: Whether to define UE timing error requirements side conditions. Based on the outcome of first round there was no consensus to define new UE timing error requirements side conditions. Based on this moderator suggest not to define UE timing error requirements side conditions.

Do not define UE timing error requirements side conditions (rely on the test case side conditions)

Please raise any concern if the tentative agreement above is not agreeable. No concerns raised and moderator conclude following as being agreement:

Do not define UE timing error requirements side conditions (rely on the test case side conditions)

## New conditions which will be used for evaluating (evaluating: how much Te can be reduced at most, under the agreed new conditions) (Sub-topic 1-6)

Issue 1-6-1: TRS BW

- Proposals

  - Option 1: Maximum BW for the given SCS proposed

  - Option 2: Other (please list also reason)

- Recommended WF

  - Use maximum BW for the given SCS proposed

All companies could support the recommended WF and no objections from any company. Based on input from a company moderator leave it open for companies to use different than maximum BW for the given SCS. All companies are encouraged to provide used TRS BW assumption.

Agreements:

- no

WF for next meeting:

- Companies can use maximum BW for the given SCS proposed or less. Companies should indicate which BW has been assumed.

Issue 1-6-2: Channel model

- Proposals

  - Option 1: LOS channel

  - Option 2: Other (please list also reason)

- Recommended WF

  - Use LOS channel

Almost aligned views in the 2nd round. Companies prefer to use same channel model as in the test case. Moderator suggest to use LOS/AWGN channel model

Agreements:

- Use the same channel model as used in the test case: LOS/AWGN channel

WF for next meeting:

- Discussion is concluded

Issue 1-6-3: SINR

- Proposals

  - Option 1: -6dB

  - Option 2: Other (please list also reason)

- Recommended WF

  - Use -6dB SINR

Diverse views among companies and only 1 company supports the recommended WF. All other companies prefer option 2 and suggest using same channel condition as the one used in the test case.

Agreements:

- Moderator suggest to at least use same SINR as used in the test case. Other SINR levels are not prohibited.

## How much can Te be reduced at most (Sub-topic 1-7)

Based on the company views it may be possible to agree on some conservative estimate using 1/(2*DL_RS_BW) and using 15KHz and 30 KHz SCS. Using the above new conditions and the maximum RS BW for DL RS, the enhanced Te can be reduced to at most:

1) 262 ns (252ns Te margin +10ns DL detection error with 50MHz DL RS BW) for 15 kHz SCS, and

2) 196ns (191ns Te margin + 5ns DL detection error with 100MHz DL RS BW) for 30kHz SCS

Issue 1-7-1: How much can Te be reduced at most

- Proposals

  - Companies to list their views.

Agreements:

- no agreements

WF for next meeting:

- RAN4 will wait for RAN1 decision for PDC method. If RAN1 decides to adopt enhanced TA based method, then RAN4 can perform further evaluation regarding how to reduce Te.

- Given the agreed assumptions. The next step is Link simulation and evaluation based on agreed evaluation conditions. This would require work to be presented at next meeting.

- The 1/(2*DL_RS_BW) expression, where e.g. DL_RS_BW = 3.6 MHz for SSB with SCS = 15 kHz., has been used to derive existing requirements, but then a substantial margin has been added in top of 1/(2*DL_RS_BW), which is evident if one compares with existing NR Te values. Margins are FFS.

- An approach like 1/(2*DL_RS_BW), can hypothetically be extended to wider BW, but the frequency and time distribution of reference symbol make SNIR performance different for different frequency and time distributions of any reference Companies may use provided formula: .

## Applicability rules and new conditions which will be used for evaluation (evaluating: how much TA command indication granularity can be reduced at most) (Sub-topic 1-8)

Issue 1-8-1: Applicability rules for new TA command indication granularity

- Proposals

  - Option 1: New TA command indication granularity is only applicable to UEs supporting PDC and it is understood that granularity and accuracy are not the same. Hence, this agreement does not impact UL timing requirement.

  - Option 2: Other applicability rules (please list also reason)

- Recommended WF

  - Agree option 1 (based on the compamies proposals in round 1)

All companies support the recommended WF.

Agreements:

- New TA command indication granularity is only applicable to UEs supporting PDC and it is understood that granularity and accuracy are not the same. Hence, this agreement does not impact UL timing requirement.

WF for next meeting:

- Discussion is concluded

Issue 1-8-2: Conditions for evaluating the TA command indication granularity

- Proposals

  - Option 1: 15KHz SCS and 30KHz SCS

  - Option 2: Other (please list also reason)

- Recommended WF

  - At least use 15KHz and 30Khz

All companies could support the recommended WF. One company was not convinced if the question was relevant due to there is no agreement on introducing new UL timing requirements. However, for evaluating how much the TA command indication granularity can be reduced, moderator suggest going forward with the recommended WF – with the note that this is not related to UL timing requirements

Agreements:

- Conditions for evaluating the TA command indication granularity, use at least 15KHz SCS and 30KHz SCS.

  - Note: this is not related to UL timing requirements.

## How much can TA command indication granularity be reduced (Sub-topic 1-9)

Issue 1-9-1: How much can TA command indication granularity be reduced at most

- Proposals

  - TA command indication granularity can be reduced to 64Tc for both 15kHz SCS and 30kHz SCS

  - It is feasible to have TA command granularity as (1/16)* (16*64*Tc/2m)

  - It is not feasible to introduce enhanced TA command indication granularity

- Recommended WF

  - Companies to list their proposals

During 2nd round not much new was added to the discussion and it is recommended to continue next meeting.

Agreements:

  - no agreement

WF for next meeting:

- RAN4 will wait for RAN1 decision for PDC method. If RAN1 decides to adopt enhanced TA based method, then RAN4 can further discuss whether there are any specific conditions to use improved TA command indication granularity, since granularity and accuracy are not the same.

# WF on topic#2 Reference point for Te requirements

Following tentative agreement has been proposed for agreement after 1st round (sub-topic 2-3)

Do not define UE timing error requirements side conditions (rely on the test case side conditions)

No objections received and hence the tentative agreement is agreed:

Do not define UE timing error requirements side conditions (rely on the test case side conditions)

## Clarification to the downlink timing definition (Sub-topic 2-5)

Issue 2-5-1: Is it agreeable to clarify the downlink timing definition from Rel-17?

- Proposals

  - Option 1: It is agreeable to clarify the downlink timing definition from Rel-17

  - Option 2: Do not clarify the downlink timing definition from Rel-17 (or earlier)

- Recommended WF

  - More discussion is needed

Diverse views and no consensus.

Agreements:

- no agreements

WF for next meeting:

- Discussion is concluded

## TP for downlink timing definition (Sub-topic 2-6)

Based on companies views for Issue 2-5-1, companies should indicate their preferred TP for the downlink timing definition for Rel-17.

Issue 2-6-1: TP for down timing definition for Rel-17

- Proposals

  - Option 1: Clarifying by including ‘at the UE antenna’ as agreed earlier:

    - ‘The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell at the UE antenna.’

  - Option 2: Clarifying by removing ‘detected’ and including ‘at the UE antenna’ as agreed earlier:

    - ‘The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame is received from the reference cell at the UE antenna.’

  - Option 3: clarifying by removing ‘detected’ and use ‘arrives’ instead of ‘is received’ and including ‘at the UE antenna’ as agreed earlier:

    - ‘The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame from the reference cell arrives at the UE antenna.’

  - Option 4: Keep current text:

    - ‘The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell.’

- Recommended WF

  - More discussion is needed

Diverse views and no consensus.

Agreements:

- no agreement

WF for next meeting:

- Companies to consider if Option 1 is agreeable for next meeting:

  - ‘The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell at the UE antenna.’

  - If agreeable it would be changed from Rel-15.

# WF on topic#3 Other (PUCCH Carrier Switching)

Two sub-topics were discussed in the meeting.

## Need for defining RRM core requirements for PUCCH carrier switching (Sub-topic 3-1)

Agreements:

- RAN4 do not define RRM core requirements for PUCCH carrier switching.

WF:

- This topic has been concluded and RAN4 need not discuss this further.

## Need for defining test case for PUCCH carrier switching (Sub-topic 3-2)

Agreements:

- RAN4 do not define test case for PUCCH carrier switching.

WF:

- This topic has been concluded and RAN4 need not discuss this further.