3GPP TSG-RAN WG4 Meeting # 101-e 											        R4-2120337

Electronic Meeting, Nov. 1-12, 2021

Agenda item:			8.24

Source:	OPPO

Title:	WF on NR SL relay RRM

Document for:	Approval

Topic #1: General requirements for NR_SL_relay_RRM

Sub-topic 1-1: RRM requirements for ProSe Direct Discovery

Issue 1-1: Whether to specify cell reselection requirements for NR Sidelink Discovery on non-serving frequency

Agreements in 1st round:

RAN4 agree not to specify cell reselection requirements for NR sidelink discovery on non-serving frequency in R17, based on the assumption of R16 V2X

Issue 1-2: Whether and how to specify other RRM requirements for ProSe Direct Discovery

Agreements in 1st round:

RAN4 agree to define interruptions at NR sidelink discovery configuration. FFS on the details of requirements.

Sub-topic 1-2: Interruption requirements

Issue 1-3: Interruption

Agreements in 1st round:

RAN4 agree to define interruption requirements at discovery configuration (for relay discovery and sidelink direct discovery).

Agreement in 2nd round:

- Interruption requirements for NR sidelink discovery should be defined based on R16 V2X sidelink assumption.

- FFS how to specify the requirements in next meeting

Sub-topic 1-3: UE transmit timing

Issue 1-4: UE transmit timing

Agreements in 1st round:

UE transmit timing requirements from Rel-16 V2X sidelink can apply.

Sub-topic 1-4: Requirements applicability

Issue 1-5: Synchronization assumption

Agreements in 1st round:

The selection/reselection procedure of synchronization reference source in Rel-16 shall be reused for NR SL relay/remote UE. The impact on performance part including test cases can be further discussed in 2nd phase.

Sub-topic 1-5: Others

Issue 1-6: Requirements for RRC connection reestablishment

Agreements in 1st round:

RAN4 should wait for RAN2 final agreements on the procedure for RRC connection reestablishment before discussing the exact requirements.

Issue 1-7: CR split

Agreements in 2nd round:

| CR | Agenda item | Volunteer Company | Note |
| --- | --- | --- | --- |
| Interruptions at NR Sidelink discovery configuration | 12.y | Huawei |  |
| Measurement requirement for Selection / reselection of relay UE | 12.z | OPPO |  |
| Intra-frequency measurement accuracy requirements | 10.x | QC |  |

Topic #2: Measurement requirements for relay discovery and (re)selection

Sub-topic 2-1: Measurement quantity (RSRP)

Issue 2-1: How to use RSRP as measurement quantity

Agreements in 1st round:

Define unified requirements for both SD-RSRP and SL-RSRP measurement in relay discovery and (re)selection

Agreements in 2nd round:

PSCCH-DMRS and/or PSSCH-DMRS can be used for the unified RSRP measurements.

Issue 2-2: RSRP accuracy

Agreements in 2nd round:

Reuse L1 SL-RSRP measurement accuracy requirement and side condition for both SL-RSRP in indirect to direct switch, and SD-RSRP direct to indirect switch.

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR V2X operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm/SCS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCS = 15kHz | SCS = 30kHz | SCS = 60kHz |  |  |
| 4.5 | 9 | 0 dB | NR_TDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_J | -116.5 | -113.5 | -110.5 | N/A | -70 |
| 8 | 11 | 0 dB | NR_TDD_FR1_B NR_TDD_FR1_J | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR V2X operating band groups are as defined in Section 3.5 for the corresponding NR operating bands. NOTE 3: The parameter Ês/Iot is the Ês/Iot of PSCCH-DMRS and/or PSSCH-DMRS. |  |  |  |  |  |  |  |  |

Sub-topic 2-2: Measurement time windows for relay discovery and (re)selection

Issue 2-3: How to define measurement time windows

Agreements in 1st round:

Use the period of the discover signal as the discovery period

- Resource reservation period (mode 2) or SPS transmission periodicity (mode 1) can be used

Issue 2-4: Whether to consider SL DRX

Agreements in 1st round:

- Not consider SL DRX for SL relay in R17

Sub-topic 2-3: Measurement requirement

Issue 2-5: Measurement requirement

Agreements in 1st round:

Reuse measurement/evaluation period requirement from LTE ProSe.

| Discovery Period [s] | Tmeasure,ProSe_Relay_Intra [s] (number of discovery periods) | Tevaluate, ProSe_Relay_intra [s] (number of discovery periods) |
| --- | --- | --- |
| 0.04≤Discovery period≤10.24 | Note 1 (4) | Note 1 (16) |
| NOTE 1: Time depends upon the configured Discovery period. |  |  |

References

[1] R4-2120234, Email discussion summary for [101-e][239] NR_SL_relay, Moderator (OPPO), RAN4 #101e

[2] R4-2120381, Email discussion summary for [101-e][239] NR_SL_relay, Moderator (OPPO), RAN4 #101e