3GPP TSG-RAN WG4 Meeting #101-e	R4-2120344

Electronic Meeting, 1 – 12 November, 2021

Agenda item:			4.1.6

Source:	Moderator (Huawei)

Title:	Email discussion summary for [101-e][201] Maintenance_R15_NR_RRM_Core

Document for:	Information

Introduction

The scope of this email discussion includes the following agenda items:

4.1.6 RRM core requirements maintenance (38.133/36.133) [NR_newRAT-Core]

In providing comments, companies are encouraged to:

- Ensure that the comments are inserted in the latest version of the document by checking the folder before uploading

- Use “Track changes” to help identify added comments/changes

- Pay attention to the rule for shortening file name

Topic #1: Rel-15 NR RRM core requirements

Companies’ contributions summary

| T-doc | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117441 | Apple | draftCR: Add clarification for the case when actual SSB periodicity is greater than SMTC periodicity for intra-frequency measurement without MG. |
| R4-2118247 | vivo | draftCR: 1. Add reference to TS 36.133 for PSCell change requriements. 2. Revise the RRC processing delay in NE-DC PSCell addition requirements. 3. Remove the RACH uncertantainty for Pcell in NE-DC PSCell addition requirements. |
| R4-2118248 | vivo | Not available |
| R4-2118249 | vivo | Not available |
| R4-2118323 | MediaTek inc. | draftCR: Revise the maximum timing changed as 3200/2µ Tc, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211. |
| R4-2118326 | MediaTek inc. | Observation 1: The maximum timing changed of 3200 Tc to apply TSSB_measurement_period is based on the SCS of 15 kHz. Observation 2: The maximum timing changed of 3200 Tc will be larger than 0.5*CP for the SCS higher than 15 kHz. It becomes several CP in FR2. The time for TPSS/SSS_sync will be needed. Proposal 1: To apply the reporting delay requirement of TMeasurement_Period, revise the maximum timing changed to be 3200 Tc /2µ Tc, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211. |
| R4-2118401 | Ericsson | draftCR: Introduce the max function between the fixed value 10s and N1 scaling factor with DRX cycles. |
| R4-2118784 | Huawei, Hisilicon | draftCR: - To clarify that RRC latency in 36.331 shall be referred if the procedure is triggered by E-UTRA RRC message, otherwise RRC latency in 38.331 shall be referred. - To clarify that the starting slot/subframe for delay requirements counting (the "slot n/subframe n") should be the last slot/subframe which overlapps with the last TTI containing RRC messages. After initial check, we find that at least following requirements should be clarified: - R15 UL carrier RRC reconfiguration delay in cl.8.4; - R15 RRC-based BWP switching delay in cl.8.6.3 - R15 RRC-based TCI state switching delay in cl.8.10.5 - R15 PSCell change delay in cl.8.11 |
| R4-2119221 | MediaTek inc. | Proposal 1: RAN4 to specify the applicable DRX cycle for the infer-frequency measurement requirement in NR-DC and NE-DC mode Proposal 2: In NR-DC mode, the applicable DRX cycle for the infer-frequency measurement requirement follows the maximum of configured MCG DRX cycle and SCG DRX cycle Proposal 3: In NE-DC mode, the applicable DRX cycle for the infer-frequency measurement requirement follows the maximum of configured MCG DRX cycle and SCG DRX cycle |
| R4-2119222 | MediaTek inc. | draftCR: To specify which DRX cycle should be in use when counting the requirement of inter-frequency meaurement - In NR-DC mode and NE-DC mode, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the longer DRX cycle bwteen master cell group and secondary cell group. To specify which DRX cycle should be in use when counting the requirement of inter-RAT meaurement In NE-DC mode, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the longer DRX cycle bwteen master cell group and secondary cell group. |
| R4-2119443 | Ericsson, Nokia, Nokia Shanghai Bell, Intel, Huawei, HiSilicon, Qualcomm | The definition of the reference point for the UE initial transmit timing control requirement is clarified. |
| R4-2119568 | Qualcomm Incorporated | Differentiated RRC processing delay for EN-DC from non EN-DC. |

Open issues summary

Note:

- Cat-A draftCRs are not listed

- R4-2118248 and R4-2118248 from vivo are reserved as Cat-F draftCRs, but they are not available. Please @vivo double check if they should be Cat-A.

- R4-2119443 from Ericsson, Nokia, Nokia Shanghai Bell, Intel, Huawei, HiSilicon, Qualcomm is not listed, and it will be treated in email #237 NR_IIOT_URLLC_enh.

Sub-topic 1-1: Measurement requirements

Issue 1-1-1: Clarification for the case when SSB periodicity is greater than SMTC periodicity

- Proposals

  - Option 1 (Apple R4-2117441)

    - Reason for change

      - No clarification for the case when actual SSB periodicity is greater than SMTC periodicity for intra-frequency measurement without MG.

    - Summary of Changes

      - Add clarification for the case when actual SSB periodicity is greater than SMTC periodicity for intra-frequency measurement without MG.

    - Example of the Changes

- Table 9.2.5.1-1: Time period for PSS/SSS detection, (Frequency range FR1) DRX cycle; TPSS/SSS_sync_intra. No DRX; max( 600ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFintra. DRX cycle≤ 320ms; max( 600ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(5] x Kp) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: During PSS/SSS detection, if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer TPSS/SSS_sync_intra is expected..

- Recommended WF

  - Further discuss if option 1 agreeable.

| Company | Comments |
| --- | --- |
| Apple | We have such clarification in cell detection requirement of IDLE mode, and therefore it is necessary to also clarify it in connected mode. |
| MTK | We prefer NOT to have such note in CONNCETED. To our understanding, such clarification was added in R16 IDEL for the case that some cells would transmit SSBs in SMTC2, which is sparser than SMTC1 in IDLE. Thus for R16 IDLE, we can see the use case for “actual SSB transmission periodicity is greater than the SMTC”. However, for the CONNECTED mode, we don’t see the case “actual SSB transmission periodicity is greater than the SMTC”. , |
| Qualcomm | To MTK, if this is the case and the reason we don’t think such clarification is needed, wouldn’t it be also good to have this somewhere in spec? The current note implies this is allowed and valid, which we do not agree with. To Apple, just for a better understanding, why does this clarification apply only to intra-frequency measurement without MG? |
| Apple | To MTK and QC, this is a common rule for all kind of measurements which are based on SMTC. We also would like to avoid such case (error case), so it’s fine to us to clarify either “actual SSB transmission periodicity shall be not greater than the configured SMTC” somewhere or if that error case happens “no requirement” or “extended requirement” is expected. |
| Huawei | This issue has been discussed in RAN4#92 and the following agreement was reached: In RAN4 understanding, network won’t configure the SMTC periodicity shorter than SSB periodicity, per the definition of SMTC. - Further discussion is needed how and whether to capture it in the specification.. We have provided CR with similar wording in R4-1911874, but at that time not all companies could agree to add such clarification in the spec because this was considered as a common understanding. We do not have strong view on the need for the CR given the agreement, but if the CR is to be further pursued, we prefer to state “no requirement would apply” because this is more like an error case. In addition, we agree with QC that the clarification should be applicable to all SMTC based measurement, so maybe we can consider to put it somewhere in section 3? |
| Ericsson | We agree with MTK and QC that such clarification is not needed. This is an error case and not sure if UE will do even measurement. |
| Nokia | It is not clear what the definition of ‘actual SSB transmission periodicity’ is. |

Issue 1-1-2: Timing change in event triggered reporting requirements

- Proposals

  - Option 1 (MTK R4-2118326, R4-2118323)

    - Reason for change

      - To apply the delay requirement of TSSB_measurement_period_intra / TSSB_measurement_period_inter, it is provided the timing of that cell has not changed more than  3200 Tc, which is based on SCS of 15 kHz. However, for the higher SCS, the  3200 Tc will exceed CP. Thus, the maximum timing changed should be corrected with SCS configuration.

    - Summary of Changes

      - Revise the maximum timing changed as  3200/2µ Tc, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211.

    - Example of the Changes

A cell is detectable only if at least one SSBs measured from the Cell being configured remains detectable during the time period T identify_intra_without_index or T identify_intra_with_index as defined in clause 9.2.5.1 or clause 9.2.6.2. If a cell which has been detectable at least for the time period T identify intra without index or T identify intra with index defined in clause 9.2.5.1 or clause 9.2.6.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again with the same spatial reception parameter and triggers an event, the event triggered measurement reporting delay shall be less than TSSB_measurement_period_intra provided the timing to that cell has not changed more than 3200/2µ Tc while the measurement gap has not been available and L3 filtering has not been used, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211 [3]. When L3 filtering is used, an additional delay can be expected.

- Recommended WF

  - Further discuss if option 1 is agreeable.

| Company | Comments |
| --- | --- |
| Apple | Agree with MTK CR. |
| Huawei | We support option 1 and the CR. |
| Nokia | Clarification seems agreeable and ensures same timing requirement in terms of +-Tc for the different SCSs |

Issue 1-1-3: Applicable DRX cycle for inter-frequency measurmenet in NE-DC and NR-DC

- Proposals

  - Option 1 (MTK R4-2119221, R4-2119222)

    - Reason for change

      - DRX cycle that infer-frequency measurement requirement shall apply was agreed in R4-1816109 and at that time only EN-DC mode was considered. However, the DRX cycle that infer-frequency measurement requirement shall apply in NR-DC and NE-DC is still unclear.

      - RAN4 to specify the applicable DRX cycle for the infer-frequency measurement requirement in NR-DC and NE-DC mode

      - In NR-DC and NE-DC mode, the applicable DRX cycle for the infer-frequency measurement requirement follows the maximum of configured MCG DRX cycle and SCG DRX cycle

    - Summary of Changes

      - To specify which DRX cycle should be in use when counting the requirement of inter-frequency measurement

        - In NR-DC mode and NE-DC mode, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the longer DRX cycle between master cell group and secondary cell group.

      - To specify which DRX cycle should be in use when counting the requirement of inter-RAT measurement

        - In NE-DC mode, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the longer DRX cycle between master cell group and secondary cell group.

    - Example of the Changes

Table 9.3.4-1: Time period for PSS/SSS detection, (Frequency range FR1) Condition NOTE1,2; TPSS/SSS_sync_inter. No DRX; Max(600ms, 8 Max(MGRP, SMTC period)) CSSFinter. DRX cycle ≤ 320ms; Max(600ms, Ceil(8*1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320ms; 8 DRX cycle CSSFinter. NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: In NR-DC mode and NE-DC mode, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the longer DRX cycle bwteen master cell group and secondary cell group..

- Recommended WF

  - Further discuss if option 1 is agreeable.

| Company | Comments |
| --- | --- |
| Apple | Fine with the CR, one minor typo shall be corrected:”… are for the longer DRX cycle between master cell group and secondary cell group” |
| Qualcomm | “Max between MCG and SCG” makes sense because it is R15 CR. |
| Huawei | We support option 1 and the CR. |
| Ericsson | Fine with the CR |
| Nokia | This CR proposes: NOTE 3: In NR-DC mode and NE-DC mode, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the longer DRX cycle bwteen master cell group and secondary cell group. Can MediaTek clarify whether this CR suggest that the measurement requirements for inter-frequency measurements on carrier1 configured e.g. by MCG could depend on either the DRX cycle of the MCG or SCG depending on which CG has the longest DRX cycle? Such an approach is not within the current spirit of the specification and the CR agreed in last meeting (R4-2115237) addressing a similar clarification. CR is not agreeable in its current form. |
| vivo | Another option would be it is up to UE implementation which DRX cycle is used for inter-frequency measurement in NR-DC/NE-DC operation scenarios. Since there is no test for this, it would also be fine in our view. Though inter-frequency measurement is discussed in this issue, we think it may also be applicable to inter-RAT measurement if requirements are based on longest DRX cycle or up to UE implementation. |

Sub-topic 1-2: Signaling characteristic related requirements

Issue 1-2-1: NE-DC PSCell additio requirements

- Proposals

  - Option 1 (vivo R4-2118247, Change#3)

    - Reason for change

      - For NE-DC PSCell addition requirements, further clarification is needed on whether PRACH collision between PCell and PSCell is considered. Even if it is needed, it is not clear why it is 20ms. sed.

    - Summary of Changes

      - Remove the RACH uncertantainty for Pcell in NE-DC PSCell addition requirements.

    - Example of the Changes

      - Note: Change #1 and Change #2 in R4-2118247 are not listed as open issue, so please provide comments directly in section 1.3.2 if you have any on them.

Tconfig_EUTRAN-PSCell = 16ms + Tactivation_time + 50ms + TE-UTRAN-PSCell_ DU Tactivation_time is the E-UTRAN PSCell activation delay. If the E-UTRAN PSCell is known, then Tactivation_time is 20ms. If the E-UTRAN PSCell is unknown, then Tactivation_time is 30ms provided the E-UTRAN PSCell can be successfully detected on the first attempt. TE-UTRAN-PSCell_DU is the delay uncertainty in acquiring the first available PRACH occasion in the E-UTRAN PSCell. TE-UTRAN-PSCell_DU is up to 30ms.

- Recommended WF

  - Further discuss if option 1 is agreeable

| Company | Comments |
| --- | --- |
| Apple | Fine with CR, because in NE-DC mode, the RACH on LTE PSCell side is always prioritized based on RAN1 as below. So if UE cannot support DPS, LTE PSCell RACH shall not be impact, otherwise if UE can support DPS, this RACH collision uncertainty is not needed either. |
| Qualcomm | Makes sense because UE can either transmit both if support DPS or prioritize LTE PRACH. |
| Huawei | We suggest to keep option 1 FFS. In our understanding, in Rel-17 FeRRM WI there are different proposals in this meeting regarding how to handle this time uncertainty considering the power control behavior as mentioned by Apple. We are fine with the other 2 changes in the CR. |
| Nokia | We are fine to remove the RACH uncertantainty for PCell in NE-DC PSCell addition requirements. It is not clear why PCell will do RACH during NE-DC PSCell addition. |
| vivo | To Huawei: R17 feRRM is for HO with PSCell from NE-DC to NE-DC, while for this case it is PSCell addition. No requirement considered for NR PCell RACH for the case in this CR. Since it is in late stage of R15 we think it would be more appropriate to conclude. R17 feRRM may also need such clarification as baseline. |

Issue 1-2-2: RRC delay for requirements of RRC-based procedures

- Proposals

  - Option 1 (HW R4-2118784)

    - Reason for change

      - Delay requirements involved with RRC procedures in 38.133 always refers to RRC latency defined in 38.331 cl.12. In other words, it is implied that these procedures are triggered by NR RRC messages. However, this assumption is not correct since RRC-based BWP switching on NR SCG serving cells can also be triggered by E-UTRA RRCConnectionReconfiguration message (in which NR RRCReconfiguration mbedded) if SRB3 is not established.

      - Please note that it’s not merely an editorial change because RRC latency requirements for NR RRC message directly sent via SRB1/SRB3 and NR RRC message embedded in E-UTRA RRC messages are different. Still taking RRC-based BWP switching delay as an example, NR RRC latency for RRCReconfiguration is 10ms, but LTE RRC latency for E-UTRA RRCConnectionReconfiguration involving NR SCG modification is 20ms. Using NR RRC latency significantly tightens requirements.

      - For Async EN-DC, slot/subframe boundary of MCG serving cells and SCG serving cells are not aligned. So the last TTI containing RRC messages in MCG Pcell may overlap with multiple SCG slots/subframes. It’s not clear which slot/subframe is the starting point for counting delay requirements.

    - Summary of Changes

      - To clarify that RRC latency in 36.331 shall be referred if the procedure is triggered by E-UTRA RRC message, otherwise RRC latency in 38.331 shall be referred.

      - To clarify that the starting slot/subframe for delay requirements counting (the “slot n/subframe n”) should be the last slot/subframe which overlaps with the last TTI containing RRC messages.

      - After initial check, we find that at least following requirements should be clarified:

        - R15 UL carrier RRC reconfiguration delay in cl.8.4;

        - R15 RRC-based BWP switching delay in cl.8.6.3

        - R15 RRC-based TCI state switching delay in cl.8.10.5

        - R15 PSCell change delay in cl.8.11

    - Example of the Changes

For RRC-based BWP switch, after the UE receives RRC reconfiguration involving active BWP switching or parameter change of its active BWP, UE shall be able to receive PDSCH/PDCCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch occurs on the first DL or UL slot right after a time duration of TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length slots which begins from the beginning of DL slot n, where DL slot n is the last slot overlapping with the TTI containing the RRC command, and NR Slot length is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch if the BWP switch involves changing of SCS. TRRCprocessingDelay is the length of the RRC procedure delay in ms as defined in clause 11.2 in TS 36.331 [16] if the corresponding RRC message is embedded in E-UTRA RRC message, otherwise it is the length of the RRC procedure delay in ms as defined in clause 12 in TS 38.331 [2], and TBWPswitchDelayRRC=6ms is the time used by the UE to perform BWP switch.

  - Option 2 (QC R4-2119568)

    - Reason for change

      - The requirements for RRC based BWP switching delay are applicable for NA standalone, NE-DC, NR-DC, and EN-DC. For EN-DC, when the BWP switching signaling comes from LTE, RRC processing delay should refer to LTE 36.331. However, in the current requirement, the RRC processing delay refers to NR 38.331.

    - Summary of Changes

      - Differentiated RRC processing delay for EN-DC from non EN-DC.

    - Example of the Changes

For RRC-based BWP switch, after the UE receives RRC reconfiguration involving active BWP switching or parameter change of its active BWP, UE shall be able to receive PDSCH/PDCCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch occurs on the first DL or UL slot right after a time duration of TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length slots which begins from the beginning of DL slot n, where DL slot n is the last slot containing the RRC command, and NR Slot length is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch if the BWP switch involves changing of SCS. TRRCprocessingDelay is the RRC procedure delay in ms as defined in clause 11 in TS 36.331 [16] for EN-DC, and as defined in clause 12 in TS 38.331 [2] for non EN-DC, and TBWPswitchDelayRRC=6ms is the time used by the UE to perform BWP switch.

- Recommended WF

  - Further discuss option 1 or option 2 for RRC based BWP switching is agreeable

  - Further discuss if same change should be applied to other RRC based procedures as listed in option 1

| Company | Comments |
| --- | --- |
| Apple | We support the wording in option 1 to capture the different RRC processing delay that is applicable. In EN-DC the RRC re-configuration message could be received over NR or LTE and always LTE RRC processing delay would not apply for EN-DC case. We support updating the core requirements for other RRC based procedures to capture the correct delay and slot/subframe start for the switch. |
| MTK | Support Option 1. It’s correct to apply LTE RRC delay if the procedure is triggered by LTE. |
| Qualcomm | Support Option 1, and we can withdraw ours given that Option 1 has a broader coverage in terms of correction. |
| Huawei | Support option 1. Option 2 is similar as option 1 but we understand option 1 is more comprehensive as also mentioned by QC, and thanks to QC for the flexibility. |
| Ericsson | Option 1 is OK. |
| Nokia | The reference clarification of RRC procedure delay triggered by LTE and NR is fine. For the clarification on the starting slot/subframe for delay requirement counting, we do not see the necessary to add the wording “overlapping with the TTI containing…”, this is NR requirements, it is already very clear that the NR slot that containing the RRC command, and “TTI” here is not clear. |

Sub-topic 1-3: Idle mode mobility

Issue 1-3-1: FR2 cell reselection in Idle mode

- Proposals

  - Option 1 (Ericsson R4-2118401)

    - Reason for change

      - The spec. specifies that UE shall initiate cell selection procedures for the selected PLMN after 10s. However, considering Rx beam sweeping factor N1 in FR2, the evaluation time Nserv is farly larger than 10s which implies the UE will initiate cell selection for the selected PLMN regardless of UE finishing once serving cell evaluation in FR2.

    - Summary of Changes

      - Introduce the max function between the fixed value 10s and N1 scaling factor with DRX cycles.

If the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information for max(10s, M1*N1*K1*DRX cycle), the UE shall initiate cell selection procedures for the selected PLMN as defined in TS 38.304 [1], where: K1=[6] if DRX cycle is 0.32s or 0.64s, otherwise K1=[3].

- Recommended WF

  - Further discuss is option 1 is agreeable

| Company | Comments |
| --- | --- |
| Apple | Since the current definition is “If the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra- frequency, inter-frequency and inter-RAT information indicated in the system information for 10 s, the UE shall initiate cell selection procedures for the selected PLMN as defined in TS 38.304 [1].”, we think the max function shall be as max(10s, Tdetect,NR_Intra, Kcarrier*Tdetect,NR_Inter, NEUTRA_carrier *Tdetect,EUTRAN). |
| MTK | We agree with the intention to introduce the max function. In principle, fine with Option 1 as long as the values are in []. To our understanding, Option 1 only impacts on FR2 and no impact on FR1. Comment on Apple’s proposal: we have concern on the term “Tdetect,NR_Intra, Kcarrier*Tdetect,NR_Inter, NEUTRA_carrier *Tdetect,EUTRAN”, because it would be longer than 10s even in FR1, which is based on LTE requirements. |
| Qualcomm | In principle, okay with Option 1. |
| Apple | To MTK: The revision is to avoid that if the measurement/detection time is greater than 10s, UE may directly trigger the cell selection procedures before completing neighbor cell detection. We think we shall use same principle for both FR1 and FR2 and to consider all kinds of the neighbor cell measurement. We don’t understand what’s the problem if the term “Tdetect,NR_Intra, Kcarrier*Tdetect,NR_Inter, NEUTRA_carrier *Tdetect,EUTRAN” is greater than 10s. The definition in spec is: “If the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra- frequency, inter-frequency and inter-RAT information indicated in the system information for 10 s, the UE shall initiate cell selection procedures for the selected PLMN as defined in TS 38.304 [1].” So if e.g., inter-frequency measurement/detection period is greater than 10s, it makes sense to at least finish one round of inter-frequency measurement/detection and then initiate cell selection procedures. |
| Huawei | Suggest FFS We do not think max function is needed between 10s and Nserv consecutive DRX cycles, because UE should have already finished one round of serving cell measurement (and determined thtat S criterion is not met) before it initiates the measurements of neighbour cells. In this sense we tend to agree with Apple’s comments that if a max is to be introduced, it should be between 10s and the neighbor cell measurement time. However, since the time for neighbor cell measurement can be rather long, we would need more time to check whether and how to introduce such max function. |
| Ericsson | To Apple, - In LTE spec, the requirement is also 10s for normal DRX which works well for UE. Tdetect, Intra is the minimum requirements in Idle mode for power saving which will be always larger than 10s in LTE and FR1 and isn’t suitable for this ‘urgent case’(S criteria cannot be met for Nserv DRX cycles). We don’t think it’s reasonable to change the requirements of LTE. If the UE has evaluated according to Table 4.2.2.2-1 in Nserv consecutive DRX cycles that the serving cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting UE measurement activities. If the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information for 10 s, the UE shall initiate cell selection procedures for the selected PLMN as defined in TS 38.304 [1].. To Huawei, - In LTE eDRX mode, the requirement updates to T=MAX(10s, one eDRX_IDLE cycle) if the UE is configured with eDRX_IDLE cycle. 1 eDRX is permitted which equals the total evaluation time for serving cell and leaves enough margin after Nserv to avoid UE to initiate cell selection for selected PLMN frequently. - From our understanding, current equation allows UE to further perform serving cell evaluation and decide whether to trigger cell selection for selected PLMN. Thus, we don’t think it needs to update the equation by Tdetect. To move forward, we suggest to further discuss with the following equations(at least N1 shall be considered, because the issue is only for FR2) and RAN4 can focus on the detail value K1 later. If the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information for max(10s, N1*K1*DRX cycle), the UE shall initiate cell selection procedures for the selected PLMN as defined in TS 38.304 [1], where: K1 is FFS.. |
| Apple | To Ericsson: LTE is a little different from NR. UE has very much flexibility to perform measurement since PSS/SSS is available every 5ms and CRS is available every subframe, and therefore in 10s UE could coordinate to perform neighbor cell measurement and work well. However, in NR, all the measurement is associated with SSB periodicity, and the worst case could be 160ms SSB periodicity. We don’t understand why this UE behavior is relevant with serving cell measurement period. As Ericsson commented, UE has already performed the serving cell measurement, but the motivation for UE to initialize the cell selection instead of reselection is because UE cannot find suitable neighbor cell in a certain period. We agree to further discuss what the certain period is, but we believe that period shall be relevant with the neighbor cell measurement/identification rather than serving cell measurement period. If the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information for 10 s, the UE shall initiate cell selection procedures for the selected PLMN as defined in TS 38.304 [1].. |
| Nokia | We have different understanding on the need for this clarification. 38.133 states: ‘If the UE has evaluated according to Table 4.2.2.2-1 in Nserv consecutive DRX cycles that the serving cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting UE measurement activities.’ If this is fulfilled, then ‘If the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information for 10 s, the UE shall initiate cell selection procedures for the selected PLMN as defined in TS 38.304 [1].’ Hence, cell selection according to 38.304 is initiated. But we can discuss further, CR needs more discussion. |

Companies views’ collection for 1st round

Open issues

CRs/TPs comments collection

No need to repeat the comments if you have already provided comments to the related open issue in section 1.2. Comments on the exact wording can be provided here, if any.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2117441 (Apple) | Moderator: Related to 1-1-1 |
|  | Huawei: please see our comments to 1-1-1. |
| R4-2118323 (MTK) | Moderator: Related to 1-1-2 |
| R4-2119222 (MTK) | Moderator: Related to 1-1-3 |
| R4-2118247 (vivo) | Moderator: Change #3 related to 1-2-1 Moderator: Change #1 and Change #2 are not listed as open issue |
|  | Huawei: please see our comments to 1-2-1 for Change #3. We are fine with Change #1 and Change #2. |
|  | Nokia: Change #1 is OK. For Change #2, It would be better to refer to 38.331 for the RRC procedure delay to align with other requirements which including RRC procedure delay, like “TRRC_delay is the RRC procedure delay as specified in TS 38.331 [2].”. For Change #3, comments provided in issue 1-2-1. |
| R4-2118784 (HW) | Moderator: Related to 1-2-2 |
|  | Apple: Suggest the following changes to wording for better readability/clarity: ### 8.10.5 RRC based TCI state switch delay If the target TCI state is known, upon receiving PDSCH carrying RRC activation command at slot n, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ (TRRC_processing +TOk*(Tfirst-SSB + TSSB-proc)) / NR slot length. The UE is not required to receive PDCCH/PDSCH/CSI-RS or transmit PUCCH/PUSCH until the end of switching period. Where - n is the last slot overlapping with the PDSCH carrying RRC activation command - TRRC_processing is the RRC processing delay defined in Clause 11.2 of TS 36.331 [16] if the corresponding RRC message is embedded in E-UTRA RRC message, otherwise it is the RRC processing delay defined in Clause 12 of TS 38.331 [2] - Tfirst-SSB is time to first SSB transmission after RRC processing by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state - TSSB-proc and TOk are defined in clause 8.10.3. ## 8.11 PSCell Change This clause defines requirements for the delay within which the UE shall be able to change PSCell to other cell in EN-DC or NR-DC. The requirements in this clause are applicable to EN-DC and NR-DC. Upon receiving PSCell change in subframe n, where n is the last subframe overlapping with the TTI containing PSCell change, the UE shall be capable of transmitting PRACH preamble towards the target PSCell no later than specified in clause 8.9.2, where the following values for Tprocessing and TRRC_delay shall override the existing ones: Huawei: Thanks Apple for the suggestions. We are fine with them and we will capture them in the revision. |
| R4-2119568 (QC) | Moderator: Related to 1-2-2 |
|  | Apple: Can CR be merged with R4-2118784? |
|  | Qualcomm: Ours can be covered by R4-2118784. |
| R4-2118401 (Ericsson) | Moderator: Related to 1-3-1 |
|  | Huawei: please see our comments to 1-3-1. |
| R4-2119443 (Ericsson, Nokia, Intel, HW, QC) | Moderator: No discussion expected, this CR is handled in email #238 |

Summary for 1st round

Open issues

- Issue 1-1-1: Clarification for the case when SSB periodicity is greater than SMTC periodicity Tentative agreements: None Candidate options: - Option 1 (Apple, QC, HW) - Add clarification for the case when actual SSB periodicity is greater than SMTC periodicity for intra-frequency measurement without MG. - Option 2 (MTK, HW, E///) - Clarification in option 1 not needed. Recommendations for 2nd round: - Further discuss based on the CR between the two options - Please Apple also address the question from Nokia about definition of ‘actual SSB transmission periodicity’

- Issue 1-1-2: Timing change in event triggered reporting requirements Tentative agreements: Revise the maximum timing changed as 3200/2µ Tc, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211. CR R4-2118323 is agreeable Candidate options: Recommendations for 2nd round:

- Issue 1-1-3: Applicable DRX cycle for inter-frequency and inter-RAT measurmenet in NE-DC and NR-DC Tentative agreements: None Candidate options: - Option 1 (MTK, QC, HW, E///) - In NR-DC mode and NE-DC mode, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the longer DRX cycle between master cell group and secondary cell group. - Option 2 (Nokia) - The principle in option 1 to use longer DRX cycle between CGs is not agreeable. - Option 3 (vivo) - Up to UE implementation which DRX cycle is used. Recommendations for 2nd round: - Further discuss based on the CR between the two options - From moderator perspective, option 3 is similar to option 1 in terms of requirements. - It is noted that Option 1 is also applicable to inter-RAT measurement, and this is now reflected in the title of the issue. Thanks vivo for spotting.

- Issue 1-2-1: NE-DC PSCell addition requirements Tentative agreements: None Candidate options: - Option 1 (vivo, Apple, QC, Nokia) - Remove the RACH uncertantainty for Pcell in NE-DC PSCell addition requirements. - Option 2 (HW) - Option 1 is FFS, apply same conclusion from Rel-17 feRRM HO with PSCell from NE-DC to NE-DC Recommendations for 2nd round: - Further discuss based on the CR between the two options

- Issue 1-2-2: RRC delay for requirements of RRC-based procedures Tentative agreements: Add clarification to differentiate RRC processing delay triggered by LTE and NR Candidate options: Recommendations for 2nd round: - Please Huawei revise the CR to address the comments on the wording from Apple and Nokia

- Issue 1-3-1: FR2 cell reselection in Idle mode Tentative agreements: None Candidate options: - Option 1 (E///, MTK, QC) - Introduce the max function between the fixed value 10s and serving cell evaluation time, e.g. Nserv DRX cycles. - Option 2 (Apple, HW) - Introduce the max function between the fixed value 10s and neighbor cell detection time, e.g. Tdetect for intra-frequency, inter-frequency and inter-RAT measurement. - Option 3 (Nokia) - Need for the max function is FFS Recommendations for 2nd round: Further discuss based on the CR between the three options

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Sub-topic 1-1: Measurement requirements

Issue 1-1-1: Clarification for the case when SSB periodicity is greater than SMTC periodicity

| Company | Comments |
| --- | --- |
| Apple | Based on moderator assignment, we would like to trigger the discussion on the CR for SSB periodicity. The revised draft CR has been uploaded to:R4-21xxxxx Draft CR on clarification for SMTC R15_v01.docx According to the comments from QC in 1st round, we agree it’s a common clarification for all the RRM requirement in the spec, and therefore we move this clarification sentence to section 3.6. To MTK, Ericsson and HW‘s comments in 1st round, we also think this is an error case, but we think it’s necessary to clarify such error case is not an assumption in all requirements, like what we did for other error cases in e.g., section 3.6.9/10. To Nokia’s comments in 1st round, we revised the wording to just using “SSB transmission periodicity” to avoid ambiguity. Your further comments are welcome! |
| Nokia | Thanks for the clarification. With this clarification it seems clear to us that it addresses a configuration error from network side. In general we do not think RAN4 shall not make requirements for network configuration errors. It is common understanding that if network configures an SMTC the UE can assume that it will contain SSB. We do not think the clarification is needed. |
| QC | It seems no company disagree that actual SSB periodicity should not be longer than configured SMTC on the same carrier. Although what would be the case when multiple SMTCs are configured on one measurement carrier in NTN down the road, but at least for now, what is written in the revised version doesn’t look any different from typical requirement applicability rules that can be found in many places in RAN4 spec. If the wording makes companies a bit uncomfortable, we are okay with softening it a bit. But still applicability rules in RAN4 spec is not directly interpreted as an error case. That is nothing but cases where corresponding UE requirements/behavior are not defined. |
| Apple | To Nokia, Thanks for your comments! At least we both agree that this is an error case and no requirement shall be applied in this case. Our understanding is this configuration is very essential to all the RRM requirements, e.g., most of delay requirements are defined on SMTC level, so we are wondering if we could clearly capture such requirement applicability in the section 3 as a general assumption for the whole spec. Actually, RAN4 did similar clarification for scheduling/measurement restriction in section 3.6.9/3.6.10 as well. May I know if there is any technical concern to capture this clarification? Thanks! |
| Nokia | To Apple/QC: Thanks for the clarification. We see that all companies do agree that this is error case and hence we this then seen as being the common understanding among the companies. We do not really see why network would ever configure the UE with an SMTC within which there would be no SSB. The abbreviation of the is rather clear from RAN2 that SMTC is for measurement timing configuration. We agree and see this scenario as clear network configuration error case. And it is clear from the NR specification that network configures SMTC to include SSB. Hence, we still do not see a clear need to capture this in the specification. |
| MTK | We also support to capture this clarification in the general section . Although we also consider this is a clear error case since R15, we still see the need to capture this clarification, because there is similar clarification captured in R16 IDLE but leave other cases unclear. |
| Apple | I uploaded the final version of this CR (only added the tdoc number) to : [Final draft]R4-2120242 Draft CR on clarification for SMTC R15.docx I think companies have same understanding on the technical part but the key is whether we need to specify it or not. So I hope we could have a bit more discussion in GTW. Thanks! |

Issue 1-1-3: Applicable DRX cycle for inter-frequency measurmenet in NE-DC and NR-DC

| Company | Comments |
| --- | --- |
| MTK | Per Moderator’s guideline, this is to kick-off email discussion on CR for DRX cycle in use for inter-freq. and inter-RAT measurement requirement. Draft CR can now be found in the following link: https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_101-e/Inbox/Drafts/%5B101-e%5D%5B201%5D%20Maintenance_R15_NR_RRM_Core/Documents/draft%20of%20R4-2120246%20CR%20on%20TS38.133%20for%202%20DRX_v01.docx Reply to the Nokia’s comments: “Can MediaTek clarify whether this CR suggest that the measurement requirements for inter-frequency measurements on carrier1 configured e.g. by MCG could depend on either the DRX cycle of the MCG or SCG depending on which CG has the longest DRX cycle? Such an approach is not within the current spirit of the specification and the CR agreed in last meeting (R4-2115237) addressing a similar clarification.” The rules agreed in R4-2115237 are for intra-freq. measurement requirement, there exist only one DRX cycle for UE to refer to. However, in this CR, what we discuss are rules for the inter-freq. and inter-RAT measurement requirement. So there might exist 2 DRX cycles for UE to refer to, e.g., in EN-DC mode a FR 2 MO is configured by MN. Based on current spec, the inter-frequency measurements on NR configured by MCG already depend on the DRX cycle of SCG. Rel-15 ENDC; Applicable DRX. Measurement objects configured by MN; Inter-frequency LTE; Follow MCG DRX configuration and state. Inter-RAT NR (36.133 8.17.4); Follow SCG DRX configuration and state. Inter-RAT UTRA/ GSM; Follow MCG DRX configuration and state. Measurement objects configured by SN; Inter-frequency NR (38.133 9.3); Follow SCG DRX configuration and state. |
| Nokia | Thank you for the clarification. We still have concerns related to the proposal in this CR. Measurement objects are configured according to cell groups. DRX configurations are also defined according to cell groups. Therefore, the UE should follow the DRX configuration of the cell group that configured the inter-frequency measurement. This is already understood from RAN2 specification, and hence even though we raised this in last meeting, there is no clear need for clarification in TS 38133. What now is clear from your proposal: In NR-DC mode and NE-DC mode, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the longer DRX cycle between master cell group and secondary cell group Is that if UE is configured by MCG in FR1 to perform inter-f measurements on carrier in FR2 where UE also has a PSCell configured, then if e.g. MCG is in no DRX while PSCell is in DRX, you propose that the inter-frequency measurement requirements on the FR2 carrier shall be performed according to PSCell DRX. Even then MCG is in no DRX. As said this in not how we see the requirements. |
| MTK | Regarding the scenario you mentioned: “UE is configured by MCG in FR1 to perform inter-f measurements on carrier in FR2 where UE also has a PSCell configured, then if e.g. MCG is in no DRX while PSCell is in DRX, the inter-frequency measurement requirements on the FR2 carrier shall be performed according to PSCell DRX. Even then MCG is in no DRX” It seems to be not aligned with RAN2’s understanding and we understand your concern. However, this issue was raised in the very early discussion in Rel-15 and RAN4 had already reached similar consensus in the EN-DC mode (Rel-15 CR R4-1816109). Rel-15 ENDC; MO; Applicable DRX. Measurement objects configured by MN; Inter-RAT NR (36.133 8.17.4); Follow SCG DRX configuration and state. Our understanding is “UE should follow the DRX configuration of the cell group that configured the inter-freq. measurement” will be kind of reversing the RAN4 agreement in Rel-15. |
| Nokia | Thanks for the clarification. We still do not agree with the CR. Let me try to explain our view further. It makes sense that, by the time R4-1816109 was agreed, the note included only EN-DC cases. And in this case, in which the NR inter-frequency measurements are configured by SCG, it makes sense that the DRX requirements are those from SCG. This is aligned with our previous response, in which we stated that “the UE should follow the DRX configuration of the cell group that configured the inter-frequency measurement.” In NR-DC, however, NR inter-frequency measurements can be configured by the SCG or the MCG. Therefore, the DRX should be the one from the Cell Group associated with the measurement, as it was agreed for the intra-frequency case. In our view, no changes are needed in this case, because it would just be a clarification of RAN2 specification. The case mentioned in your last reply is for Inter-RAT NR measurements. Therefore, if any changes are needed, they should be proposed to clause 8.17.4 in TS 36133, and not in TS 38133. We are, however, not certain if changes are needed in this case. Anyway, despite this discussion, it is difficult to see how that relates to the proposed change: NOTE 3: In NR-DC mode and NE-DC mode, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the longer DRX cycle between master cell group and secondary cell group. This proposal seems only to account which CG has the longest DRX. And this seems to have negative impact on the UE performance requirements. We need more time to evaluate these issues. Related to the proposed change in the CR, more discussion is needed. |
| MTK | Considering that I have not received any further modification request regarding the CR content, so the latest version is my final version. Regarding the question raised by Nokia, we hope that further discussion in GTW can help us collect more views. Thanks a lot. |

Sub-topic 1-2: Signaling characteristic related requirements

Issue 1-2-1: NE-DC PSCell additio requirements

| Company | Comments |
| --- | --- |
| vivo | According to moderator's suggestion, this is to kick-off the email thread to further discuss the CR on NE-DC PSCell addition requirements. Based on comments in the 1st round, slightly majority is shown in approving the CR with removal of PRACH occasion uncertainty. Therefore, the draft version of the CR provides the same changes as R4-2118247. Please provide your comments based on the CR draft_R4-2120243 draft CR on requirements for PSCell change and NE-DC PSCell addition in R15.docx, with 1st round companies' discussion and replies in mind. I'll share the R16/R17 mirror CRs once the conclusion seems stable. |
| Nokia | Thanks for sharing the draft revised version. For Change #2, It would be better to refer to 38.331 for the RRC procedure delay to align with other requirements which including RRC procedure delay, like “TRRC_delay is the RRC procedure delay as specified in TS 38.331 [2].” The change is as below, please check if they are acceptable. Thanks. draft_R4-2120243 draft CR on requirements for PSCell change and NE-DC PSCell addition in R15_Nokia.docx |
| QC | It seems no company disagree that actual SSB periodicity should not be longer than configured SMTC on the same carrier. Although what would be the case when multiple SMTCs are configured on one measurement carrier in NTN down the road, but at least for now, what is written in the revised version doesn’t look any different from typical requirement applicability rules that can be found in many places in RAN4 spec. If the wording makes companies a bit uncomfortable, we are okay with softening it a bit. But still applicability rules in RAN4 spec is not directly interpreted as an error case. That is nothing but cases where corresponding UE requirements/behavior are not defined. |
| vivo | Thanks for the comments. We are OK to your revision. Companies are encouraged to provide further comments a.s.a.p, if any. |
| Huawei | Based on companies’ comments from first round, we can accept the change for removal of the PRACH occasion uncertainty, and we are also fine with revision from Delia. |
| vivo | Thanks for the confirmation! Given no further comments on the version shared by Nokia, we think the CR would be quite stable now. Please be informed that the final version of Cat F CR and Cat A CRs have been uploaded to the draft folder for final checking. Thanks for the good discussion. draft_R4-2120243 draft CR on requirements for PSCell change and NE-DC PSCell addition in R15_final.docx draft_R4-2118248 draft CR on requirements for PSCell change and NE-DC PSCell addition in R16_final.docx draft_R4-2118249 draft CR on requirements for PSCell change and NE-DC PSCell addition in R17_final.docx |

Issue 1-2-2: RRC delay for requirements of RRC-based procedures

| Company | Comments |
| --- | --- |
| Huawei | Based on the first round discussion, all companies supported to clarify that the RRC procedure delay can be different depending on whether it is triggered by LTE or NR, while there were some comments related to the detailed wording in the CR from Nokia and Apple. Below we provided some responses to the comments, and the corresponding changes can be found in the revision uploaded at draft_R4-2120245 revised CR on delay for RRC-based procedures.docx . Your further comments are welcomed. Thanks very much for Nokia's comments. The reason of using "TTI" is to cover both the case that NR RRC Msg is embeded in LTE RRC Msg and the case NR RRC Msg is directly sent to UE. As depicted by follow figures, "slot n" shall be the last NR slot fully or partially overlapping with the LTE subframe carrying RRC Msg for the former case, and it shall be the last NR slot which carries RRC Msg for the latter case. We use the word "TTI" to indicate "LTE subframe" or "NR slot". figure 1: NR RRC Msg is embeded in LTE RRC Msg figure 2: NR RRC Msg is directly sent to UE To avoid any potential ambiguity, we replaced "TTI" with "PDSCH", i.e. change the wording to "...the last NR slot overlaping with the PDSCH carrying RRC message" |
| QC | We support the revised version. |
| Nokia | Thanks for the revised CR. The revised version looks good. We have one more comment on section 8.11, should we change “subframe n” to “slot n”? |
| Huawei | Thanks for your careful review. We have updated the CR, where “subframe n” in section 8.11 is changed to “slot n”. The revision can be found in draft_R4-2120245 revised CR on delay for RRC-based procedures_v01.docx. Your further comments are welcomed. |
| Nokia | Thank you for considering our comments. We are fine with this version. |

Sub-topic 1-3: Idle mode mobility

Issue 1-3-1: FR2 cell reselection in Idle mode

| Company | Comments |
| --- | --- |
| Ericsson | Thanks for moderator’s summary! As suggested by moderator, this is the mail to kick off of the discussion on issue 1-3-1. Based on 1st round summary, most of companies believe the max function is needed, but the detail value is FFS. Thus, to move forward, we update the CR to only capture the general equation max(10s, K1*N1*DRX cycles), K1 is FFS in draft CR below. draft R4-2120244 draftCR on FR2 cell reselection in Idle mode - r15.docx To Nokia, From our understanding, as 38.133 states: ‘If the UE has evaluated according to Table 4.2.2.2-1 in Nserv consecutive DRX cycles that the serving cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting UE measurement activities.’. If the above condition is fulfilled, then the possible UE’s behaviours are as follow. - UE initiates all neighbour cells measurements - UE continue to evaluate serving cell’s quality However, in FR2, considering beam sweeping factor N1, UE cannot finish cell evaluation for at least once a time due to Rx beam sweeping. Thus, UE had to initiate cell selection procedures after 10s regardless of whether any suitable cell can be found or serving cell’s quality can be better than S criterion. ‘If the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information for 10 s, the UE shall initiate cell selection procedures for the selected PLMN as defined in TS 38.304 [1].’. To avoid frequent initiating the cell selection which will cause severe performance degradation, the evaluation time shall be updated to at least to permit UE finishing once evaluation time in FR2. |
| Nokia | Thanks for sharing the draft revised version. We still have different understanding. The current requirements are clear. Normal measurements are according to table 4.2.2.2-1. However, 'the UE shall initiate the measurements of all neighbour cells indicated by the serving cell' is not stated being according to 4.2.2.2-1 but being within 10 seconds. It is not clear what K1 is, we think more discussion is needed. |
| Ericsson | Considering Rx beam sweeping, 10s is not enough for FR2 measurements and evaluation. That means UE will always trigger the cell selection procedure even that UE haven’t finished the evaluation. K1 is the value for UE to finish the serving cell evaluation and neighbor cell measurements. |
| Nokia | Our understanding of the specification is the UE shall search for 10 second, if the UE has determined that the serving cell has not fulfilled the cell selection criteria for Nserv consecutive DRX cycles. The evaluation of the serving cell selection criteria is done according to the DRX cycles and table 4.2.2.2-1 (Nserv). However, the search following UE detecting that the cell selection criteria is no longer fulfilled is done for a period of 10 second and no more. How the UE actually implements this search is UE implementation specific. |
| Huawei | We have some concerns to make this change to Rel-15. Based on current requirements, UE will search and measure neighbor cell for 10s, and after that UE will initiate cell selection. If we change the requirements by adding a max function between 10s and some DRX cycles, it means UE should search and measure neighbor cell for a time period longer than 10s, as such a legacy UE may not be able to meet the requirements. Therefore, we suggest to further study the propose change for a later release. |
| Ericsson | Thanks for all your comments. We think Nokia and Huawei’s comments are valid and we can further discuss this issue and where to capture it if agreed. |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on remaining issues in Rel-15 NR RRM core requirements | Huawei, HiSilicon | Reserved, in case not all issues are resolvable in this meeting |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2117441 | Draft CR on clarification for intra-frequency measurement R15 | Apple | Revised |  |
| R4-2118247 | draft CR on requirements for PSCell change and NE-DC PSCell addition in R15 | vivo | Revised |  |
| R4-2118323 | Correction on timing change during measurement procedure in R15 | MediaTek inc. | Agreeable |  |
| R4-2118401 | draftCR on FR2 cell reselection in Idle mode -r15 | Ericsson | Revised |  |
| R4-2118784 | Correction to requirements of R15 NR RRC-based procedures_R15 | Huawei, Hisilicon | Revised |  |
| R4-2119222 | CR on TS38.133 for applicable DRX cycle in EN-DC, NR SA, NE-DC, and NR-DC | MediaTek inc. | Revised |  |
| R4-2119568 | Rel-15 Cat-F CR to RRC based BWP switching delay on single CC in EN-DC | Qualcomm Incorporated | Merged |  |

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2120242 | Draft CR on clarification for intra-frequency measurement R15 | Apple | Return to | suggested to be discussed in GTW |
| R4-2120243 | draft CR on requirements for PSCell change and NE-DC PSCell addition in R15 | vivo | Agreeable |  |
| R4-2118401 | draftCR on FR2 cell reselection in Idle mode -r15 | Ericsson | Postpone |  |
| R4-2120244 | draftCR on FR2 cell reselection in Idle mode -r15 | Ericsson | Withdraw |  |
| R4-2120245 | Correction to requirements of R15 NR RRC-based procedures_R15 | Huawei, Hisilicon | Agreeable |  |
| R4-2120246 | CR on TS38.133 for applicable DRX cycle in EN-DC, NR SA, NE-DC, and NR-DC | MediaTek inc. | Return to | suggested to be discussed in GTW |
| R4-2120241 | WF on Rel-15 NR RRM core requirements maintenance | Huawei, Hisilicon | Return to | pending on outcome of 0242 and 0246 |

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)