3GPP TSG-RAN WG4 Meeting # 101-e												R4-2120609

Electronic Meeting, 01 – 12 November, 2021

Agenda item:			8.6.1

Source:	Moderator (Ericsson)

Title:	Email discussion summary for [101-e][308] NR_DL1024QAM_BSRF

Document for:	Information

# Introduction

This thread deals with the remaining open issue for BS core RF (EVM for 1024QAM). Also, in this meeting TUs are allocated for conformance and a number of BS conformance issues are presented for discussion. The thread also contains the UE core CR and an open issue for UE conformance.

The EVM for 1024QAM should be resolved this meeting so that the core can be completed. In this case, the BS and UE core CRs should be proposed for agreement.

As much progress as possible should be made for the conformance issues.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: EVM for 1024QAM, MCS for UE RMC

- 2nd round: Resolve conformance issues, update core CRs as needed. Possibly even finalize conformance CRs.

# Topic #1: Basestation TX RF requirements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2118915 | Nokia, Nokia Shanghai Bell, Ericsson, Verizon, KDDI, SoftBank, NTT DOCOMO, AT&T, SK Telecom, T-Mobile USA | Proposal: To agree option 1 with 2.5% EVM BS Tx requirement for 1024QAM. |
| R4-2117393 | CATT | Observation 1: The crossover SNR for rank 1 with 2.8% TX EVM and 2.5% TX EVM in TDL-A is 29.89dB and 28.54dB respectively. Observation 2: The throughput gain of 1024QAM compared to 256QAM for rank 1 with 2.8% TX EVM and 2.5% TX EVM in TDL-A is 20.2% and 22.6% respectively. Observation 3: The 2.8% TX EVM compared to 2.5% TX EVM only losses 1.35dB SNR gain and 2.4% throughput gain, which is not obvious, but it is benefit for product implementation for NR. Proposal 1: To adopt Option 3: 2.8% for all BS classes. |
| R4-2117873 | Huawei, HiSilicon | Observation: from link level simulation, for RANK1 even 3% TX EVM can provide observed gain for 1024 QAM compared to 256 QAM. |
| R4-2117874 | Huawei, HiSilicon | Proposal: 2.8% TX EVM for all BS classes |
| R4-2118027 | Intel | Observation 1: With Tx EVM = 2.5%, 1024QAM overperforms 256QAM at at most 2dB lower SNR comparing to Tx EVM =2.8% Proposal 1: Define Tx EVM requirements equal to 2.5% |
| R4-2118084 | NEC | Proposal 1: To agree option 1, 1024QAM EVM to be 2.5% for all classes. |
| R4-2118913 | Nokia | Observation 1: Simulation results for 1024QAM show higher throughput compared to 256QAM for higher, but still reasonable SNR conditions and reasonable EVM requirements. Observation 2: Simulation results for 1024QAM show that 3% Tx EVM reduces performance in many simulation cases, in some even below 256QAM. Proposal 1: We support option 1 i.e. 2.5% for all BS classes as BS Tx EVM requirement. |
| R4-2112192 | ZTE | Proposal 1: support the option 2 as the compromised solution |

## Open issues summary

### Sub-topic 1-1 EVM for 1024QAM

The remaining issue for the BS RF is to decide the EVM core requirement for 1024QAM. Some companies propose 2.5% for all classes, arguing that simulations demonstrate a benefit for 1024QAM with 2.5% compared to 2.8% and this is used for LTE. Other companies argue that the additional benefit shown by simulations is small and that some NR related differences to LTE (e.g., higher channel bandwidth, AAS with a large number of transmitters, higher frequencies) necessitate agreement of 2.8% EVM.

Issue 1-1: EVM for 1024QAM

- Proposals

  - Option 1: 2.5% for all classes (Nokia, Nokia Shanghai Bell, Ericsson, Verizon, KDDI, SoftBank, NTT DOCOMO, AT&T, SK Telecom, T-Mobile USA, Intel, NEC)

  - Option 2: 2.5% for LA and MR classes; 2.8% for WA class (ZTE)

  - Option 3: 2.8% for all BS classes (Huawei, HiSilicon, CATT)

- Recommended WF

## Companies views’ collection for 1st round

### Open issues

Sub topic 1-1 EVM for 1024QAM

| Company | Comments |
| --- | --- |
| Huawei | Lower EVM values will result in more power back-off at the cost of decreased coverage and increased cost. To consider the differences to LTE in higher frequencies, wider channel bandwidth and larger number of TRX, we propose 2.8% as the minimum requirements. |
| ZTE | We would like to know the needed power back level from other vendors to meet 2.5% EVM requirements, if power backoff is too larg, the overall system performance would be impacted at the end. Indeed this declaration parameter is also critical for operators deployment. |
| Qualcomm | We support option 1 as we do not see a need to relax the requirements compared to LTE. |
| SoftBank | To Huawei/ZTE: Judging from the responses in Issue 2-2 below, it seems that two companies want to have an option to declare backoff. For better understanding of the discussion here, a clarification is likely on the relation between relaxed EVM/questions above and the backoff permitted in Issue 2-2: for example, do you intend to prohibit (or limit the amount of) backoff if we adopt 2.8% EVM? We still prefer Option 1 and have a concern if a relaxed EVM is proposed with backoff possible to be declared since the combination of relaxed EVM and backoff sounds like just a relaxation comparing to LTE equivalent. |
| AT&T | Option 1. From the contributions provided, there does not seem to be any justification for relaxation of the requirements compared to LTE. |
| CATT | We should not just repeat the previous statement to reuse LTE’s 3.5% EVM since5G NR and LTE are different systems; wider channel bandwidth will bring larger challenges for NR. And it is found that the difference in terms of performance gain is marginal between 2.5% and 2.8%. So we think 2.8% is a reasonable value for NR. |
| KDDI | We support option 1. |
| Nokia | We support option 1. We don’t see real arguments to relax EVM requirement compare to LTE. Power backoff is anyway agreed to be allowed to declare by BS vendors. |
| China Unicom | We support option 3. We know that 2.8% EVM means that the BS can transmit in higher Tx power that will have additional benefit for network coverage. |
| Docomo | We prefer Option 1. It seems to be relaxed compared to LTE, if we accept the relaxed EVM and additional power back off. |
| T-Mobile USA | We support Option 1. The simulation results show that small increases in Tx EVM result in significant changes in throughput. While there are wider bandwidths in NR than LTE for some bands, LTE had 5 CC carrier aggregation in the wider bands. Operators need NR to perform at least as well as LTE. Cell coverage is usually uplink limited, so we don’t agree with the concerns raised about power backoff (if needed) impacting coverage. Lower EVM will help operators maximize throughput. |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2117875 | CR for TS 38.104: 1024QAM, Huawei, HiSilicon |
|  | Moderator: The CR should be presented for approval after the EVM is resolved. In the meantime, any other comments on the CR should be added here. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1 | Tentative agreements: The following tentative agreement was discussed in the GTW session, to checked in the 2nd round: Tentative Agreement: DL 1024QAM with 2.5% EVM for all BS classes, - Ffurther discuss in conformance test part whether any limitation needed for power back-off during test Comeback in final round to make final agreement for this issue by end of this meeting. Recommendations for 2nd round: Check if the tentative agreement can be agreed. Companies are welcome to provide comments in this document or in a GTW session. |

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2117875 | Revise |

## Discussion on 2nd round (if applicable)

Tentative Agreement:

DL 1024QAM with 2.5% EVM for all BS classes,

  - Ffurther discuss in conformance test part whether any limitation needed for power back-off during test

Comeback in final round to make final agreement for this issue by end of this meeting.

| Company | Comments |
| --- | --- |

# Topic #2: Basestation conformance

This topic deals with BS conformance topics

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117394 | CATT | Proposal 1: EVM test requirement for 1024QAM should equal to the EVM requirement for 1024QAM in TS 38.104 + 1%. Proposal 2: To define the following test model for 1024 QAM - NR-FR1-TM2b with single 1024QAM PRB allocation - NR-FR1-TM3.1b with all 1024QAM PRBs allocation. Proposal 3: To support up to three rated output power declaration for 1024QAM capable BS. |
| R4-2118914 | Nokia, Nokia Shanghai Bell | Observation 1: Updates are needed in both conducted test specification TS 38.141-1 and radiated test specification TS 38.141-2. Observation 2: No need to modify test specification 38.141-1 for RE power control dynamic range due to 1024QAM introduction. Proposal 1: To modify manufacturers declarations in test specification 38.141-1 and 38.141-2 to allow separate declaration for 1024QAM. Proposal 2: To define test EVM requirement as EVM core requirement + 1%. Proposal 3: To add additional test model 2b and 3.1b for 1024QAM. Proposal 4: To update procedures for TPDR test and EVM test for 1024QAM. |

## Open issues summary

### Sub-topic 2-1 EVM test tolerance

Sub-topic description: Test tolerance applied to EVM requirement

Open issues and candidate options before e-meeting:

Issue 2-1: EVM test tolerance

- Proposals

  - Option 1: 1%

- Recommended WF

  - Agree 1% ?

### Sub-topic 2-2 Declarations

Sub-topic description: Update of power back-off declaration for 1024QAM

Open issues and candidate options before e-meeting:

Issue 2-2: Declaration

- Proposals

  - (Nokia, CATT):

    - If a BS is capable of 1024QAM DL operation then up to three rated output power declarations may be made. One declaration is applicable when configured for 1024QAM transmissions, a different declaration is applicable when configured 256QAM transmissions and the other declaration is applicable when configured neither for 256QAM nor 1024QAM transmissions.

- Recommended WF

  - Is this declaration agreeable (adapted for conducted and radiated appropriately) ?

### Sub-topic 2-3 Test models

Sub-topic description: Test model updates for 1024QAM

Open issues and candidate options before e-meeting:

Issue 2-3: Test models

- Proposals

  - (Nokia, CATT):

    - NR-FR1-TM2b with single 1024QAM PRB allocation

    - NR-FR1-TM3.1b with all 1024QAM PRBs allocation.

    - Detailed TM descriptions are provided in R4-2117394 and R4-2118914

- Recommended WF

  - Are the test models agreeable ?

### Sub-topic 2-4 Total Power Dynamic Range and EVM procedures

Sub-topic description : Update of procedures for total power dynamic range and EVM

Open issues and candidate options before e-meeting:

Issue 2-4: Total power dynamic range and EVM

- Proposals

  - (Nokia, CATT):

- Test model configuration for total power dynamic range:

- For BS transmitting a signal at max power, set the BS to transmit a signal according to:

- -	NR-FR1-TM3.1b if 1024QAM is supported by BS without power back off, or

- -	NR-FR1-TM3.1a if 1024QAM is supported by BS with power back off, or

- -	NR-FR1-TM3.1a if 1024QAM is not supported by BS.

- For BS transmitting a signal at min power, set the BS to transmit a signal according to:

- -	NR-FR1-TM2b if 1024QAM is supported by BS, or

- -	NR-FR1-TM2a if 1024QAM is not supported by BS.

- Test model configuration for modulation quality:

- Using the corresponding test models:

- -	NR-FR1-TM3.1b if 1024QAM is supported by BS without power back off, or

- -	NR-FR1-TM3.1b at manufacturer's declared rated output power if 1024QAM is supported by BS with power back off, and NR-FR1-TM3.1a at maximum power.

- Recommended WF

  - Are the proposed updates acceptable ?

## Companies views’ collection for 1st round

### Open issues

Topic 2 Basestation conformance

| Company | Comments |
| --- | --- |
| Huawei | Issue 2-1: ok with the option Issue 2-2: agree Issue 2-3: agree Issue 2-4: for the procedures, the case that both 256 QAM and 1024 QAM with power back off is not included. Shall we consider it? Others: |
| ZTE | Issue 2-1: EVM test tolerance Fine with option 1 Issue 2-2: Declaration agree Issue 2-3: agree Issue 2-4: The following two potential case is missed in the above proposal, we need further discussion on how to accommodate these following cases. - NR-FR1-TM3.1 if 256QAM is supported by BS with power back off, or - NR-FR1-TM3.1 if 256QAM is not supported by BS. |
| Qualcomm | Issue 2-1: ok with option 1. Issue 2-2: Agree with the proposed declaration. Issue 2-3: Agree with the test models. Issue 2-4: We agree with the proposed test scenarios. Including the 256 QAM options along with the 1024 QAM with and without power backoff is not necessary in our opinion. Others: |
| Ericsson | Issue 2-1: Agree Issue 2-2: Proposal seems similar to what has been implemented in draft CR R4-2118780. If understanding is different please indicate how the text shall be captured differently Issue 2-3: Yes, TM need to be updated in draft CR R4-2118780. Since a revision is also needed due to cover page issues perhaps we can incorporate these proposals described in R4-2117394 and R4-2118914 already in this meeting? Issue 2-4: See comment from Issue 2-2. Others: |
| AT&T | Issue 2-1: Agree with option 1. Same EVM tolerance used for LTE. Issue 2-2: Agree with the proposed declaration. Seems to match the declaration used for LTE. |
| CATT | Issue 2-1: Agree with the moderator recommendation. Issue 2-2: Agree Issue 2-3: Agree Issue 2-4: Agree Others: |
| Nokia | Issue 2-1: Support option 1. Issue 2-2: Agree Issue 2-3: Agree Issue 2-4: In general OK, but it would be good to work on draft CRs as there is quite many details to be checked for 1024QAM especially in procedures for TPDR and EVM tests. There is submitted draft R4-2118780 for 38.141-2, that is good starting point. There could be also prepared draft CR for 38.141-1 this meeting if companies agree. |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2118780 | Draft CR to TS 38.141-1: Introduction of 1024 QAM in FR1 Ericsson |
|  | Moderator: Note the title says 38.141-1, but the CR is to 38.141-2. The CR could be updated and proposed for agreement if the core EVM and all conformance issues are agreed. In the meantime, companies are requested to provide any other update on the CR. A CR for 38.141-1 is also needed. Companies are requested to present views on whether, assuming all of the open issues are agreed a CR for 38.141-1 can also be generated and agreed in this meeting. In the event that all issues are not agreed or there is no 38.141-1 CR then agreement of the CR will be postponed until the next meeting. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Test tolerance for EVM is 1% The following text will be included regarding power backoff declatation: - If a BS is capable of 1024QAM DL operation then up to three rated output power declarations may be made. One declaration is applicable when configured for 1024QAM transmissions, a different declaration is applicable when configured 256QAM transmissions and the other declaration is applicable when configured neither for 256QAM nor 1024QAM transmissions. The following test models will be introduced: - NR-FR1-TM2b with single 1024QAM PRB allocation - NR-FR1-TM3.1b with all 1024QAM PRBs allocation. - Detailed TM descriptions are provided in R4-2117394 and R4-2118914 Candidate options: The following is almost agreeable for the test model configuration. During the discussion, there was a question whether an additional configuration is needed for a BS that supports both 256QAM and 1024QAM - Test model configuration for total power dynamic range: - For BS transmitting a signal at max power, set the BS to transmit a signal according to: - - NR-FR1-TM3.1b if 1024QAM is supported by BS without power back off, or - - NR-FR1-TM3.1a if 1024QAM is supported by BS with power back off, or - - NR-FR1-TM3.1a if 1024QAM is not supported by BS. - For BS transmitting a signal at min power, set the BS to transmit a signal according to: - - NR-FR1-TM2b if 1024QAM is supported by BS, or - - NR-FR1-TM2a if 1024QAM is not supported by BS. - Test model configuration for modulation quality: - Using the corresponding test models: - - NR-FR1-TM3.1b if 1024QAM is supported by BS without power back off, or - - NR-FR1-TM3.1b at manufacturer's declared rated output power if 1024QAM is supported by BS with power back off, and NR-FR1-TM3.1a at maximum power. Recommendations for 2nd round: Clarify whether the test configurations can be used as a basis for the conformance specification text or further clarification is needed for a BS supporting both 1024QAM and 256QAM. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2118780 | Note |

## Discussion on 2nd round (if applicable)

Is the following text sufficient as a baseline for the conformance specifications or should text be included for the case that the BS supports 256QAM and 1024QAM ?

- Test model configuration for total power dynamic range:

- For BS transmitting a signal at max power, set the BS to transmit a signal according to:

- -	NR-FR1-TM3.1b if 1024QAM is supported by BS without power back off, or

- -	NR-FR1-TM3.1a if 1024QAM is supported by BS with power back off, or

- -	NR-FR1-TM3.1a if 1024QAM is not supported by BS.

- For BS transmitting a signal at min power, set the BS to transmit a signal according to:

- -	NR-FR1-TM2b if 1024QAM is supported by BS, or

- -	NR-FR1-TM2a if 1024QAM is not supported by BS.

- Test model configuration for modulation quality:

- Using the corresponding test models:

- -	NR-FR1-TM3.1b if 1024QAM is supported by BS without power back off, or

- -	NR-FR1-TM3.1b at manufacturer's declared rated output power if 1024QAM is supported by BS with power back off, and NR-FR1-TM3.1a at maximum power.

| Company | Comments |
| --- | --- |

# Topic #3: UE RF

This topic deals with UE RF topics

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117392 | CATT | Proposal 1: The MCS index 23 in 1024QAM mcs table can be adopted for RMCs for maximum input level for 1024QAM. |

## Open issues summary

### Sub-topic 3-1 RMC

Sub-topic description: MCS index for RMC

Open issues and candidate options before e-meeting:

Issue 3-1: MCS index for RMC

- Proposals

  - Option 1: MCS 23 (CATT)

- Recommended WF

  - Agree MCS 23 ?

## Companies views’ collection for 1st round

### Open issues

Sub topic 3-1 MCS index for RMC

| Company | Comments |
| --- | --- |
| Ericsson | Issue 3-1: Agree to consider MCS23 for FRC for max input level with 1024QAM, since the effective CR is 0.78 which is close to the existing RMC for the maximum input level with 256QAM. |
| AT&T | Issue 3-1: Agree with Option 1. |
| CATT | Agree with Option 1 |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2117876 | UE RX RF requirements for 1024QAM for NR FR1 (Huawei, HiSilicon) |
|  | Moderator: Once the BS core is agreed and BS CR updated, this CR can be proposed for agreement with FFS to close the core. If the RMC MCS is agreeable (and the BS core CR is agreeable) then the CR could be updated with the MCS and presented for agreement. Please provide any other comments |
|  | Ericsson: Technically ok, but we prefer to put ‘A.3.2.x’ instead of ‘FFS’ in Table 7.4-1 Note 5 and Table 7.4A.1-1 Note 4. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: MCS index for RMC is 23 for maximum input level for 1024QAM |

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2117876 | Revise |

## Discussion on 2nd round (if applicable)

No further discussion needed

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on 1024QAM conformance requirements | CATT | Capture the agreements on conformance requirements. The CRs should then be discussed next meeting. |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2117875 | CR for TS 38.104: 1024QAM | Huawei, HiSilicon | Revise |  |
| R4-2118780 | Draft CR to TS 38.141-1: Introduction of 1024 QAM in FR1 Ericsson | Ericsson | Note | Conformance CRs will be discussed next meeting |
| R4-2117876 | UE RX RF requirements for 1024QAM for NR FR1 | Huawei, HiSilicon | Revise |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-211xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-211xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-211xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| SoftBank | Kenichi Kihara | kenichi.kihara@g.softbank.co.jp |
| Nokia | Bartlomiej Golebiowski | bartlomiej.golebiowski@nokia.com |
| T-Mobile USA | Bill Shvodian | bill.shvodian@t-mobile.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)