3GPP TSG-RAN WG4 Meeting #101-e	R4-2120699

Electronic Meeting, 1 November – 12 November 2021

Agenda Item:	8.6.5.1

Source:	Ericsson

Title:	WF on DL1024QAM demodulation and CQI reporting requirements

Document for:	Approval

# 1	PDSCH demodulation requirements

- Define new PDSCH requirements for 1024 QAM by using the existing new applicability rule.

- Interested companies are encouraged to provide the PDSCH simulation results according to Table 1 in the next meeting to decide the test parameters.

Table 1	Simulation parameters for PDSCH demodulation for DL1024QAM.

| Parameters | FDD | TDD |
| --- | --- | --- |
| SCS/CBW | 15kHz and 10MHz | 30kHz and 40MHz |
| TDD UL/DL configuration | N/A | 7D1S2U, with S=6:4:4 FFS to schedule PDSCH in the special slots |
| MCS (See appendix) | Option 1: MCS23 Option 2: MCS24 Option 3: MCS25 |  |
| Rank | 1 |  |
| Antenna configuration | 2x2 and 2x4 with ULA Low |  |
| Propagation channel | Option 1: TDLA30-10 Option 2: TDLD30-5 |  |
| PDSCH configuration | Type A mapping, Start symbol 2, Duration 12 |  |
| PDSCH DMRS configuration | Type 1, Single symbol, additional DMRS: pos1 |  |
| Tx EVM assumed for simulation | Option 1: 2.0% Option 2: 2.5% Option 3: 2.8% Note: The assumed Tx EVM should be same or less than the BS Tx EVM requirements. |  |
| SSB configuration | Periodicity 20 ms, Allocated in first slot within 20ms |  |
| TRS configuration | 20 ms periodicity, 2 slots, Offset 10 ms |  |
| Number of HARQ processes | 4 for FDD 15kHz, 8 for TDD 30kHz |  |
| Transform precoding | CP-OFDM |  |
| Allocated RBs | Full BWP |  |
| PRB bundling | 2 |  |
| Precoding model | Random Precoding, per slot, WB granularity (codebook configuration Single panel Type 1) |  |
| Receiver type | MMSE-IRC |  |
| Test metric | 70% of maximum throughput |  |

# 2	SDR requirements

- Define SDR requirements for 1024 QAM with rank 1 and rank 2.

- Add the MCS indexes for SDR test with 1024QAM based on Table 2.

- Interested companies are encouraged to propose MCSpracticalj to decide MCSreqj, where

  - MCSreqj= min⁡(MCSupperboundj, MCSpracticalj)

- Simulation parameters: TS38.101-4 Table 5.5A-1

Table 2	MCS indexes for SDM test with 1024QAM

| vLayersj | Qmj | fj | MCSupperboundj | MCSpracticalj | MCSreqj |
| --- | --- | --- | --- | --- | --- |
| 1 | 10 | 1 | 25 | TBD | TBD |
| 1 | 10 | 0.8 | 21 |  | TBD |
| 1 | 10 | 0.75 | 19 |  | TBD |
| 1 | 10 | 0.4 | 9 |  | TBD |
| 2 | 10 | 1 | 25 | TBD | TBD |
| 2 | 10 | 0.8 | 21 |  | TBD |
| 2 | 10 | 0.75 | 19 |  | TBD |
| 2 | 10 | 0.4 | 9 |  | TBD |

# 3	CQI reporting tests

- Define new CQI definition requirements for CQI table with 1024QAM entries (See Appendix).

- Interested companies are encouraged to provide the simulation results to decide the SNR test point and rank.

  - Test parameters: Reuse the existing CQI definition tests

  - Rank

    - Option 1: 1

    - Option 2: 2

  - Tx EVM assumed for simulation: Aligned with PDSCH demodulation requirements.

  - Test metric: Reuse the existing CQI definition tests

  - Test points: One SNR test point corresponding to CQI index 14/15

# Appendix

## MCS table with 1024QAM entries

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 193 | 0.3770 |
| 2 | 2 | 449 | 0.8770 |
| 3 | 4 | 378 | 1.4766 |
| 4 | 4 | 490 | 1.9141 |
| 5 | 4 | 616 | 2.4063 |
| 6 | 6 | 466 | 2.7305 |
| 7 | 6 | 517 | 3.0293 |
| 8 | 6 | 567 | 3.3223 |
| 9 | 6 | 616 | 3.6094 |
| 10 | 6 | 666 | 3.9023 |
| 11 | 6 | 719 | 4.2129 |
| 12 | 6 | 772 | 4.5234 |
| 13 | 6 | 822 | 4.8164 |
| 14 | 6 | 873 | 5.1152 |
| 15 | 8 | 682.5 | 5.3320 |
| 16 | 8 | 711 | 5.5547 |
| 17 | 8 | 754 | 5.8906 |
| 18 | 8 | 797 | 6.2266 |
| 19 | 8 | 841 | 6.5703 |
| 20 | 8 | 885 | 6.9141 |
| 21 | 8 | 916.5 | 7.1602 |
| 22 | 8 | 948 | 7.4063 |
| 23 | 10 | 805.5 | 7.8662 |
| 24 | 10 | 853 | 8.3301 |
| 25 | 10 | 900.5 | 8.7939 |
| 26 | 10 | 948 | 9.2578 |
| 27 | 2 | reserved |  |
| 28 | 4 | reserved |  |
| 29 | 6 | reserved |  |
| 30 | 8 | reserved |  |
| 31 | 10 | reserved |  |

## CQI table with 1024QAM entries

| CQI index | modulation | code rate x 1024 | efficiency |
| --- | --- | --- | --- |
| 0 | Out of range |  |  |
| 1 | QPSK | 78 | 0.1523 |
| 2 | QPSK | 193 | 0.377 |
| 3 | QPSK | 449 | 0.877 |
| 4 | 16QAM | 378 | 1.4766 |
| 5 | 16QAM | 616 | 2.4063 |
| 6 | 64QAM | 567 | 3.3223 |
| 7 | 64QAM | 666 | 3.9023 |
| 8 | 64QAM | 772 | 4.5234 |
| 9 | 64QAM | 873 | 5.1152 |
| 10 | 256QAM | 711 | 5.5547 |
| 11 | 256QAM | 797 | 6.2266 |
| 12 | 256QAM | 885 | 6.9141 |
| 13 | 256QAM | 948 | 7.4063 |
| 14 | 1024QAM | 853 | 8.3301 |
| 15 | 1024QAM | 948 | 9.2578 |