3GPP TSG-RAN WG4 Meeting #101-e															R4-2120704

Electronic Meeting, November 1st – 12th, 2021

Agenda item:			8.9.5.1

Source:	Intel Corporation

Title:	Simulation assumption for PDSCH requirement for FR2 HST

Document for:	Approval

# Background

This contribution is to capture the agreed PDSCH simulation assumptions for the further analysis and discussions on HST FR2 PDSCH demodulation performance requirements definition.

The detailed discussions can be found in the e-mail discussion summary R4-2120736

# PDSCH simulation assumptions

Simulation assumptions for results alignment and further requirements definition are summarized in Table 1.

Table 1. PDSCH Simulation assumptions

| Parameter |  |  | Value |
| --- | --- | --- | --- |
| Carrier frequency |  |  | 30 GHz |
| Duplex mode |  |  | TDD |
| SCS |  |  | 120 kHz |
| CBW |  |  | 200 MHz |
| Antenna configuration |  |  | 2x2 ULA Low |
| TDD pattern |  |  | DDDSU, S: 10D + 2G + 2U Schedule PDSCH in special slots (Note 1) |
| PDSCH DMRS configuration | DMRS type |  | Type 1 |
|  | Number of additional DMRS |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | Starting symbol (S) |  | 1 |
|  | Length (L) |  | 13 |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RGB size |  | Config2 |
| CSI-RS for tracking | Resource set #1 | First OFDM symbol in the PRB used for CSI-RS | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
|  |  | CSI-RS density | 3 |
|  |  | CSI-RS periodicity | 80 or 160 slots for CSI-RS resource 1,2,3,4 (Note1) |
|  |  | CSI-RS offset | 2 for CSI-RS resource 1 and 2 3 for CSI-RS resource 3 and 4 |
|  |  | Frequency occupation | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |
|  |  | QCL info | TCI state #2 |
|  | Resource set #2 | First OFDM symbol in the PRB used for CSI-RS | l0 = 6 for CSI-RS resource 5 and 7 l0 = 10 for CSI-RS resource 6 and 8 |
|  |  | CSI-RS density | 3 |
|  |  | CSI-RS periodicity | 80 or 160 slots for CSI-RS resource 5,6,7,8 (Note1) |
|  |  | CSI-RS offset | 2 for CSI-RS resource 5 and 6 3 for CSI-RS resource 7 and 8 |
|  |  | Frequency occupation | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |
|  |  | QCL info | TCI state #3 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource | CSI-RS for resource 1 from “CSI-RS for tracking Resource set #1” configuration |
|  |  | QCL type | Type A |
|  | Type 2 QCL information | CSI-RS resource | CSI-RS for resource 1 from “CSI-RS for tracking Resource set #1” configuration |
|  |  | QCL type | Type D |
| TCI state #1 | Type 1 QCL information | CSI-RS resource | CSI-RS for resource 5 from “CSI-RS for tracking Resource set #2” configuration |
|  |  | QCL type | Type A |
|  | Type 12QCL information | CSI-RS resource | CSI-RS for resource 5 from “CSI-RS for tracking Resource set #2” configuration |
|  |  | QCL type | Type D |
| TCI state #2 | Type 1 QCL information | SSB index | SSB #0 |
|  |  | QCL type | Type C |
|  | Type 2 QCL information | SSB index | SSB #0 |
|  |  | QCL type | Type D |
| TCI state #3 | Type 1 QCL information | SSB index | SSB #1 |
|  |  | QCL type | Type C |
|  | Type 2 QCL information | SSB index | SSB #1 |
|  |  | QCL type | Type D |
| PTRS configuration | Frequency density (KPT-RS) |  | 2 |
|  | Time density (TPT-RS) |  | 1 |
| MCS and Rank |  |  | MCS 17 and Rank 2 (Note 2) |
| Number of HARQ processes |  |  | 8 |
| SSB and CSI-RS for tracking assumptions |  |  | 2 SSBs associated with 2 CSI-RS resources sets for PDSCH requirement, where SSB # (k mod 2), CSI-RS (for tracking) resource set # ((k mod 2) + 1), CSI-RS (for CSI acquisition) resource set # ((k mod 2) + 3) and CSI-RS (for beam refinement) resource set # ((k mod 2)+5) are transmitted by kth RRH |
| SSB periodicity |  |  | 20ms |
| Propagation conditions |  |  | Uni-directional Scenario A and B, Bi-directional Scenario B The details can be found in [R4-2120702] |
| Maximum Doppler shift |  |  | 9722, 7000 and 5652 Hz (Note 1) |
| Test metric |  |  | SNR at 70% of maximum throughput |
| Note 1: For further study Note 2: As a starting point Note 3: Other remaining parameters can be found in TS 38.101-4 Table 7.2-1 and Table 7.2.2.2.1-2 |  |  |  |