3GPP TSG-RAN WG4 Meeting #101-e	R4-2120761

# Electronic meeting, November 1st – 12th, 2021	(Revised from R4-2118157)

Source:		Ericsson

Title:			TP to TR  38.863 - Regulatory aspects

Agenda item:		8.13.1.3

Document for:		Approval

# Introduction

The NTN WI ([1]) has been approved in RAN#88e meeting to specify requirements for the support of NTN. It has been revised in last RAN#99-e meeting [2].

In last RAN4#100-e meeting, the TR skeleton was approved. This contribution is a text proposal to TR focusing on the Regulatory aspects. Also, it focuses on frequency ranges below 6 GHz as it was agreed to not discuss any FR2 band before March, 2022.

# Text proposal

<Start of changes>

# 2. References

[1]	ITU-R Radio Regulations, Article 5 – Frequency Allocations 2020 Edition

<End of changes>

<Start of changes>

# 5. Regulatory aspects

## 5.1	ITU-R

The following services are among those defined in the Radio Regulations ([1]):

- Fixed (1.20): A radiocommunication service between specified fixed points.

- Fixed satellite (1.21) : A radiocommunication service between earth stations at given positions, when one or more satellites are used; the given position may be a specified fixed point or any fixed point within specified areas; in some cases this service includes satellite-to-satellite links, which may also be operated in the inter-satellite service; the fixed-satellite service may also include feeder links for other space radiocommunication services.

- Mobile (1.24): radiocommunication service between mobile and land stations, or between mobile stations (CV).

- Mobile satellite (1.25): A radiocommunication service:

between mobile earth stations and one or more space stations, or between space stations used by this service; or

between mobile earth stations by means of one or more space stations.

This service may also include feeder links necessary for its operation.

Based on ITU-R Radio Regulations ([1]), the following frequency ranges are allocated to MSS and have been identified as first candidate bands for NTN satellite operations:

S-band: UL: 1980–2010 MHz / DL: 2170–2200 MHz.

L-band: UL: 1626.5-1660.5 MHz / DL: 1525-1559 MHz.

## 5.2	NTN Satellite band: UL: 1980–2010 MHz / DL: 2170–2200 MHz

<To be updated>

## 5.3	NTN Satellite band: UL 1626.5–1660.5 MHz / DL 1525–1559 MHz

<To be updated>

<End of changes>