3GPP TSG-RAN WG4 Meeting #102-e	R4- 2203756

Electronic Meeting, Feb. 21 – Mar. 03, 2022

Agenda Item:	10.9.3.5

Source:	Apple

Title:	Discussion on the measurement requirement in FR2 HST RRM

Document for:	Discussion

# 1	Introduction

In Rel-17 NR support for high-speed train scenario in FR2 [1], potential RRM enhancement is proposed:

  - Study and identify RRM requirements impact and possible enhancement for

    - Idle/inactive mode cell reselection requirements enhancement

    - Connected mode requirements

      - Handover delay requirement

      - Measurement requirements including both L1 and SSB based L3 measurement

      - Beam management requirements including beam failure detection, candidate beam detection performance requirements

      - Other requirements if identified

[2,3] captures RRM related discussion in RAN4 101-e.

In this paper, we discuss the measurement requirement in HST FR2.

# 2	Discussion

In [4], the main open issue on measurement procedure is captured as option 1 and option 2.

Way forward:

- Discuss the following options to define the scaling factors:

  - Option 1: Scaling factors (Mpss/sss_synch_w/o_gaps and Mmeas_period_w/o_gaps) equal 6 for Set 1 and 18 for Set 2

  - Option 2: Scaling factors (Mpss/sss_synch_w/o_gaps and Mmeas_period_w/o_gaps) equal 10 for Set 1 and 24 for Set 2

Current intra-frequency PSS/SSS detection define Mpss/sss_sync_w/o_gaps is 24 in FR2 for UE power class 2,3,4. For measurement period requirement Mmeas_period_w/o_gaps is also 24 in FR2 for UE power class 2,3,4. Those values are significantly larger than FR1 case due to UE Rx beam sweeping. In current FR2, N=8, and 24 samples can be written as 3xN. Following number of Rx beam discussion, the intra-frequency PSS/SSS detection and measurement period can be enhancement for scenario A and scenario B separately.

Proposal 1:

- Mpss/sss_sync_w/o_gaps = 6 for set 1 and 18 for set 2.

- Mmeas_preriod_w/o_gaps = 6 for set 1 and 18 for set 2.

On the applicability of requirements to non-PC6 UE,

Way forward:

FFS, whether enhanced requirement should be applied to other than PC6 UEs even when HST FR2 flags are configured.

In our understanding FR2 HST enhancement is targeted for roof mounted UE in dedicated deployment. Therefore the enhanced requirement should not apply to other than PC6 UEs even when HST FR2 flags are configured.

Proposal 2: Enhanced requirement should be NOT applied to other than PC6 UEs even when HST FR2 flags are configured.

# 3	Summary

In this paper, we provide our view on mobility aspect of RRM enhancement for HST FR2.

Proposal 1:

- Mpss/sss_sync_w/o_gaps = 6 for set 1 and 18 for set 2.

- Mmeas_preriod_w/o_gaps = 6 for set 1 and 18 for set 2.

Proposal 2: Enhanced requirement should be NOT applied to other than PC6 UEs even when HST FR2 flags are configured.

# References

- RP-202118, New WID on NR support for high-speed train scenario in FR2, Samsung, Nokia, Sep 2020

- R4-2120292, Way forward for FR2 HST RRM (part 1), Nokia, Nov 2021

- R4-2120416, Way forward for FR2 HST RRM part 2, Samsung, Nov 2021

- R4-2202594, WF on FR2 HST RRM (part 1), Jan 2022

- R4-2202767, Way forward on Uplink timing for FR2 HST, Samsung, Jan 2022