3GPP TSG-RAN WG4 Meeting # 102-e	R4-2203766

# Electronic Meeting, Feb 21- Mar 3, 2022

Agenda item:	10.12.2.1.2

Source:	Apple Inc.

Title:	Discussion on CSI reporting requirements in intercell interference scenarios

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#101-bis-e CSI requirements for inter-cell interference MMSE-IRC was discussed and way forward [1] was agreed. In this contribution we present our simulation results and propose test metrics for CQI reporting in ICI.

# 2. Discussion

In RAN4#101-bis-e for CQI reporting requirements in ICI the following open issue related to requirements definition was remaining:

Issue 2-2-1: Test setup methodology for signal power

- Option 1: Define test based on SINR

- Option 2: Define test based on SNR

The test setup is based on comparing the Throughput with and without interference at a certain SINR. If we use SNR as the test point, TP with no interference or AWGN would be higher than the TP with interference and TP ratio would be < 1. This wouldn’t be a suitable metric as we a comparing SNR with INR=10 dB and SNR with no interference. Using SINR would be more appropriate as all the interference and noise would be accounted for in a single value. Hence, we should define the test based on SINR.

Proposal #1: Define the test based on SINR.

Based on the simulation assumptions agreed in [1] we present simulation results below for 2RX and 4RX.

2RX

Table 1: TP Gain with MMSE and MMSE-IRC processing for CQI -2x2

| SINR | MMSE IRC |  | CSI MMSE |
| --- | --- | --- | --- |
|  | TP Gain | BLER | TP Gain |
| -6 | 3.5 | 0.30 | 1.5 |
| -5 | 3.3 | 0.30 | 1.3 |
| -4 | 3.3 | 0.29 | 1.2 |
| -3 | 3.1 | 0.28 | 1.1 |
| -2 | 2.8 | 0.30 | 1.1 |
| -1 | 2.7 | 0.32 | 1.1 |
| 0 | 2.5 | 0.32 | 1.1 |
| 1 | 2.4 | 0.33 | 1.2 |
| 2 | 2.3 | 0.33 | 1.2 |
| 3 | 2.1 | 0.32 | 1.2 |
| 4 | 1.9 | 0.34 | 1.2 |
| 5 | 1.9 | 0.32 | 1.2 |
| 6 | 1.8 | 0.34 | 1.3 |
| 7 | 1.8 | 0.31 | 1.3 |
| 8 | 1.9 | 0.27 | 1.3 |
| 9 | 1.7 | 0.27 | 1.3 |
| 10 | 1.8 | 0.22 | 1.3 |

4RX

Table 2: TP Gain with MMSE and MMSE-IRC processing for CQI -2x4

| SINR | MMSE-IRC |  | CSI MMSE |
| --- | --- | --- | --- |
|  | TP Gain | BLER | TP Gain |
| -6 | 4.4 | 0.32 | 1.5 |
| -5 | 4.1 | 0.34 | 1.3 |
| -4 | 3.6 | 0.31 | 1.2 |
| -3 | 3.0 | 0.32 | 1.1 |
| -2 | 2.8 | 0.31 | 1.1 |
| -1 | 2.7 | 0.32 | 1.1 |
| 0 | 2.7 | 0.32 | 1.1 |
| 1 | 2.5 | 0.31 | 1.2 |
| 2 | 2.4 | 0.30 | 1.2 |
| 3 | 2.2 | 0.30 | 1.2 |
| 4 | 2.2 | 0.26 | 1.2 |
| 5 | 1.9 | 0.31 | 1.2 |
| 6 | 2.0 | 0.28 | 1.3 |
| 7 | 1.8 | 0.31 | 1.3 |
| 8 | 1.9 | 0.27 | 1.3 |
| 9 | 1.7 | 0.27 | 1.3 |
| 10 | 1.8 | 0.22 | 1.3 |

Based on the results we recommend choosing 0/1/2 dB for 2RX and -3/-2/-1 for 4RX. The gains at that SINR are high and the SINR value is not very low.

Proposal #2: Use SINR of 0/1/2 dB for 2RX and -3/-2/-1 for 4RX.

The TP gain metric can be set to 2 for 2RX and 2.5 for 4RX.

Proposal #3: Set TP gain (γ) metric as 2 for 2RX and 2.5 for 4RX.

For BLER (X) in the requirements, we propose to use 2% which is widely used in CQI reporting requirements and also is suitable based on simulation results.

Proposal #4: Define requirements with BLER metric (X) with follow CQI in ICI as 2%

# 3. Conclusion

In this paper, we present our simulation results and propose test metrics for CQI reporting in ICI. Our observations and proposals are captured below:

Proposal #1: Define the test based on SINR.

Proposal #2: Use SINR of 0/1/2 dB for 2RX and -3/-2/-1 for 4RX.

Proposal #3: Set TP gain (γ) metric as 2 for 2RX and 2.5 for 4RX.

Proposal #4: Define requirements with BLER metric (X) with follow CQI in ICI as 2%

# Reference

- R4-2203009, “WF on CSI requirements for inter-cell interference MMSE-IRC”, Ericsson.