3GPP TSG-RAN WG4 Meeting # 102-e																				R4-2203816

Electronic Meeting, February 21 – March 3, 2022

Source:		Verizon, Samsung

Title:				TP for TR TR 38.717-04-02: CA_n2-n5-n48-n66

Agenda item:		9.12.2

Document for:		Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-04-02 to include CA_n2-n5-n48-n66 according to the request in [1].

# 2. Reference

- R4-2112431, Revised WID on Rel-17 NR inter-band Carrier Aggregation/Dual connectivity for DL 4 bands and 2UL bands.

# 3. Text Proposal

<Start of Text Proposal>

## 5.1.x		CA_n2-n5-n48-n66

### 5.1.x.1	Operating bands for CA

Table 5.1.x.1-1: Inter-band CA operating bands

| NR CA Band | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- |
| CA_n2-n5-n48-n66 | n2, n5, n48, n66 |

### 5.1.x.2	Channel bandwidths per operating bands for CA

Table 5.1.x.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n2A-n5A-n48A-n66A | CA_n2A-n5A CA_n2A-n48A CA_n2A-n66A CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n2 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n5 | 5 | 10 | 15 | 20 | 25 |  |  |  |  |  |  |  |  |  |
|  |  | n48 | 5 | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n2A-n5A-n48(2A)-n66A | CA_n2A-n5A CA_n2A-n48A CA_n2A-n66A CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n2 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n5 | 5 | 10 | 15 | 20 | 25 |  |  |  |  |  |  |  |  |  |
|  |  | n48 | See CA_n48(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n2 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 2 |
|  |  | n5 | 5 | 10 | 15 | 20 | 25 |  |  |  |  |  |  |  |  |  |
|  |  | n48 | See CA_n48(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n2A-n5A-n48B-n66A | CA_n2A-n5A CA_n2A-n48A CA_n2A-n66A CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n2 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n5 | 5 | 10 | 15 | 20 | 25 |  |  |  |  |  |  |  |  |  |
|  |  | n48 | See CA_n48B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n2 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 2 |
|  |  | n5 | 5 | 10 | 15 | 20 | 25 |  |  |  |  |  |  |  |  |  |
|  |  | n48 | See CA_n48B Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n2 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 3 |
|  |  | n5 | 5 | 10 | 15 | 20 | 25 |  |  |  |  |  |  |  |  |  |
|  |  | n48 | See CA_n48B Bandwidth Combination Set 2 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |

#### 5.1.x.3	UE co-existence studies

The coexistence studies have been captured in the fallback modes in TR 38.717-03-02, there is no additional harmonic and intermodulation impact for the additional band receiver.

#### 5.1.x.4	∆TIB,c and ∆RIB,c values

The TIB,c and RIB,c can reuse the same values for CA_n2-n5-n48-n66 with 1UL that have been captured in TR 38.716-04-01

#### 5.1.x.5	REFSENS requirements

There is no need to specify additional MSD requirement for this UL CA configuration.

<End of Text Proposal>