3GPP TSG-RAN WG4 Meeting #102e	R4-2203820

Electronic Meeting, Feb 21 - Mar 03, 2022

Source:	China Telecom

Title:	On phase continuity tolerance for DMRS bundling

Agenda Item:	10.18.2.2

Document for:	Discussion

- Introduction

In RAN4 #101e-bis meeting, the WF on phase continuity and power consistency was approved in [1]. This contribution provides our views on the remaining issues.

- Discussion

2.1  Phase continuity tolerance

In RAN4 #101e-bis, the following agreements were reached on the phase continuity tolerance:

Issue 1-1: Model of phase variation

Candidate options:

•	Phase offset Option 1: for each individual slot k (k=1…n) within the bundle, an independent offset is generated and applied with respect to the slot 0.

•	Phase offset Option 2: for each individual slot k (k=1…n) within the bundle, an independent offset is generated and applied with respect to the slot k-1. (i.e., the offset is allowed to accumulate)

Issue 1-2: Phase continuity tolerance

WF recommendation:

- Down select between the following two options:

  - Option 1: Adopt [-30, 30] degrees if Phase offset Option 1 in Issue 1-1 is agreed.

  - Option 2: Adopt [-15, 15] degrees if Phase offset Option 2 in Issue 1-1 is agreed.

In our contribution submitted in the last meeting [2], we have run simulations to compare the two options for phase offset model, and made the following observations based on our simulation results:

Observation 1: For phase offset option 1 with [-30, 30] degrees and phase offset option 2 with [-15, 15] degrees, the JCE performance is the same or very similar, in scenario of FR1 15 & 30 kHz SCS with 16 and 32 repetitions, and FR2 60Hz SCS with 16 repetitions.

Observation 2: JCE performance based on phase offset option 1 with [-30, 30] degrees is better than that of phase offset option 2 with [-15, 15] degrees, in scenario of FR2 60Hz SCS with 32 repetitions.

So, in general, if it is the truth that phase offset option 2 could better model the real UE behavior, we are fine with either phase offset option 1 or option 2.

Proposal 1: Select either option 1 or option 2 in this meeting:

  - Option 1: [-30, 30] degrees with Phase offset Option 1

  - Option 2: [-15, 15] degrees with Phase offset Option 2

2.2  Reference point for phase tolerance test

The following options were captured in the last meeting WF:

Issue 3-2: Reference point for phase/amplitude tolerance test

Candidate options:

- Option 1: Define the reference point for phase/amplitude tolerance requirement in Figure 1 for both options of defining RF requirements in Issue 3-1.

- Option 2: The reference point would be between “Channel estimation” and “equalization”, since the amplitude and phase values can be taken directly from the channel estimation.

- Option 3:

- For testing phase and power offset: Reference point to calculate the phase continuity and power consistency is after IDFT (time domain).

- For testing EVM: Before IDFT for CP-OFDM case. After IDFT for DFT-s-OFDM case.

In the last meeting, TE experts suggested to measure the phase value of frequency-domain signal. In this way, option 2 will be selected. In addition, we’d like to clarity two things:

- One thing is that when the phase value is taken directly from the channel estimation, this method can be used for both DFT-s-OFDM and CP-OFDM waveforms, given that no difference for the DMRS of DFT-s-OFDM and CP-OFDM waveforms (i.e., DFT/IDFT is not applied to DMRS).

- The other thing is whether only the channel estimation on DMRS symbols in each slot will be used to derive the phase value.

Proposal 2: Select option 2 as the reference point for phase tolerance test:

- Option 2: The reference point would be between “Channel estimation” and “equalization”, since the amplitude and phase values can be taken directly from the channel estimation.

Proposal 3: For option 2 of the reference point, clarify that it applies to both DFT-s-OFDM and CP-OFDM waveforms, considering that DFT/IDFT is not applied to DMRS.

Proposal 4: For option 2 of the reference point, discuss whether only the channel estimation on DMRS symbols in each slot will be used to derive the phase value.

- Conclusion

This contribution presented our views on phase continuity tolerance requirements, with the following observations and proposals:

For the phase continuity tolerance:

Observation 1: For phase offset option 1 with [-30, 30] degrees and phase offset option 2 with [-15, 15] degrees, the JCE performance is the same or very similar, in scenario of FR1 15 & 30 kHz SCS with 16 and 32 repetitions, and FR2 60Hz SCS with 16 repetitions.

Observation 2: JCE performance based on phase offset option 1 with [-30, 30] degrees is better than that of phase offset option 2 with [-15, 15] degrees, in scenario of FR2 60Hz SCS with 32 repetitions.

So, in general, if it is the truth that phase offset option 2 would better model the real UE behavior, we are fine with either phase offset option 1 or 2.

Proposal 1: Select either option 1 or option 2 in this meeting:

  - Option 1: [-30, 30] degrees with Phase offset Option 1

  - Option 2: [-15, 15] degrees with Phase offset Option 2

For reference point for phase tolerance test:

Proposal 2: Select option 2 as the reference point for phase tolerance test:

- Option 2: The reference point would be between “Channel estimation” and “equalization”, since the amplitude and phase values can be taken directly from the channel estimation.

Proposal 3: For option 2 of the reference point, clarify that it applies to both DFT-s-OFDM and CP-OFDM waveforms, considering that DFT/IDFT is not applied to DMRS.

Proposal 4: For option 2 of the reference point, discuss whether only the channel estimation on DMRS symbols in each slot will be used to derive the phase value.

- References

R4-2202418, WF on phase continuity and power consistency for PUCCH and PUSCH transmissions, Huawei, HiSilicon, RAN4 #101e-bis, Jan 2022.

R4-2200022, On phase continuity and power consistency tolerance, China Telecom, RAN4 #101e-bis, Jan 2022.