3GPP TSG-RAN WG4 Meeting #102e	R4-2203823

# Electronic Meeting, February 21 - March 3, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0437 | rev |  | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 38.101-2 CR: UE RF requirements for DMRS bundling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_cov_enh-Core |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To support DMRS bundling and joint channel estimation for PUSCH and PUCCH in Rel-17 coverage enhancements WI, phase continuity and power consistency need to be maintained with certain tolerances. For power consistency, the existing requirement for aggregate power tolerance applies and no dedicated new requirements are to be introduced. For phase continuity, new requirements should be introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the phase continuity requirements for DMRS bundling for PUSCH and PUCCH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DMRS bundling cannot be supported in Rel-17. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New sub-clauses: 6.4.2.6, 6.4D.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of first change >

6.4.2.6	Phase continuity requirements for DMRS bundling

For bands that UE declares the support for DMRS bundling, the maximum allowable difference between the measured phase value in any slot k and in [the first slot or its previous slot k - 1] for one DMRS port is listed in Table 6.4.2.6-1 for the parameters defined in Table 6.4.2.6-2, within a measurement time window determined by the UE’s capability of maximum duration for DMRS bundling [maxDMRS-BundlingDuration]. The phase value for each slot is measured [after the channel estimation and before the equalization], as shown in Annex F.x.

Table 6.4.2.6-1: Maximum allowable phase difference for DMRS bundling

| UL channel | Modulation order | Phase difference between any slot k and [the first slot or its previous slot k - 1] | Measurement time window |
| --- | --- | --- | --- |
| PUSCH | Pi/2 BPSK, QPSK | [30 or 15] degrees | Maximum duration reported by UE |
| PUCCH | Pi/2 BPSK, BPSK, QPSK |  |  |
| NOTE: For one freuqncy band, the same length of maximum duration applies for PUSCH and PUCCH, and for DFT-s-OFDM and CP-OFDM. |  |  |  |

Table 6.4.2.6-2: Parameters for the maximum allowable phase difference

| Parameter | Unit | Level |
| --- | --- | --- |
| UE EIRP | dBm | PUMAX,f,c in clause 6.2.4, P-MPR = 0 |
| UE downlink received power |  | Not change |
| Operating conditions |  | Normal conditions |

The above requirements are applicable when all the following conditions are met within the specified time window:

-	RB allocation in terms of length and frequency position does not change, and intra-slot and inter-slot frequency hopping does not apply.

-	Modulation order does not change.

-	No network commanded TA takes effectt.

-	The TPMI precoder does not change.

-	UE is not scheduled with uplink transmission of other physical channel/signal in-between the PUSCH or PUCCH transmissions.

-	In the case of non-back-to-back transmission, the un-scheduled gap in-between PUSCH or PUCCH transmission is no more than 13 symbols and [11] sybmols for different SCSs with normal CP and extended CP respectively.

-	No downlink slot or downlink reception or downlink monitoring in-between the PUSCH or PUCCH transmissions.

-	No uplink beam switching occurs.

< End of first change >

< Start of second change >

6.4D.2	Transmit modulation quality for UL MIMO

For UE supporting UL MIMO, the transmit modulation quality requirements are specified per layer in terms of:

Error Vector Magnitude (EVM) for the allocated resource blocks (RBs)

EVM equalizer spectrum flatness derived from the equalizer coefficients generated by the EVM measurement process

Carrier leakage (caused by IQ offset)

For UE supporting UL MIMO, the transmit modulation quality requirements are specified as the total component of EIRP in terms of:

In-band emissions for the non-allocated RB

For bands that UE declares the support for UL MIMO and DMRS bundling, the transmit modulation quality requirements apply at [each transmit antenna connector] in terms of:

Phase continuity requirements for DMRS bundling

The requirements are defined as directional requirements. The requirements are verified in beam locked mode in the TX beam peak direction (Link=TX beam peak direction, Meas=Link angle).

In case the parameter 3300 or 3301 is reported from UE via the parameter txDirectCurrentLocation in UplinkTxDirectCurrentList IE (as defined in TS 38.331 [13]), carrier leakage measurement requirement in clause 6.4D.2.2 and 6.4D.2.3 shall be waived, and the RF correction with regard to the carrier leakage and IQ image shall be omitted during the calculation of transmit modulation quality.

< End of second change >