3GPP TSG-RAN WG4 Meeting #102-e	R4-2204017

21st February – 3rd March 2022

Source:	Qualcomm Incorporated

Title:	Frequency error measurement period for NR SL MIMO and NR V2X TxD

Agenda item:	10.15.2.3

Document for:	Approval

# 1.	Introduction

Currently the frequency error observation period for V2X NR SL MIMO and V2X NR TxD is specified to be 0.5 ms while other features in NR use 1 ms [1,2].  In this paper we would like to present our views on this topic.

# 2. 	Discussion

In TS38.101-1, section 6.4E.1.1 [1] states the following for V2X UE supporting NR SL MIMO:

Also, the frequency error observation period for NR V2X TxD captured in [2] states the following:

As can be seen the frequency error observation period for NR SL MIMO and NR V2X TxD is 0.5 ms.

Observation 1: The frequency error observation period for NR SL MIMO and NR V2X TxD is 0.5 ms

However, for NR Uu UL MIMO TS38.101-1, section 6.4H.1 [1] states the following for the frequency error observation period:

and the frequency error observation period for NR Uu TxD given in TS38.101-1, section 6.4G.1[1] states the following:

As can be seen the frequency error observation period for Uu UL MIMO and Uu TxD is 1.0  ms.

Observation 2: The frequency error observation period for NR Uu UL MIMO and NR Uu TxD is 1.0ms

As the specifications for V2X should not be more stringent than for Uu it is suggested that the frequency error observation period for NR SL MIMO and NR V2X TxD be revised to 1.0 ms.

Proposal 1: The frequency error observation period for NR SL MIMO and NR V2X TxD should be revised to 1.0 ms to align with the Uu specifications

# Conclusion

In this paper the frequency error observation period SL MIMO and V2X TxD is discussed, and the following observations and proposals are made:

Observation 1: The frequency error observation period for NR SL MIMO and NR V2X TxD is 0.5 ms

Observation 2: The frequency error observation period for NR Uu UL MIMO and NR Uu TxD is 1.0 ms

Proposal 1: The frequency error observation period for NR SL MIMO and NR V2X TxD should be revised to 1.0 ms to align with the Uu specifications

# References

[1] TS38.101-1, User Equipment (UE) radio transmission and Reception; Part 1: Range 1 Standalone (Release 17)

[2] R4-2201952, TP for Tr 37.785: TxD requirements for NR V2X, RAN4#101-bis-e, January 2022