3GPP TSG-RAN WG4 Meeting # 102-e	R4-2204039

# Electronic Meeting, February 21 – March 3, 2022

Source:	Sony

Title:	Minimum Rx requirement for handheld UEs at 60 GHz

Agenda item:	10.16.3.2

Document for:	Approval

# Background

In RAN4#101-bis-e, the UE RF performance in FR2-2 was further discussed [1], and RAN4 needs to explore the minimum requirement of various types of UEs. It was agreed that eight-element array is used as the baseline to derive the minimum requirement for handheld UE. This paper shares our analysis on the minimum Rx requirement for handheld UEs.

# REFSENS for mobile handsets at 60 GHz

The peak sensitivity (REFSENS) of a FR2-2 antenna array can be calculated by the equation below:

Sensitivity = -174dBm(kT) + 10*log(RX BW) + NF + SNR +IL – diversity gain-array gain+ roll off factor

The link budget of an 8 element array in band n263 (57-71 GHz) with 400 MHz BW is shown in the table below, the estimated REFSENS is -76 dBm /400MHz.

| Parameter | Unit | Value |
| --- | --- | --- |
| Modulation |  | QPSK |
| SNR requirement | dB | -1 |
| Bandwidth | MHz | 400 |
| Thermal noise | dBm/Hz | -174 |
| Noise Figure (NF) | dB | 13 |
| Number of elements in an array |  | 8 |
| Realized Array gain | dB | 11 |
| Antenna roll off factor | dB | 1 |
| Diversity gain | dB | 0 |
| Total implementation losses (IL) | dB | 10 |
| REFSENS | dBm | -76 |

Proposal 1: REFSENS -76 dBm/400MHz for n263

# Spherical coverage of handheld UEs at 60 GHz

Simulation results in [2] shows 8.5 dB degradation on from the peak gain to the 50% array gain can be achieved with two antenna panels with 8*1 arrays in a mobile handset. Therefore, the 50% spherical coverage EIS for band n263 is about 67.5 dBm/400MHz.

Proposal 2: 8.5 dB drop from REFSEN to the spherical coverage EIS, which leads to 67.5 dBm/400MHz for n263

# Conclusions

In this contribution, we make the following observations and conclusions:

Proposal 1: REFSENS -76 dBm/400MHz for n263

Proposal 2: 8.5 dB drop from REFSEN to the spherical coverage EIS at 50% CDF, which leads to 67.5 dBm/400MHz for n263.

# References

- R4-2202366	WF on 60 GHz UE RF	Qualcomm

- R4-2204038	Minimum Tx requirement for handheld and FWA UEs at 60 GHz   Sony