3GPP TSG-RAN WG4 Meeting #102e                	    	    	    	    	    	    	    	    	    	    	    	    	    	    	    	    	           R4-2204054

Electronic Meeting, Feb 21 – March 3, 2022

Agenda Item:	8

Source:	MediaTek Inc.

Title:	Inputs to Rel-17 NR UE features for measurement gap enhancement and UE power saving enhancement

Document for:	Discussion

# Introduction

In this paper, we provide our views on the UE feature list for Rel-17 MG enhancement and UE power saving enhancement.

# Measurement gap enhancement

Pre-configured gaps

In last RAN4 meeting, there was agreements in the WF [1] to introduce the UE features, as captured below:

| Issue 6-1 | UE feature list for pre-MG Feature group; Components; Consequence if the feature is not supported by the UE; Note; Mandatory/Optional. X-1-1; Pre-configured measurement gap with network-controlled activation and deactivation mechanism; UE does not support pre-configured measurement gap with Network-controlled mechanism; Optional with capability signalling. X-1-2; Pre-configured measurement gap with UE autonomous activation and deactivation mechanism; UE does not support pre-configured measurement gap with UE autonomous mechanism; Optional with capability signalling. |
| --- | --- |

Proposal 1: Following the agreements of RRM session, introduce separate UE capabilities of network-controlled and UE autonomous mechanism for preconfigured measurement gap activation and deactivation.

Concurrent gaps

A baseline UE capability for the concurrent gap feature has been introduced in [2]. There are still some ongoing discussions with potential impact on UE capabilities. We should wait for the RRM session to conclude those issues in [3].

- Issue 2-1-1: Whether concurrent gaps are allowed in the case when only E-UTRAN measurement objectives are configured - Open issue - Option 1: Yes - Option 1a: Yes, provided that UE supports LTE measurement with concurrent MGs, which is up to UE capability - Option 1b: Yes, under the condition that only one per-UE MG is configured for UE - Option 2: No - Issue 2-3-1: X value in proximity condition for overlapping in FR1. - Agreement - Consider as least X=4 in proximity condition for overlapping in FR1 - FFS to introduce X=0 as an optional UE capability - Issue 2-3-2: X value in proximity condition for overlapping in FR2 - Open issue - FFS to consider as least X=4 in proximity condition for overlapping in FR2 - FFS to introduce X=0 as an optional UE capability - Issue 2-4-1: Whether to define the overhead cap - Open issue - Option 1: Yes - Option 2: No - Option 3: Up to UE capability

Proposal 2: For concurrent gaps, wait for the RRM session to conclude the following issues with potential UE capability impact: E-UTRAN only measurement, proximity condition for overlapping and overhead cap.

Network control small gap (NCSG)

A baseline UE capability for NCSG feature has been introduced in [2]. Furthermore, there are still open issues in [4] about whether to introduce UE capability for the support of NCSG patterns and for NR-only measurements. We can wait for the conclusion of the RRM session

Issue 2-2: The existing gap applicability in Rel-16 for NR-only measurement are reused for NCSG UE can indicate support of some NCSG patterns which can only be used for NR-only measurement. FFS on how to indicate support of NR-only NCSG pattern: - Option 1: reuse supportedGapPattern-Nronly (require mapping between legacy gap patterns and NCSG patterns) - Option 2: introduce a new signaling, e.g. supportedNCSGPattern-Nronly - Option 3: up to RAN2 Issue 3-2: Whether additional UE capability is needed for per-UE and per-FR differentiation for NCSG on top of that defined for legacy gap - Option 1: No - Option 2: Define a per BC indication for per FR NCSG. - Option 3: do not rely on R15 capability independentGapConfig. Define a new NCSG per-UE and per-FR capability, e.g. independentNCSGConfig Issue 5-4: UE feature list discussion on NCSG support - X-1 is agreed. FFS on X-2 and X-3 Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Applicable to the capability signalling exchange between UEs (V2X WI only)”.; Consequence if the feature is not supported by the UE; Type; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Note; Mandatory/Optional. X-1; Network controlled small gap (NCSG); Support of NCSG; yes; no; UE cannot be configured with NCSG; per-UE; No; No; Optional with capability signalling. X-2; Network controlled small gap (NCSG); Supported NCSG patterns; yes; no; Network does not know whether some NCSG patterns can be configured to UE; per-UE; No; No; Optional with capability signalling NCSG patterns #0, #1, [x, y, …] are conditional mandatory if UE support X-1. X-3; Network controlled small gap (NCSG); Support of NCSG per band in target MO in a band combination for inter-frequency measurement; Per-band.

Proposal 3: For NCSG, wait for the RRM session to conclude the following issues with potential UE capability impact: NCSG pattern reporting, NCSG pattern reporting for NR-only measurement and per BC indication for per FR NCSG

Below, we provide a candidate feature list for Rel-17 measurement gap enhancement.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD diff. | Need of FR1/FR2 diff. | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/ Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 19. NR_MG_enh | 19-3-1 | Network-controlled mechanism for preconfigured measurement gap activation and deactivation | Pre-configured measurement gap with network-controlled activation and deactivation mechanism |  | yes | no | UE does not support pre-configured measurement gap with Network-controlled mechanism | per UE | No | No |  |  | Optional with capability signalling |
|  | 19-3-2 | UE autonomous mechanism for preconfigured measurement gap activation and deactivation | Pre-configured measurement gap with UE autonomous activation and deactivation mechanism |  | yes | no | UE does not support pre-configured measurement gap with UE autonomous mechanism | per UE | No | No |  |  | Optional with capability signalling |

# UE power saving enhancement

In last meeting, RAN4 agreed to introduce a UE capability for supporting RLM/BFD relaxation [5].

Issue 1-1-1: Feature for RLM/BFD relaxation in Rel-17 feature table - RAN4 to introduce a UE capability to indicate the support of RLM/BFD relaxation in general in Rel-17 feature table.

However, the details are still missing. Therefore, here we suggest the corresponding details in below table

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD diff. | Need of FR1/FR2 diff. | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/ Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Y. NR_UE_pow_sav_enh | Y-1 | RLM/BFD relaxation | Capability of supporting RLM/BFD relaxation in CONNECTED mode |  | yes | no | UE does not support RLM/BFD relaxation | per UE | No | No |  |  | Optional with capability signalling |

Proposal 4: Introduce UE capabilities for RLM/BFD measurement relaxation.

# Conclusion

In the contribution, we provide our input on the UE feature list about Rel-17 measurement gap enhancement. We have the following proposals.

Proposal 1: Following the agreements of RRM session, introduce separate UE capabilities of network-controlled and UE autonomous mechanism for preconfigured measurement gap activation and deactivation.

Proposal 2: For concurrent gaps, wait for the RRM session to conclude the following issues with potential UE capability impact: E-UTRAN only measurement, proximity condition for overlapping and overhead cap.

Proposal 3: For NCSG, wait for the RRM session to conclude the following issues with potential UE capability impact: NCSG pattern reporting, NCSG pattern reporting for NR-only measurement and per BC indication for per FR NCSG

Proposal 4: Introduce UE capabilities for RLM/BFD measurement relaxation. .

We also provided a candidate feature tables for information.

# Reference

[1] R4-2202614, “WF on R17 NR MG enhancements – Pre-configured MG”, Intel Corporation

[2] R4-2202400, “Rel-17 UE feature list”, CMCC

[3] R4-2202603, “WF on R17 NR MG enhancements – General issues and multiple concurrent MGs”, MediaTek Inc.

[4] R4-2202625, “WF on NCSG”, Apple

[5] R4-2002640, “WF on RLM/BFD relaxation for UE Power Saving enhancements”, MediaTek Inc.