3GPP TSG RAN WG4 Meeting #102-e	R4-2204112

Online, 21th Feb. – 3rd Mar. 2022

Agenda item:	9.16.2

Source:	SoftBank Corp.

Title:	TP for TR 37.717-31-11: EN-DC_1-8-11_n79

Document for:	Approval

1	Introduction

EN-DC of 3B LTE and 1B NR of DC_1-8-11_n79 was approved in RAN#93[1]. This TP is to capture the basic aspects for the EN-DC.

2	Combination proposed

Combinations proposed in [1] are as below.

Table 1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-8A-11A_n79A2 | DC_1A_n79A DC_8A_n79A DC_11A_n79A |
| NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |

3.  Delta Tib/Rib

The delta Tib/Rib of subset EN-DC/CA are as follows.

Table 2: ΔTIB of constituent 4 band combo.

| ΔTIB | B1+B8+B11 | B1+B8+n79 | B1+B11+n79 | B8+B11+n79 | Proposed Value |
| --- | --- | --- | --- | --- | --- |
| B1 | 0.3 | 0.3 | 0.3 |  | 0.3 |
| B8 | 0.3 | 0.3 |  | 0.3 | 0.3 |
| B11 | 0.4 |  | 0.3 | 0.4 | 0.4 |
| n79 |  | 0 | 0 | 0 | 0 |

Table 3: ΔRIB of constituent 4 band combo.

| ΔRIB | B1+B8+B11 | B1+B8+n79 | B1+B11+n79 | B8+B11+n79 | Proposed Value |
| --- | --- | --- | --- | --- | --- |
| B1 | 0 | 0 | 0 |  | 0 |
| B8 | 0 | 0 |  | 0 | 0 |
| B11 | 0 |  | 0 | 0 | 0 |
| n79 |  | 0 | 0 | 0 | 0 |

Then the proposal for this 3LTE-1NR EN-DC is:

Table 4: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-11_n79 | 1 | 0.3 |
|  | 8 | 0.3 |
|  | 11 | 0.4 |
|  | n79 | 0 |

Table 5: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-8-11_n79 | 1 | 0 |
|  | 8 | 0 |
|  | 11 | 0 |
|  | n79 | 0 |

5.  REFSENS

MSD of this 4DL/2UL DC configuration are already covered by those of the 2DL/2UL and 3DL/2UL. Therefore, no additional MSD requirement is needed.

6.  Conclusion

This paper proposes study results of EN-DC 1-8-11_n79.

TP is shown in Annex.

7.  Reference

[1]  RP-211758  Revised WID: Basket WI Dual Connectivity (EN-DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL), Ericsson

Annex.   Proposed text for TR37.717-31-11: LTE 3B -NR 1B DC

[Unchanged Parts Skipped]

5.1	Inter-band EN-DC within FR1

## 5.1.xx	DC_1-8-11_n79

### 5.1.xx.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-8A-11A_n79A2 | DC_1A_n79A DC_8A_n79A DC_11A_n79A |
| NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |

### 5.1.xx.2	∆TIB and ∆RIB values

For DC_1-8-11_n79, the TIB,c and RIB,c values are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-11_n79 | 1 | 0.3 |
|  | 8 | 0.3 |
|  | 11 | 0.4 |
|  | n79 | 0 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-8-11_n79 | 1 | 0 |
|  | 8 | 0 |
|  | 11 | 0 |
|  | n79 | 0 |

### 5.1.xx.3	Reference sensitivity exceptions

Co-existence study for DC_1-8-11_n79 was covered by the studies for the fallback modes of DC_1-8_n79, DC_1-11_n79 and DC_8-11_n79.

No additional MSD requirement need to be defined for this dual connectivity configuration.

[Unchanged Parts Skipped]