3GPP TSG RAN WG4 Meeting #102-e	R4-2204118

Online, 21th Feb. – 3rd Mar. 2022

Agenda item:	9.23.2

Source:	SoftBank Corp.

Title:	TP for TR 37.717-11-41: EN-DC_8_n3-n28-n77-n79

Document for:	Approval

1	Introduction

EN-DC of 1B LTE and 4B NR of DC_8_n3-n28-n77-n79 was approved in RAN#94 [1]. This TP is to capture the basic aspects for the EN-DC.

2	Text Proposal

[Unchanged Parts Skipped]

6.X	DC_8_n3-n28-n77-n79

6.X.1	Operating bands for DC

Table 6.X.1-1: EN-DC band combination (five bands)

| EN-DC Band | E-UTRA Band | NR CA Band |
| --- | --- | --- |
| DC_8_n3-n28-n77-n79 | 8 | CA_n3-n28-n77-n79 |

6.X.2	Inter-band DC Configurations

Table 6.X.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_8A_n3A-n28A-n77A-n79A | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_8A_n79A |

6.X.3	Co-existence studies

Co-existence study for DC_8_n3-n28-n77-n79 was covered by the studies for the fallback modes of DC_8_n3-n28-n77, DC_8_n3-n28-n79, DC_8_n3-n77-n79 and DC_8_n28-n77-n79.

No additional MSD requirement needs to be defined for this dual connectivity configuration.

6.X.4	∆TIB and ∆RIB values

DC_8_n3-n28-n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 6.X.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n3-n28-n77-n79 | 8 | 0.6 |
|  | n3 | 0.6 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |
|  | n79 | 0.8 |

Table 6.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n3-n28-n77-n79 | 8 | 0.2 |
|  | n3 | 0.2 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |
|  | n79 | 0.5 |

6.X.5	MSD

As mentioned in 6.X.3, there is no need to specify additional MSD requirement for this UL DC configuration.

[Unchanged Parts Skipped]

3	Reference

[1]  RP-213142  Revised WID on Rel-17 Dual Connectivity (DC) of x bands (x=1,2) LTE inter-band CA (xDL1UL) and 4 bands NR inter-band CA (4DL1UL), Huawei, HiSilicon