3GPP TSG-RAN WG4 Meeting # 102-e	R4-2204150

# Electronic Meeting, February 21 – March 3, 2022

Source:	LG Electronics

Title:	Discussion on multiple concurrent and independent MG patterns

Agenda item:	10.11.2.2

Document for:	Discussion

Introduction

In this paper, we provide our view on multiple concurrent and independent MG patterns for MG enhancements.

Discussion

In the last RAN4 meeting, one WF[1] was made for multiple concurrent and independent MG patterns. Based on the WF, we would like to continue discussing the overlapping and overhead issues.

- Overlapping - Issue 2-3-1: X value in proximity condition for overlapping in FR1. - Agreement - Consider as least X=4 in proximity condition for overlapping in FR1 - FFS to introduce X=0 as an optional UE capability - Issue 2-3-2: X value in proximity condition for overlapping in FR2 - Open issue - FFS to consider as least X=4 in proximity condition for overlapping in FR2 - FFS to introduce X=0 as an optional UE capability - Issue 2-3-3: UE behavior during colliding gap occasion - Open issue - Option 1: Priority rule - UE will only do the measurement w.r.t. the gap with higher priority on all colliding occasions - The priority can be configurable or fixed - Option 5: Compromised proposal from moderator - Introduce gap sharing rule. - Request RAN2 to reserve some RRC signaling for different sharing factors. - The signalling design may consider the possibility of resuming data scheduling on dropped gaps - Rel-17 requirements will only consider sharing ratios 0% and 100%. - The requirements for other sharing factors are FFS in later releases. - Agreement: CRs can be drafted based on Option 1 with the editor’s note: “The detail UE behavior can be revised based on the later RAN4 agreement on UE behavior during colliding gap occasion.” - Issue 2-3-4: Whether to resume data scheduling on the dropped gap occasions - Agreement - Data scheduling is resumed on the dropped gap occasions - Issue 2-3-5: Whether to introduce a UE capability to indicate whether UE supports only 0% and 100% gap sharing ratios or UE supports arbitrary configured sharing ratios. (If Option 5 in Issue 2-3-3 is agreed) - Postpone to next meeting - Issue 2-3-6: Detail gap sharing ratios (If Option 5 in Issue 2-3-5 is agreed) - Postpone to next meeting - Issue 2-3-7: Whether to introduce FO, FPO, PFO, PPO scenarios. - Open issue - Option 1: Introduce all scenarios - Option 2: Only introduce PFO, PPO scenarios - Option 3: Only introduce FO, FPO scenarios - Issue 2-3-8: Whether to introduce UE capability for different overlapping scenarios (FO, FPO, PFO, PPO). - Agreement - Do not introduce UE capability for different overlapping scenarios (FO, FPO, PFO, PPO) - Overhead - Issue 2-4-1: Whether to define the overhead cap - Open issue - Option 1: Yes - Option 2: No - Option 3: Up to UE capability - Issue 2-4-2: how to define overhead cap - Open issue - Option 1: The max overhead that UE can support in Rel-15/16. - Option 2:m=1NMGLmMGRPmMGLrMGRPr≤1+ threshold(K) m=1NMGLmMGRPmMGLrMGRPr≤1+ threshold(K) m=1NMGLmMGRPmMGLrMGRPr≤1+ threshold(K) - N : number of multiple MG patterns - MGLr : MGL of referenced MG - MGRPr : MGRP - Option 3: When concurrent MGs are configured, the MGRP for each MG cannot be smaller than 40ms

- Overlapping issues

For Issue 2-3-1 & Issue 2-3-2, it is not necessary to consider X=0 as an optional UE capability. Because UE processing time should be considered as introducing non-zero X.

Proposal 1: Not consider X=0 as an optional UE capability.

For Issue 2-3-5 and Issue 2-3-6, as mentioned in Issue 2-3-3 in the last RAN4 meeting, Option 1 is preferred in Issue 2-3-3. If needed, we prefer to postpone to Rel-18.

Proposal 2: Postpone Issue 2-3-5 and Issue 2-3 6 to Rel-18 if needed.

For Issue 2-3-7, option 1 is better because  all specific cases can be configured by network and general requirements need to be specified.

Proposal 3: Specify general requirements no matter specific overlapping scenarios.

- Overhead issue

As mentioned in the last RAN4 meeting, our preference is Option 1 in Issue 2-4-1, and Option 2 in Issue 2-4-2.

Proposal 4: Define overhead cap.

Proposal 4-1: If defined, consider Option 2 in Issue 2-4-2.

Conclusion

In this contribution, we provided our views on overlapping and overhead for multiple concurrent and independent MG patterns for MG enhancements. Proposals are as follows.

- Overlapping issues

Proposal 1: Not consider X=0 as an optional UE capability.

Proposal 2: Postpone Issue 2-3-5 and Issue 2-3 6 to Rel-18 if needed.

Proposal 3: Specify general requirements no matter specific overlapping scenarios.

- Overhead issue

Proposal 4: Define overhead cap.

Proposal 4-1: If defined, consider Option 2 in Issue 2-4-2.

- Reference

[1] R4-2202603, “	WF on R17 NR MG enhancements – multiple concurrent MGs”, MediaTek