3GPP TSG-RAN WG4 Meeting # 102s-e	R4-2204162

Electronic Meeting, February 21 – March 3, 2022

Source:	ZTE Corporation

Title:	Discussion on RRM requirements for HO with PSCell

Agenda item:	10.10.2.2

Document for:	Discussion

# 1	Introduction

In the last 101-bis-e meeting, RRM requirements for HO with PSCell were discussed and related WF document was approved[1]. In this contribution ,we further provide our views on the open issues.

- Discussion

Delay requirement

Issue 2-2-3b: If UE SW processing and RF warm-up for PCell HO and PSCell addition/change are performed in parallel - Proposals - Option 1: (CATT, Apple, OPPO, ZTE, Huawei, Ericsson) - Tprocessing for HO with PSCell = max(Tprocessing for PCell HO, Tprocessing for PSCell addition/change) - Option 1a: (Qualcomm, vivo, Xiaomi, MTK, Apple, OPPO) - Tprocessing for HO with PSCell = max(Tprocessing for PCell HO, Tprocessing for PSCell addition/change) + [X] ms. - X=5 (Qualcomm, Xiaomi, OPPO) - X=10 (MTK, Apple, Qualcomm) - X=FFS and can be different for different HO with PSCell scenarios (vivo) - Option 2a: (Intel, CMCC, [Nokia]) - Tprocessing applies independently for PCell and PSCell. - FFS whether margin is needed

If UE SW processing and RF warm-up for PCell HO and PSCell addition/change are performed in parallel, the actual Tprocessing the UE experiences is the maximum value between Tprocessing for PCell HO and Tprocessing for PSCell addition/change.

- Proposal 1:Tprocessing for HO with PSCell = max(Tprocessing for PCell HO, Tprocessing for PSCell addition/change)

Requirements for NR-U

Issue 2-5-1: Requirements for HO with PSCell for NR-U - Proposals - Option 1 (CATT, MTK, Apple, Qualcomm, ZTE, Intel, vivo): - Postpone the requirement design of NR-U HO with PSCell until RAN4 completes the baseline requirement for HO with PSCell on licensed band. - Option 2 (Ericsson): - Requirements for HO with PSCell when PSCell is on NR-U shall be discussed in parallel with licensed carrier requirements and be specified in Rel-17 - Option 3 (Apple, vivo): - Discuss requirements for NR-U in Rel-18.

- For this issue, we would like to postpone the requirement design of NR-U till the discussion of RRM requirements for HO with PSCell on license band is basically stabilized. For option 3, whether to continue this topic in R18 may depend on the progress and we are fine if it is supported by most companies.

- Proposal 2: Postpone the requirement design of NR-U HO with PSCell until RAN4 completes the baseline requirement for HO with PSCell on licensed band.

- Conclusion

- Proposal 1:Tprocessing for HO with PSCell = max(Tprocessing for PCell HO, Tprocessing for PSCell addition/change)

- Proposal 2: Postpone the requirement design of NR-U HO with PSCell until RAN4 completes the baseline requirement for HO with PSCell on licensed band.

References

- R4-2202598, WF on further RRM enhancement for NR and MR-DC – HO with PSCell，vivo