3GPP TSG-RAN WG4 Meeting #102	R4-2204163

Electronic Meeting, February 21 – March 3, 2022

Agenda item:	10.19.4.2.1

Source: 		NTT DOCOMO, INC.

Title: 	Views on Rel-17 HST-SFN scheme

Document for:	Discussion

1. Introduction

At the last RAN4 meeting, we discussed Rel-17 HST-SFN and RAN4 agreed a WF [1]. This contribution presents our views on Rel-17 HST-SFN requirements.

2. Discussion

2.1. Rel-17 HST SFN scheme A

We show our views on following options [1].

Test setup - Option 1: - Reuse existing Rel-16 HST-SFN test set-up as a baseline - PDCCH/PDSCH/ SFN transmitted from two RRHs - TCI state 1 and TCI state 2 applied for TRP/RRH #2n, #2n+1 separately; TRS 1 and TRS 2 transmitted from TRP#2n, and #2n+1 separately Parameter; Value. FDD 15 kHz SCS; TDD 30 kHz SCS. CBW; 10 MHz; 40 MHz. Antenna configuration; 2x2; 2x4. DMRS type; Type 1. Number of DMRS symbols; 1+1+1. TDD pattern; 7D1S2U, S: 6D 4G 4U. TRS configuration; 10ms, 2 slot pattern. PDSCH mapping; Type A, Start symbol 2, Duration 12. Ds and Dmin; Ds =700m; Dmin=150m. Test metric; SNR @70% of maximum throughput. - Other options are not precluded. Maximum Doppler shift - 15 kHz SCS: - Option 1: 972 Hz - Option 2: 870 Hz - Other options are not precluded - 30 kHz SCS: - Option 1: 1667 Hz - Other options are not precluded

Test setup

We basically support treating the existing Rel-16 HST-SFN setup as baseline of Re1-17 HST-SFN scheme A setup. Also, we are open for other options.

Proposal 1: Reuse existing Rel-16 HST-SFN test set-up as a baseline (i.e. Option1)

- PDCCH/PDSCH/ SFN transmitted from two RRHs

- TCI state 1 and TCI state 2 applied for TRP/RRH #2n, #2n+1 separately; TRS 1 and TRS 2 transmitted from TRP#2n, and #2n+1 separately

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15 kHz SCS | TDD 30 kHz SCS |
| CBW | 10 MHz | 40 MHz |
| Antenna configuration | 2x2; 2x4 |  |
| DMRS type | Type 1 |  |
| Number of DMRS symbols | 1+1+1 |  |
| TDD pattern |  | 7D1S2U, S: 6D 4G 4U |
| TRS configuration | 10ms, 2 slot pattern |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |
| Ds and Dmin | Ds =700m; Dmin=150m |  |
| Test metric | SNR @70% of maximum throughput |  |

Maximum Doppler shift

We would like to share the following table first. Table 1 summarizes the maximum Doppler shift for the UE demodulation requirements of HST-SFN and Single-tap in NR/LTE [2][3].

Table 1: Summary of the Maximum Doppler shift in NR/LTE

|  | Deployment | Maximum Doppler shift |  |
| --- | --- | --- | --- |
| NR | Rel-16 HST-SFN | FDD: 870Hz | TDD: 1667Hz |
|  | Rel-16 Single-tap | FDD: 972Hz | TDD: 1667Hz |
| LTE | Rel-16 HST-SFN | 972Hz |  |
|  | Rel-16 Single-tap | 972Hz |  |

In Rel-16 RAN4 discussion, RAN4 agreed to introduce the UE demodulation requirement for HST-SFN deployment. In this deployment, the same TRS/PDSCH are transmitted from the all RRHs, so UE need to track the Doppler shift jump with negative to positive in the middle point of RRHs. This sudden Doppler shift jump has an impact on the determination of the maximum Doppler shift, and Rel-16 RAN4 agreed to define the requirement of HST-SFN FDD with 870Hz that is lower than the range of maximum Doppler shift that can be compensated by TRS (i.e. -875Hz ~ +875Hz) [2]. In Rel-17 RAN1 discussion, RAN1 agreed on the HST-SFN scheme A and scheme B. In HST-SFN scheme A, the same PDSCH is transmitted from all RRHs, but each RRH transmits the different TRS. It means that UE can track the Doppler shift per each RRH and need not track the Doppler shift jump that is observed in Rel-16 HST-SFN. Considering this feature, we think the Doppler shift of each RRH is the same as Single-tap model. Since Rel-16 RAN4 agreed to define the requirement of Single-tap FDD with 972Hz [2], Rel-17 HST SFN scheme A can support a larger maximum Doppler shift than that of Rel-16 HST-SFN (i.e. 870Hz).

In addition, it is important to consider the UE demodulation requirements of LTE HST-SFN deployment. In Rel-16 LTE discussion, RAN4 agreed to define the requirement of HST-SFN with 972Hz [3]. Therefore, the Maximum Doppler shift supported by Rel-16 LTE HST-SFN is higher than that of Rel-16 NR. Based on the basic NR motivation, we believe that RAN4 should aim to ensure that NR UEs have better performance than that of LTE UEs, or at least the same performance, so we prefer to support the following Maximum Doppler shift for HST-SFN scheme A.

- FDD 15kHz: 972Hz (i.e. same as Rel-16 LTE HST-SFN)

- TDD 30kHz: 1667Hz

Proposal 2: Define the following maximum Doppler shift

- 15 kHz SCS:

  - Option 1: 972 Hz

- 30 kHz SCS:

  - Option 1: 1667 Hz

2.2. Rel-17 HST SFN scheme B

We show our views on following options [1].

MCS and Rank - MCS 17 with rank 2 as a starting point Test setup and Channel Model definition - Option 1: Reuse existing Rel-16 HST-SFN test set-up as a baseline - Two TCI states with QCL A type information, and another one configured QCL type B information - PDCCH/PDSCH/PBCH SFN transmitted from two RRHs - TCI state 1 and TCI state 2 applied for for TRP/RRH #2n, #2n+1 separately; TRS 1 and TRS 2 transmitted from TRP#2n, and #2n+1 separately - HST SFN channel model specified in B.3.2 of TS 38.101-4 reused without modelling Doppler shift - Option 2: Reuse existing Rel-16 HST-SFN test set-up as baseline. Reuse the existing Rel-16 HST-SFN channel model (Ds=700m, Dmin=150m) with removing the two furthest paths corresponding to the two furthest TRP Select typical network implementation and consider the network implementation as a part of channel model (i.e., specify the function between the time and the pre-compensation value) to make sure TE implementation of pre-compensation has no impact on the UE performance during the test

We basically support treating the existing Rel-16 HST-SFN setup as baseline of Rel-17 HST-SFN scheme B setup except for Maximum Doppler shift. As for Maximum Doppler shift, we would like to clarify how to treat the pre-compensation first.

Observation 1: As for Maximum Doppler shift of Rel-17 HST-SFN scheme B, we would like to clarify how to treat the pre-compensation first. (If RAN4 agrees to introduce the requirement of HST-SFN scheme B)

3. Conclusion

In this contribution, we present our views on Rel-17 HST-SFN scheme. Our proposals and obsevations are summarized below.

Proposal 1: Reuse existing Rel-16 HST-SFN test set-up as a baseline (i.e. Option1)

- PDCCH/PDSCH/ SFN transmitted from two RRHs

- TCI state 1 and TCI state 2 applied for TRP/RRH #2n, #2n+1 separately; TRS 1 and TRS 2 transmitted from TRP#2n, and #2n+1 separately

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15 kHz SCS | TDD 30 kHz SCS |
| CBW | 10 MHz | 40 MHz |
| Antenna configuration | 2x2; 2x4 |  |
| DMRS type | Type 1 |  |
| Number of DMRS symbols | 1+1+1 |  |
| TDD pattern |  | 7D1S2U, S: 6D 4G 4U |
| TRS configuration | 10ms, 2 slot pattern |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |
| Ds and Dmin | Ds =700m; Dmin=150m |  |
| Test metric | SNR @70% of maximum throughput |  |

Proposal 2: Define the following maximum Doppler shift

- 15 kHz SCS:

  - Option 1: 972 Hz

- 30 kHz SCS:

  - Option 1: 1667 Hz

Observation 1: As for Maximum Doppler shift of Rel-17 HST-SFN scheme B, we would like to clarify how to treat the pre-compensation first. (If RAN4 agrees to introduce the requirement of HST-SFN scheme B)

References

- R4-2203091, WF on demodulation requirement for Enhancement on HST-SFN deployment, Intel Corporation

- TS 38.101-4

- TS 36.101