3GPP TSG-RAN WG4 Meeting # 102-e	R4-2204286

# Electronic Meeting, February 21 – March 3, 2022

Agenda item:	10.20.3.3

Source: 	OPPO

Title: 	RRM measurement relaxations for RedCap UE

Document for:	Discussion and decision

Introduction

A WF [1] on RRM measurement relaxations for RedCap UE have been approved in RAN4#101-bis-e meeting.

In this paper, we provide our views on the remaining issues of RRM measurement relaxation.

Discussion

For Rel-17 requirements, RAN4 agreed relaxation requirements the following scenarios:

| 1 | Rel-17 stationary criterion alone is configured and satisfied | YES and requirements are defined |
| --- | --- | --- |
| 2 | Rel-17 stationary and Rel-17 not at the cell edge criterion are both configured and satisfied | YES and requirements are defined |
| 3 | Rel-17 stationary and Rel-17 not at the cell edge criterion are both configured however only Rel-17 stationary criterion is satisfied | YES (same RRM relaxation requirements as scenario 1) |
| 4 | Rel-17 stationary and Rel-17 not at the cell edge criterion are both configured however only Rel-17 not at cell edge criterion is satisfied | No relaxation |
| 5 | Rel-17 stationary and Rel-17 not at the cell edge criterion are both configured however none of them is satisfied | No relaxation |
| 6 | Rel-17 stationary criterion alone is configured but not satisfied | No relaxation |

RAN2 agreement means that no any relaxation when only R17 not-at-cell-edge criteria is met. If network also configured R16 not-at-cell-edge is met, R16 legacy relaxation can be applied, otherwise, no relaxation is allowed.

Issue 2-1-2: Relaxation when multiple criteria of Rel-16 and Rel-17 are satisfied - Proposals - Option 1: UE is allowed to meet the requirements that are most relaxed out of Rel-16 and Rel-17 requirements. (Nokia vivo, Ericsson, CMCC MTK ZTE) - Option 2: Up to UE implementation (Huawei xiaomi oppo ZTE) - Option 3: The UE shall perform Rel-17 RRM relaxation method (Apple MTK xiaomi) - Option 3a: A note shall be added to the rel-17 RRM relaxation to mention that when rel-17 RRM relaxation criterion is fulfilled then the rel-16 RRM relaxation shall be disabled (MTK)

It is allowed to configure R16 low mobility criteria for R17 redcap UE. When NW configures both R16/R17 relaxation criteria and the UE fulfills both, it is up to UE implementation to select either Rel-16 or Rel-17 relaxation operation.

Proposal 1: It is up to UE implementation to select either Rel-16 or Rel-17 relaxation operation if UE is configured with R16 and R17 relaxation criteria.

RRM measurement relaxation for Redcap at Idle/Inactive state Issue 2-2-2: Scaling factor value when Rel-17 single criteria (stationary) is satisfied - Proposals - Option 1: 8 (apple) - Option 2: >3 (Huawei xiaomi oppo MTK) - Option 3: 4 (CMCC Ericsson) - Option 4: [5, 10, 30, 100] (Nokia) - Option 5: [5, 10, 20, 50, 100] (Qualcomm) - Option 6: between 4 and 8 (vivo) - Option 7: 5 (MTK) Tentative agreement: Select one from [4 5 6 8]

For stationary criterion, RAN2 agreed to reuse R16 low mobility criterion, as part or whole of Rel-17 stationary criterion in RRC_IDLE/INACTIVE. When NW configures both Rel-17 stationary criterion and Rel-16 low mobility criterion, NW configures different Rel-17 thresholds (i.e., SSearchDeltaP_stationary/TSearchDeltaP_stationary) from Rel-16 (SSearchDeltaP / TSearchDeltaP). How to configure the criterion (e.g. more stringent) is left to NW implementation (i.e. no specification impact to RAN2).  The requirements for R16 low mobility criterion can be used as baseline for Rel-17 stationary criterion, but they could be further relaxed with larger scaling factors (e.g., K_RedCap = 6) due to different Rel-17 thresholds when single Rel-17 criteria is satisfied.

Proposal 2: When single Rel-17 criteria is satisfied, define larger scaling factor K_RedCap =6.

Issue 2-2-3: The value of the one fixed long measurement period when both Rel-17 criteria are satisfied - Proposals - Option 1: 8 hours (Apple MTK) - Option 2: > 1 hour (Huawei xiaomi oppo Ericsson) - Option 3: 2 hours (CMCC) - Option 5: one value between 2 and 24 hours (vivo) - Option 6: [1h, 2h, 6h, 24h] (Nokia) - Option 7: [2 hour, 4 hours, 8 hours, 12 hours, 24 hours] (Qualcomm) Tentative agreement: Select one from [2 8 12 24] hours

when both Rel-17 stationary and Rel-17 not-at-cell-edge criteria are satisfied, longer measurement period(s) are expected than 1 hour. Whether multiple values should be defined depends on the conclusion for the case when single Rel-17 criteria is satisfied. If it was agree to select one value, we prefer 8 hours.

Proposal 3: When both Rel-17 stationary and Rel-17 not-at-cell-edge criteria are satisfied, longer measurement period(s) are expected as 8 hours.

Conclusion

In this paper, we provide our views on RRM measurement relaxation.

Proposal 1: It is up to UE implementation to select either Rel-16 or Rel-17 relaxation operation if UE is configured with R16 and R17 relaxation criteria.

Proposal 2: When single Rel-17 criteria is satisfied, define larger scaling factor K_RedCap =6.

Proposal 3: When both Rel-17 stationary and Rel-17 not-at-cell-edge criteria are satisfied, longer measurement period(s) are expected as 8 hours.

- References

[1] R4-2202672, WF on RedCap RRM requirements, RAN4#101-bis-e

[2] R4-2201148, RRM measurement relaxations for RedCap UE, OPPO