3GPP TSG-RAN4 Meeting #102-e	R4-2204342

# Electronic Meeting, Feb. 21 – Mar. 3, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR on L1-RSRP measurement requirements for inter-cell BM in R17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Core |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduce L1-RSRP measurement requirements for inter-cell BM in R17 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce L1-RSRP measurement requirements for inter-cell BM in R17 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirement for inter-cell L1-RSRP measurements is specified in R17 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.12.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of change 1>>

9.12.4	L1-RSRP measurement requirements

9.12.4.1	Inter-cell SSB based L1-RSRP Reporting for Known Cells

If a cell with PCI different from serving cell in known according 9.12.2, the UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_InterCell_Known.

The value of TL1-RSRP_Measurement_Period_SSB_InterCell_Known is defined in Table 9.12.4.1-1 for FR1 and Table 9.12.4.1-2 for FR2, where

-	M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

-	N= 8.

For FR1 and FR2,

-	P=MCell1-TSSBTSMTCperiod, when SSB is not overlapped with measurement gap and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P is Psharing factor_InterCell_L1, when SSB is not overlapped with measurement gap and SSB is fully overlapped with SMTC period (TSSB = TSMTCperiod)

-	P=MCell1-TSSBMGRP-TSSBTSMTCperiod, when SSB is partially overlapped with measurement gap and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TSSB < 0.5*TSMTCperiod

-	P is Psharing factor_InterCell_L11-TSSBMGRP, when SSB is partially overlapped with measurement gap and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and TSMTCperiod = MGRP and TSSB = 0.5*TSMTCperiod

-	P= MCell1-TSSBTSMTCperiod, when SSB is partially overlapped with measurement gap (TSSB <MGRP) and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap.

-	P is  Psharing factor_InterCell_L11-TSSBMRGP, when SSB is partially overlapped with measurement gap and SSB is fully overlapped with SMTC occasion (TSSB = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP)

-	Psharing factor_InterCell_L1 = 1, if the SSB configured for L1-RSRP measurement outside measurement gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped with the RSSI symbols indicated by ss-RSSI-Measurement and 1data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured.

-	Otherwise,

-	Psharing factor_InterCell_L1 = 3/2, if higher layer parameter timeRestrictionForChannelMeasurement is not configured

-	Psharing factor_InterCell_L1 = 3, if higher layer parameter timeRestrictionForChannelMeasurement is configured

-		Longer evaluation period is expected, if Psharing factor = 3 for serving cell L1-RSRP measurement requriements according clause 9.2.4.1, and higher layer parameter timeRestrictionForChannelMeasurement is configured for both serving cell and any cell with a different PCI, and no requirements are specified.

Where:

-	TSSB = ssb-periodicityServingCell

-	TSMTCperiod = the configured SMTC period

-	MCell = the number of cells with different PCIs whose SSBs are overlapped.

If the high layer in TS 38.331 [2] signaling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, provided the SMTC offset of all CCs in FR2 have the same offset.

Longer evaluation period would be expected if the combination of SSB, SMTC occasion and measurement gap configurations does not meet pervious conditions.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 serving cell, longer L1 RSRP measurement period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

Table 9.12.4.1-1: Inter-cell L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_InterCell_Known for known cells with different PCIs in FR1

| Configuration | TL1-RSRP_Measurement_Period_SSB_InterCell_Known (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1 when TSSB ≤ 40 ms and highSpeedMeasFlag-r16 are configured; otherwise K = 1.5. Note 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or [intraRAT-MeasurementEnhancement-r16] |  |

Table 9.12.4.1-2: Inter-cell L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_InterCell_Known for known cells with different PCIs in FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB_InterCell_Known (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

9.12.4.2	Inter-cell SSB based L1-RSRP Reporting for Unknown Cells

If a cell with PCI different from serving cell in unknown according 9.12.2, the UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_InterCell_Unknown.

If the configured SSB resource for inter-cell L1-RSRP measurements on unknown cells are fully non-overlapped with SMTC, no RRM requirements are specified.

The value of TL1-RSRP_Measurement_Period_SSB_InterCell_Unknown is defined in Table 9.12.4.2-1 for FR1 and Table 9.12.4.2-2 for FR2, where

-	M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

-	N= 8.

For FR1 and FR2,

-	P is Psharing factor_InterCell_L1, when SMTC occasion is not overlapped with measurement gap and SSB is not fully non-overlapped with SMTC period

-	P is Psharing factor_InterCell_L11-TSMTCperiodMRGP, when SSB is not fully non-overlapped with SMTC period and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP)

-	Psharing factor_InterCell_L1 = 1, if the SSB configured for L1-RSRP measurement outside measurement gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped with the RSSI symbols indicated by ss-RSSI-Measurement and 1data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured,

-	Otherwise,

-	Psharing factor_InterCell_L1 = 3/2, if higher layer parameter timeRestrictionForChannelMeasurement is not configured

-	Psharing factor_InterCell_L1 = 3, if higher layer parameter timeRestrictionForChannelMeasurement is configured

-		Longer evaluation period is expected, if Psharing factor = 3 for serving cell L1-RSRP measurement requriements according clause 9.2.4.1, and higher layer parameter timeRestrictionForChannelMeasurement is configured for both serving cell and any cell with a different PCI, and no requirements are specified.

Where:

-	TSSB = ssb-periodicityServingCell

-	TSMTCperiod = the configured SMTC period

If the high layer in TS 38.331 [2] signaling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, provided the SMTC offset of all CCs in FR2 have the same offset.

Longer evaluation period would be expected if the combination of SSB, SMTC occasion and measurement gap configurations does not meet pervious conditions.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 serving cell, longer L1 RSRP measurement period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

Table 9.12.4.2-1: Inter-cell L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_InterCell_Unknown for unknown cells with different PCIs in FR1

| Configuration | TL1-RSRP_Measurement_Period_SSB_InterCell_Unknown (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)* TSMTCperiod) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX, TSMTCperiod)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1 when TSMTCperiod ≤ 40 ms and highSpeedMeasFlag-r16 are configured; otherwise K = 1.5. Note 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or [intraRAT-MeasurementEnhancement-r16] |  |

Table 9.12.4.1-2: Inter-cell L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_InterCell_Unknown for unknown cells with different PCIs in FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB_InterCell_Unknown (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)* TSMTCperiod) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX, TSMTCperiod )) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

9.12.4.3	Inter-cell CSI-RS based L1-RSRP Reporting for Known Cells

[Editor’s note: TBD]

<< End of change 1>>