3GPP TSG-RAN WG4 Meeting # 102-e	R4-2204392

# Electronic Meeting, February 21 – March 3, 2022

Agenda item:	10.9.4.3.1

Source:	Intel Corporation

Title:	HST FR2 PUSCH simulation results

Document for:	Discussion

# Introduction

In RAN4 #101-bis-e meeting WF on HST FR2 BS demodulation requirements was agreed[1]. In this paper we provide simulation results for PUSCH performance requirements derivation.

# Simulation results

Table 1. HST Bi-directional channel model, 50 MHz

|  | 1 DMRS | 1+1 DMRS |
| --- | --- | --- |
| MCS 16 | 10 | 9.4 |
| MCS 17 | 10.5 | 9.6 |
| MCS 18 | 11.3 | 11.2 |
| MCS 19 | 12.6 | 12.3 |
| MCS 20 | 14.4 | 14.3 |

Table 2. HST Bi-directional channel model, 200 MHz

|  | 1 DMRS | 1+1 DMRS |
| --- | --- | --- |
| MCS 16 | 8.2 | 8.1 |
| MCS 17 | 8.4 | 8.4 |
| MCS 18 | 9.4 | 9.4 |
| MCS 19 | 10.4 | 10.4 |
| MCS 20 | 12.3 | 12.2 |

Table 3. Static channel model with frequency offset, 50 MHz

|  | 1 DMRS | 1+1 DMRS |
| --- | --- | --- |
| MCS 16 | 10 | 9.3 |
| MCS 17 | 10.5 | 9.6 |
| MCS 18 | 11.3 | 11.2 |
| MCS 19 | 12.5 | 12.3 |
| MCS 20 | 14.4 | 14.3 |

Table 4. Static channel model with frequency offset, 200 MHz

|  | 1 DMRS | 1+1 DMRS |
| --- | --- | --- |
| MCS 16 | 8.2 | 8.1 |
| MCS 17 | 8.4 | 8.4 |
| MCS 18 | 9.4 | 9.4 |
| MCS 19 | 10.4 | 10.4 |
| MCS 20 | 12.3 | 12 |

Observation #1: There is almost the same demodulation performance at 70% of max throughput with HST bi-directional and static channel model.

Observation #2: The performance in scenarios with 50MHz CBW is worse compared to the performance in scenarios with 20MHz CBW.

# Conclusion

In this paper we provide link-level evaluations of HST-FR2 PUSCH performance.

# References

- R4-2203006 “WF on BS demodulation requirement for FR2 HST”, Nokia, Nokia Shanghai Bell, RAN4 #101-bis-e, January 2022