3GPP TSG-RAN WG4 Meeting # 102-e	R4-2204400

Electronic Meeting, 21th February–3th March, 2022

Agenda Item:	10.10.2.2

Source:	Intel Corporation

Title:	Discussion about HO with PSCell

Document for:	Discussion

# Introduction

According to WF about HO with PSCell[1], there are many open issues:

- Timeline of Tprocessing - Timeline for HO with PSCell - Requirements for HO with PSCell for NR-U

In this contribution, we will provide our views regarding these issues.

# Discussion

Issue 2-2-3b: If UE SW processing and RF warm-up for PCell HO and PSCell addition/change are performed in parallel - Proposals - Option 1: (CATT, Apple, OPPO, ZTE, Huawei, Ericsson) - Tprocessing for HO with PSCell = max(Tprocessing for PCell HO, Tprocessing for PSCell addition/change) - Option 1a: (Qualcomm, vivo, Xiaomi, MTK, Apple, OPPO) - Tprocessing for HO with PSCell = max(Tprocessing for PCell HO, Tprocessing for PSCell addition/change) + [X] ms. - X=5 (Qualcomm, Xiaomi, OPPO) - X=10 (MTK, Apple, Qualcomm) - X=FFS and can be different for different HO with PSCell scenarios (vivo) - Option 2a: (Intel, CMCC, [Nokia]) - Tprocessing applies independently for PCell and PSCell. - FFS whether margin is needed

For this issue, the assumption is that UE SW processing and RF warm-up for PCell HO and PSCell addition/change are performed in parallel, there is no dependency between PCell and PSCell processing. The question is also related to issue 2-2-8a about the definition of Tprocessing in each delay requirement for PSCell and PCell.

Issue 2-2-8a: How the HO with PSCell delay requirements are specified - For the parallel processing case of HO with PSCell, PSCell addition delay requirements and HO delay requirements are defined separately: - PSCell addition delay= TRRC_delay + Tprocessing + Tsearch + T∆ + TPSCell_ DU + 2 ms - HO delay = TRRC_delay +Tinterrupt = TRRC_delay +Tsearch + TIU + Tprocessing + T∆ + Tmargin ms

From our understanding, Tprocessing in  Issue 2-2-8a will apply independently for PCell and PSCell, where some margin, X, can be considered as well. Therefore, the equation will be:

- For the parallel processing case of HO with PSCell, PSCell addition delay requirements and HO delay requirements are defined separately:

    - PSCell addition delay= TRRC_delay + (Tprocessing_PSCell +X) + Tsearch + T∆ + TPSCell_ DU + 2 ms

    - HO delay = TRRC_delay +Tinterrupt = TRRC_delay +Tsearch + TIU + (Tprocessing_PCell +X) + T∆ + Tmargin ms

Where Tprocessing_PCell and Tprocessing_PSCell are UE processing time for PCell HO and PSCell addition respectively, and X is margin with FFS value.

Proposal 1: For the parallel processing case of HO with PSCell, PSCell addition delay requirements and HO delay requirements are defined separately:

    - HO delay = TRRC_delay +Tsearch + TIU + (Tprocessing_PCell +X) + T∆ + Tmargin ms

    - PSCell addition delay= TRRC_delay + (Tprocessing_PSCell +X) + Tsearch + T∆ + TPSCell_ DU + 2 ms

Where Tprocessing_PCell and Tprocessing_PSCell are UE processing time for PCell HO and PSCell addition respectively, and X is margin with FFS value.

For sequential processing, there is agreement in last meeting:

- Introduce extra margin Y ms for sequential processing case comparing to parallel processing case for UE SW processing and RF warm-up for [PCell handover] and PSCell addition/change - Y = [10] ms - Note: no extra interruption is required

Therefore, for the sequential processing case of HO with PSCell, the delay requirements for PCell and PSCell are:

    - THO = TRRC_delay + Tsearch_PCell + (Tprocessing_PCell + X+ [10]ms) + TIU  + T∆_PCell + Tmargin

    - TPSCell= TRRC_delay + Tsearch_PCell + T∆_PCell + Tmargin + Tsearch_PSCell + T∆_PSCell + (Tprocessing_PSCell + X + [10]ms) +TPSCell_ DU + Tmargin

Proposal 2: For the sequential processing case of HO with PSCell, the delay requirements for PCell and PSCell are:

    - THO = TRRC_delay + Tsearch_PCell + (Tprocessing_PCell + X+ [10]ms) + TIU  + T∆_PCell + Tmargin

    - TPSCell= TRRC_delay + Tsearch_PCell + T∆_PCell + Tmargin + Tsearch_PSCell + T∆_PSCell + (Tprocessing_PSCell + X + [10]ms) +TPSCell_ DU + Tmargin

Where Tprocessing_PCell and Tprocessing_PSCell are UE processing time for PCell HO and PSCell addition respectively, and X is margin with FFS value.

Issue 2-5-1: Requirements for HO with PSCell for NR-U - Proposals - Option 1 (CATT, MTK, Apple, Qualcomm, ZTE, Intel, vivo): - Postpone the requirement design of NR-U HO with PSCell until RAN4 completes the baseline requirement for HO with PSCell on licensed band. - Option 2 (Ericsson): - Requirements for HO with PSCell when PSCell is on NR-U shall be discussed in parallel with licensed carrier requirements and be specified in Rel-17 - Option 3 (Apple, vivo): - Discuss requirements for NR-U in Rel-18.

Due to the limited time, we suggest to postpone the requirement design of NR-U HO with PSCell until RAN4 completes the baseline requirement for HO with PSCell on licensed band.

Proposal 3: Postpone the requirement design of NR-U HO with PSCell until RAN4 completes the baseline requirement for HO with PSCell on licensed band.

# Conclusion

In this contribution, we provide our views regarding the HO with PSCell:

Proposal 1: For the parallel processing case of HO with PSCell, PSCell addition delay requirements and HO delay requirements are defined separately:

    - HO delay = TRRC_delay +Tsearch + TIU + (Tprocessing_PCell +X) + T∆ + Tmargin ms

    - PSCell addition delay= TRRC_delay + (Tprocessing_PSCell +X) + Tsearch + T∆ + TPSCell_ DU + 2 ms

Where Tprocessing_PCell and Tprocessing_PSCell are UE processing time for PCell HO and PSCell addition respectively, and X is margin with FFS value.

Proposal 2: For the sequential processing case of HO with PSCell, the delay requirements for PCell and PSCell are:

    - THO = TRRC_delay + Tsearch_PCell + (Tprocessing_PCell + X+ [10]ms) + TIU  + T∆_PCell + Tmargin

    - TPSCell= TRRC_delay + Tsearch_PCell + T∆_PCell + Tmargin + Tsearch_PSCell + T∆_PSCell + (Tprocessing_PSCell + X + [10]ms) +TPSCell_ DU + Tmargin

Where Tprocessing_PCell and Tprocessing_PSCell are UE processing time for PCell HO and PSCell addition respectively, and X is margin with FFS value.

Proposal 3: Postpone the requirement design of NR-U HO with PSCell until RAN4 completes the baseline requirement for HO with PSCell on licensed band.

# Reference

[1] R4-2202598, WF on further RRM enhancement for NR and MR-DC – Handover with PSCell, vivo