3GPP TSG RAN4 Meeting #102-e	R4-2204428

Electronic Meeting, 21st February – 3rd March, 2022

Title: 	Discussion on Rel-17 RAN4 UE feature list

Source: 	Intel Corporation

Agenda item:	8

Document for:	Discussion

# Introduction

This meeting marks the second meeting of RAN4 Rel-17 UE feature list. In the previous RAN4 WG meeting, an initial RAN4 UE feature list for Rel-17 was approved in R4-2202400 [1] and an LS was sent to RAN2 about the approved feature list in [2].

# Discussion

## NR and MR-DC measurement gap enhancements [NR_MG_enh]

The following UE features proposed for the Rel-17 NR and MR-DC measurement gap enhancements WI were stable in the last meeting. However, they are somehow not included in the last approved version of feature list. Thus, we propose in this meeting to include them in the feature list and LS to RAN2.

Table 1. NR_MG_enh feature list

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 19-3-1 | Pre-configured measurement gap with network-controlled activation and deactivation mechanism | Capability of supporting preconfigured measurement gap with network-controlled mechanism for activation and deactivation |  | yes | no | UE does not support pre-configured measurement gap with Network-controlled mechanism | per UE | No | No |  |  | Optional with capability signalling |
| 19-3-2 | Pre-configured measurement gap with UE autonomous activation and deactivation mechanism | Capability of supporting preconfigured measurement gap with UE autonomous mechanism for activation and deactivation |  | yes | no | UE does not support pre-configured measurement gap with UE autonomous mechanism | per UE | No | No |  |  | Optional with capability signalling |

## Extending current NR operation to 71GHz [NR_ext_to_71GHz]

The following UE features are proposed for the Rel-17 Extending current NR operation to 71GHz WI.

Table 3. NR_ext_to_71GHz feature list

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15-2 | FR2-2 channel bandwidths for each SCS in each band for DL and UL for a single CC | Support of FR2-2 channel bandwidths 1) 120 kHz SCS: {100, 400} MHz CBW 2) 480 kHz SCS: {400, 800, 1600} MHz CBW 3) 960 kHz SCS: {400, 800, 1600, 2000} MHz CBW | FFS | Yes | No | UE cannot support some UE channel bandwidths | Per band | N/A | Applicable to FR2-2 only | N/A | UE indicating the support of specific SCS per band (RAN1 features X-Y) is required to support all CBWs corresponding to this SCS No additional capability signalling is needed. | NA |
| 15-3 | [Improved ON/ON transient period] | 1) Support of improved ON/ON transient period of X < 5us (X is FFS) | FFS | Yes | No | UE does not support improved ON/ON transient period and support 5us transient period | Per UE | N/A | Applicable to FR2-2 only | N/A | Further RAN4 discussion is required on whether to support improved ON/ON transient period and X value | Optional with capability signalling |

## NR HST enhancements in FR2 [NR_HST_FR2]

The following UE features are proposed for the Rel-17 NR FR2 HST WI. More details can be found in our companion paper [6].

Table 4. NR_HST_FR2 feature list

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x-1 | Support of FR2 HST operation | 1) Support of FR2 UE PC6 2) Support of enhanced RRM requirements for FR2 HST 3) Support of demodulation processing for FR2 HST | N/A | Yes | No | UE is not able to meet the enhanced requirements in HST FR2 | Per Band | No | Applicable to FR2 only | N/A | FR2 UE power class PC6 signalling is used to indicate support of feature group | Optional with capability signalling |

## Further enhancement on NR demodulation performance [NR_demod_enh2-Perf]

In our companion paper [3], we provide the detailed view on feature list for Rel-17 Further enhancement on NR demodulation performance WI. In Table 5, we summarize our proposal for this WI.  Based on WF on CRS-IM requirements [4], it was agreed to define the separate capabilities for Scenario 1 (DSS) and Scenario 2 (Non-DSS). However, it is still open whether to define the separate capability signalling for Scenario 2 for UEs capable of performance CRS-IM without the NWA signalling on LTE channel bandwidth. Table 5 captures our proposal that definition of such capability is not needed, but it can be updated based on outcome of RAN4 discussion.

Table 5. NR_demod_enh2-Perf feature list

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x-1 | CRS-IM receiver for DSS scenario | 1) Support of CRS-IM receiver in scenarios with serving and interference cells are in DSS mode and with overlapping spectrum |  | Yes | No | UE can’t apply mitigation of inter-cell LTE CRS interference | Per UE | No | FR1 only | N/A |  | Optional with capability signalling |
| x-2 | CRS-IM receiver for non-DSS scenario | 1) Support of CRS-IM receiver in scenarios with serving cell is NR cell and interference cell(s) is(are) LTE cell(s) and with overlapping spectrum |  | Yes | No | UE can’t apply mitigation of inter-cell LTE CRS interference | Per UE | No | FR1 only | N/A |  | Optional with capability signalling |

## NR HST enhancements in FR1 [NR_HST_FR1_enh]

In the previous RAN4 meeting, features for FR1 HST enhancements were agreed. The only open issue is granularity of FR1 HST CA Demodulation features. The detailed discussion on this issue is provided in our companion paper [5]. Summary of our proposal is presented in Table 6.

Table 6. NR_HST_FR1_enh feature list

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 18-4 | Support of enhanced Demodulation requirements for CA in HST SFN FR1 | 1) Support of demodulation processing for HST SFN CA scenario in FR1 | Rel-16 RAN4 feature 10-2 | Yes | No | UE is not able to apply demodulation processing for HST SFN CA scenario in FR1 | Per FSPC | No | FR1 only | N/A |  | Optional with capability signalling |

# Conclusions

In the last meeting we sent our initial version of feature list [1] in an LS in [2]. It is important to provide our feature list to other WG-s after this meeting so in this paper we continue to share some of our views on specific topics.

# References

- R4-2202400, Rel-17 RAN4 UE feature list for NR, CMCC

- R4-2202401, LS on Rel-17 RAN4 UE features list for NR, CMCC

- R4-2204375 “Discussion on UE feature list and capability signalling”, Intel Corporation, RAN4 #102-e, February 2022.

- R4-2203131 “WF on general part and 15 kHz NR SCS scenario for CRS-IM receiver” , China Telecom, RAN4 #101-bis-e, January 2022.

- R4-2204384 “Discussion on HST FR1 CA PDSCH performance requirements”, Intel Corporation, RAN4 #102-e, February 2022.

- R4-2201877, Views on FR2 HST PDSCH performance requirements, Intel Corporation.