3GPP TSG-RAN WG4 Meeting # 102-e 	R4-2204503

# Electronic Meeting, February 21 – March 3, 2022

Agenda Item:		10.13.2.2

Source: 		Qualcomm Incorporated

Title: 	Coexistence simulation results for HAPS

Document for:	Discussion

- Introduction

The HAPS and TN coexistence performance were presented with simulation results in this contribution. The simulations were updated based on the latest agreed HAPS simulation assumptions captured in [1]. The major change is the latest agreed UL scheduled UE number and UE bandwidth allocation method for HAPS per [1] are considered.

Through the simulation result, ACIR/ACS values and interference coordination methods were proposed to conclude the HAPS study.

# 2. Discussion

## 2.1 Simulation topology for coexistence

The topology used for coexistence is showed in Figure 1 and Figure 2 that TN clusters are dropped uniformly in the HAPS coverage. Only one TN cluster is dropped in each snapshot, and the centre of TN cluster is dropped uniformly in the HAPS coverage. Through this way, different HAPS performance levels can be reached as showed in Figure 3. Wrap-around is not considered.

It needs to be mentioned that the HAPS topology is 7 equal-side hexagons. The UE is uniformly dropped, and the UE access will depend on the calculated coupling loss.

Figure 1. HAPS and TN coexistence topology

38km

100km

Figure 2. HAPS topology

Figure 3. HAPS DL SINR Distribution Heatmap in rural

## 2.2 Simulation assumptions for coexistence

The simulation assumptions are aligned with the agreements in [1] of RAN4 #101-bis e-meeting.

The HAPS UEs and TN UEs are assumed in the same coverage area, which means the HAPS UE is randomly dropped in the TN coverage.

For the HAPS UL interfering to TN UL scenario, since only 9 HAPS UEs in one cell, the HAPS’s interference will be averaged in the 19 base stations.

For the TN UL interfering to HAPS UL scenario, the accumulated interference from all the TN UEs in the HAPS coverage should be considered. In current simulation, only one TN cluster’s UEs are accumulated as the TN interference.

3-step ACLR is assumed. The UL bandwidth for HAPS UE and TN UE are different, and thus the ACLR calculation for asymmetric bandwidth is referred to 3GPP TR36.942 Chapter 5.1.1.4.1.

## 2.3 Simulation cases for coexistence

The coexistence test cases are summarized in Table 1. The target of HAPS is to extend the coverage; therefore, the interested scenario is rural.

Table 1. Coexistence test case summary

| Case No. | Scenario |
| --- | --- |
| 1 | HAPS DL interfering to AAS TN DL |
| 2 | AAS TN DL interfering to HAPS DL |
| 3 | HAPS UL interfering to AAS TN UL |
| 4 | AAS TN UL interfering to HAPS UL |
| Note: only rural scenario is considered. |  |

## 2.4 Simulation results for coexistence

The simulation results are summarized in Table 2 for better comparing the differences. The results are also included in the spreadsheet in the annex.

Table 2. Coexistence performance summary

| 1 | HAPS DL to AAS TN DL | Required ACIR [dB] | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Mean throughput loss | 38.41% | 23.95% | 12.26% | 5.14% | 1.86% | 0.62% | 0.20% | 0.06% |
|  |  | 5-ile throughput loss | 100.00% | 68.11% | 39.34% | 17.38% | 6.66% | 2.37% | 0.77% | 0.21% |
| 2 | AAS TN DL to HAPS DL | Required ACIR [dB] | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 55 |
|  |  | Mean throughput loss | 9.35% | 6.55% | 4.43% | 2.85% | 1.69% | 0.98% | 0.48% | 0.22% |
|  |  | 5-ile throughput loss | 100.00% | 100.00% | 100.00% | 100.00% | 25.79% | 11.51% | 7.52% | 2.37% |
| 3 | HAPS UL to AAS TN UL | Required ACIR [dB] | 5 | - | - | - | - | - | - | - |
|  |  | Mean throughput loss | 0.16% | - | - | - | - | - | - | - |
|  |  | 5-ile throughput loss | 1.22% | - | - | - | - | - | - | - |
| 4 | AAS TN UL to HAPS UL | Required ACIR [dB] | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |
|  |  | Mean throughput loss | 67.07% | 48.88% | 32.19% | 18.72% | 9.07% | 3.61% | 1.26% | 0.4% |
|  |  | 5-ile throughput loss | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |

Table 3. ACLR/ACS requirement for TN in 2GHz

| System |  | NR |
| --- | --- | --- |
| BS | ACLR | 45 dB |
|  | ACS | 46 dB |
| UE | ACLR | 30 dB (ACLR1), 43 dB (ACLR2) |
|  | ACS | 33 dB |

- HAPS interfering AAS TN

From test case 1, the needed ACIR is about 26dB to ensure that the TN DL edge throughput loss brought by HAPS DL is smaller than 5%. From test case 3, the performance impact brought by the HAPS UL is very limited.

Therefore, comparing with the ACLR/ACS requirement listed in Table 3, the HAPS deployment will not impact AAS TN deployment in 2GHz with current defined ACLR/ACS requirement in 3GPP.

- AAS TN interfering HAPS

From test case 2, the required ACIR value is about 52dB for 5-ile throughput loss, and 27dB for mean throughput loss. Due to the very low elevation angle (11 degree) in the edge area and the large coverage radius (100km), the edge performance of HAPS is vulnerable and easily impacted by external interference.

In test case 4, the 5-ile SINR is smaller than -10dB, therefore the edge throughput is 0. It is difficult to use 5-ile performance loss as the metric to measure the interference impact from TN to HAPS. If the mean throughput loss can be referred to measure the interference level, the required ACIR is about 29dB for mean throughput loss for test case 4.  The AAS TN UL interference is accumulated interference  and it is related closely with the statistical UE number. In current simulation, only one TN cluster is counted, if more TN clusters are considered in the HAPS coverage area, the required ACIR would be higher.

Table 4. Required ACIR summary for HAPS

| Case No. | Required ACIR | HAPS | TN |
| --- | --- | --- | --- |
| 1 | 26dB | BS ACLR | UE ACS |
| 2 | 27dB | UE ACS | BS ACLR |
| 3 | < 5dB | UE ACLR | BS ACS |
| 4 | 29dB | BS ACS | UE ACLR |
| Note: for case 2 and 4, the mean throughput loss < 5% is used as the criterion |  |  |  |

Observation 1: The interference from HAPS to AAS TN is acceptable. Introducing HAPS will not impact current TN deployment.

Observation 2: The edge performance of HAPS is vulnerable. It is difficult to measure the interference’s impact brought by other systems using 5-ile performance loss. The operators’ coordination mechanism is needed to enable the co-coverage of HAPS and TN, e.g., HAPS UE can handover/roam to TN network configured by network in this case.

Proposal 1: The ACLR/ACS for TN UE is also applicable for HAPS UE. HAPS can support existing TN UE.

Proposal 2: The frequency coordination measures are needed to enable HAPS and TN coexistence in the same coverage.  The HAPS operator should plan its frequency deployment considering the ACI impact from TN but there is no need to specify the corresponding RAN4 requirements.

# 3. Conclusion

In this paper, we present the HAPS coexistence performance with AAS TN, including both DL to DL and UL to UL scenarios.  We have the following observations and proposals:

Observation 1: The interference from HAPS to AAS TN is acceptable. Introducing HAPS will not impact current TN deployment.

Observation 2: The edge performance of HAPS is vulnerable. It is difficult to measure the interference’s impact brought by other systems using 5-ile performance loss. The operators’ coordination mechanism is needed to enable the co-coverage of HAPS and TN, e.g., HAPS UE can handover/roam to TN network configured by network in this case.

Proposal 1: The ACLR/ACS for TN UE is also applicable for HAPS UE. HAPS can support existing TN UE.

Proposal 2: The frequency coordination measures are needed to enable HAPS and TN coexistence in the same coverage.  The HAPS operator should plan its frequency deployment considering the ACI impact from TN but there is no need to specify the corresponding RAN4 requirements.

# References

- R4-2202992, Simulation assumptions for HAPS co-existence