3GPP TSG-RAN WG4 Meeting #102 -e	R4-2204538

Electronic Meeting, February 21 – March 3, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on measurement procedures for RedCap UEs

Agenda item:	10.20.3.1.5

Document for:	Discussion and Decision

Introduction

The RedCap WID [1] defines among its objectives the specification of support for UEs with reduced maximum UE bandwidth and reduced minimum number of Rx branches, which impacts on the UE performance. Since these assumptions are different from the ones used on the definition of the RRM requirements for NR UEs, RAN4 is currently working on defining requirements for RedCap UEs.

In RAN4 #100e, the simulation assumptions for SSB-based RRM measurements were agreed in [2] and the simulation assumptions for L1-RSRP measurements were agreed in [3]. In RAN4 #101e and #101-bis-e, companies brought simulation results based on the simulation assumptions.

In this contribution, we discuss the issues related to measurement procedures for RedCap UEs.

Discussion

Cell detection

In the last meeting, the following was agreed regarding PSS/SSS detection:

Whether to extend number of attempts (samples) for PSS/SSS detection for FR1 Number of attempts for PSS/SSS detection for FR1 is extended for RedCap UE with 1 Rx. Whether to extend number of attempts (samples) for PSS/SSS detection for FR2 Number of attempts for PSS/SSS detection for FR2 is extended for RedCap UE with 1 Rx. If number of attempts are increased, how much to increase for FR2 Extend the PSS/SSS detection delay by 1*8 SMTC without changing the lower boundary ‘600ms’ (8 is the beam sweeping factor) for a UE supporting power class 3, Mpss/sss_sync_w/o_gaps_RedCap = 32. - NOTE: FR2 power classes may need to be reconsidered depending on the conclusion of RF session

And the following still remains as open issues:

- PSS/SSS detection with 1 Rx If number of attempts are increased, how much to increase for FR1 - Option 1 (Apple, HW): 2 samples - Option 2 (vivo, MTK, Nokia, HW, E///): 1 sample - Option 3 (QC): Double the number of attempts Whether to extend the lower bound in PSS/SSS detection delay - Option 1 (Apple, HW, E///, vivo, Nokia, MTK): No in FR1 and FR2 - Option 2 (QC, OPPO): Yes for FR1 - Extend lower bound by factor X, e.g. X=2 - Option 2 (MTK): Extend the lower bound if detection delay is extended

Our simulation results are given in Table 1:

Table 1 – NR PSS-SSS acquisition time, in number of samples, required for a RedCap UE with 1Rx successfully detecting PSS and SSS in different scenarios in FR1 and FR2

|  | Cell Id: 674 |  | Cell Id: 673 |  |
| --- | --- | --- | --- | --- |
|  | Sync | Async | Sync | Async |
| FR1,15kHz,AWGN | 1 | 1 | 1 | 1 |
| FR1,30kHz,AWGN | 1 | 1 | 1 | 1 |
| FR1,15kHz,TDLA 30 ns | 1 | 1 | 1 | 2 |
| FR1,30kHz,TDLA 30 ns | 1 | 2 | 1 | 1 |
| FR1,15kHz,TDLB 100 ns | 1 | 1 | 1 | 1 |
| FR1,30kHz,TDLB 100 ns | 1 | 2 | 1 | 1 |
| FR1,15kHz,TDLC 300 ns | 1 | 1 | 1 | 1 |
| FR1,30kHz,TDLC 300 ns | 1 | 2 | 1 | 1 |
| FR2,120kHz,AWGN | 1 | 1 | 1 | 1 |
| FR2,240kHz,AWGN | 1 | 1 | 1 | 1 |
| FR2,120kHz,TDLA 30 ns | 1 | 2 | 1 | 1 |
| FR2,240kHz,TDLA 30 ns | 1 | 1 | 1 | 1 |

The results show that the worst cell detection performance cases, require at most 2 samples for successfully detecting PSS / SSS. This is the same as what was assumed with 2 RXs during the development of RRM requirements for NR Rel-15.

According to our simulation results, the successful acquisition of NR PSS/SSS for a RedCap UE with 1 Rx requires at most: 2 samples in FR1 and FR2, which is the same as in Rel-15.

However, it was already agreed that the time period for PSS/SSS detection is extended. Therefore, we propose that this extension is limited to 1 sample.

Time period for PSS/SSS detection, TPSS/SSS_sync_intra and TPSS/SSS_sync_inter, for RedCap UEs with 1 RX is extended by 1 sample in FR1.

According to the Rel-15 requirements, the lower bound of the PSS/SSS detection delay is 600 ms, which, for most SMTC periodicities is already enough to account for the extension of the number of required samples, according to our proposal in FR1. Therefore, we do not see reason to extend the lower bound.

Do not extend the lower bound for PSS/SSS detection delay in FR1 and FR2

Time index detection with 1 RX

In the last RAN4 meeting, the following was agreed regarding

Time index detection with 1 Rx Whether to extend time index delay in FR2 (MIB decoding) - Time index delay in FR2 (MIB decoding) is extended. Updates to time index detection performance metric - No consensus to change or update the already agreed simulation assumptions for time index detection performance metric. RAN4 to use the agreed simulation assumption as the baseline. Whether to extend time index delay in FR1 (PBCH-DMRS detection) - RAN4 to extend time index delay in FR1 (PBCH-DMRS detection) Whether to extend the lower boundary of time index detection delay - RAN4 to extend the lower boundary of time index detection delay, how much to extend is TBD.

The following is still listed as open issues:

- Time index detection with 1 Rx If extended, how much to extend time index delay in FR1 In section 6.1.2, it is agreed that RAN4 to extend time index delay in FR1 (PBCH-DMRS detection). - Option 1 (Apple, E///, MTK): by 1 SMTC - Option 2 (HW): Doubled, i.e. 6 samples needed in total - Option 3 (QC, HW): 7 attempts If extended, how much to extend time index delay in FR2 In section 6.1.2, it is agreed that time index delay in FR2 (MIB decoding) is extended. - Option 1 (vivo): 8 samples needed in total. - Option 2 (HW,MTK): 11 samples needed in total - Option 3 (E///): 4 samples needed in total

The simulation parameters are shown in Appendix A. In this section, the results are presented in terms of the number of acquisition attempts required to successfully reading the DMRS time index to achieve a target detection rate of 99% given a false alarm rate of 1%.

FR1 results

Table 2 – PBCH-DMRS time index reading for 1 Rx and 2 Rx for AWGN

|  | 15kHz |  | 30kHz |  |
| --- | --- | --- | --- | --- |
| SNR (dB) | 1Rx | 2Rx | 1Rx | 2Rx |
| -8 | 1 | 1 | 1 | 1 |
| -7 | 1 | 1 | 1 | 1 |
| -6 | 1 | 1 | 1 | 1 |
| -5 | 1 | 1 | 1 | 1 |
| -4 | 1 | 1 | 1 | 1 |
| -3 | 1 | 1 | 1 | 1 |

Table 3 – PBCH-DMRS time index reading for 1 Rx and 2 Rx for TDL-A-30

|  | 15kHz |  | 30kHz |  |
| --- | --- | --- | --- | --- |
| SNR (dB) | 1Rx | 2Rx | 1Rx | 2Rx |
| -8 | 4 | 3 | 5 | 3 |
| -7 | 4 | 2 | 4 | 2 |
| -6 | 4 | 2 | 3 | 2 |
| -5 | 3 | 2 | 3 | 2 |
| -4 | 3 | 2 | 3 | 2 |
| -3 | 2 | 1 | 2 | 1 |

Table 4 – PBCH-DMRS time index reading for 1 Rx and 2 Rx for TDL-C-300

|  | 15kHz |  | 30kHz |  |
| --- | --- | --- | --- | --- |
| SNR (dB) | 1Rx | 2Rx | 1Rx | 2Rx |
| -8 | 6 | 3 | 7 | 3 |
| -7 | 5 | 2 | 6 | 3 |
| -6 | 4 | 2 | 4 | 2 |
| -5 | 3 | 2 | 4 | 2 |
| -4 | 3 | 1 | 3 | 2 |
| -3 | 2 | 1 | 2 | 1 |

When comparing the number of acquisition attempts required for RedCap UEs with 1 Rx to legacy UEs with 2 Rx, we observe the following:

The simulations show that 4 samples are required for time index detection delay in RedCap with 1 Rx for FR1 at -6dB.

The simulations show that the time index detection delay should be extended by 2 samples in RedCap with 1 Rx for FR1 at -6dB compared to legacy UE with 2 Rx.

The simulations show that 7 samples are required for time index detection delay in RedCap with 1 Rx for FR1 at -8dB.

The simulations show that the time index detection delay should be extended by 4 samples in RedCap with 1 Rx for FR1 at -8dB compared to legacy UE with 2 Rx.

In the last RAN4 meeting, the moderator suggested to check the simulation condition (specially SNR and channel model) aiming to analyse the large span in the results within different companies. In this contribution, we propose to focus the analysis on the worst-case scenario within the channel profiles evaluated for a side condition of SNR=-6dB.

Define the time index detection delay in FR1 for RedCap UEs based on the side condition of -6dB

Define the time index detection delay in FR1 for RedCap UEs for the worst-case scenario (within the proposed channel profiles)

Extend the time index detection delay in FR1 by 2 samples at for RedCap UE with 1 RX

SSB based L3 measurements

In the last RAN4 meeting, the following issues were discussed without a conclusion:

Method for defining 1 Rx requirements for SSB based measurement, FR1 and FR2 - Option 1 (vivo, ZTE, Nokia, HW, E///, MTK): Keep measurement period same as Rel-15 - Relax the accuracy based on 3 samples - Option 2 (Apple, Oppo, QC): Only lower bound is extended while keeping the same number of samples. How much to relax? FR1 and FR2 - Option 2 (Nokia, E///, Apple, HW): 1 dB - Option 3 (MTK): 1.5 dB – 2 dB If measurement period is extended, how much to extend? FR1 and FR2 - Option 1 (Apple): Lower bound extended to 400 ms and 800 ms for FR1 and FR2 resp as follows: - Delay is max(400ms, ceil( 5 x Kp) x SMTC period) x CSSFintra for FR1 - Delay is max(800ms, ceil(Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period) x CSSFintra for FR2 Whether legacy RF margin can be considered for RedCap for FR1 - Option 2 (vivo, CMCC, E///, Nokia): Use same RF margin as in Rel-15 NR for RSRP accuracy requirements.

Method for defining 1 Rx requirements for SSB based measurement, FR1 and FR2

In the last RAN4 meeting, both of the remaining options listed in this open issue proposed to keep the same number of samples for SSB based measurements as Rel-15. It is up for discussion whether the lower bound of the delay is to be extended and whether to relax the accuracy based on 3 samples. In this section, the results of the SS-RSRP estimation and accuracy degradation are shown, and some of the results are taken from our previous contribution in [7].

For the requirements of SSB based measurements for RedCap UEs with 1 RX, keep the same measurement period as in Rel-15.

Since our proposal is to keep the same number of samples as in Rel-15, we see no reason to extend the lower bound of this measurement.

For the requirements of SSB based measurements for RedCap UEs with 1 RX, do not extend the lower bound.

The results presented in our contribution for RAN4 101-bis [7] show the RSRP degradation between RedCap UEs with 2 Rx branches and RedCap UEs with 1 Rx branch. The degradation is defined as follows:

- DeltaRSRP = measurement RSRP – ideal RSRP

- Accuracy = max( abs( DeltaRSRP(95th percentile),1Rx, NSamples), abs(DeltaRSRP(5th percentile, NSamples) ,1Rx, NSamples)

- DeltaRSRP = estimated RSRP – ideal RSRP

- Degradation(5th percentile) := (DeltaRSRP(5th percentile), 1Rx, N samples) - (DeltaRSRP(5th percentile), 2Rx, 5 samples)

- Degradation(95th percentile) := (DeltaRSRP(95th percentile), 1Rx, N samples) - (DeltaRSRP(95th percentile), 2Rx, 5 samples)

- Degradation = max(abs(degradation(95th percentile) ), abs(degradation(5th percentile))

Table 5 - Comparison of the RSRP degradation for 1 RX UEs and 2 RX UEs, from [7]

|  |  | Degradation [dB] |
| --- | --- | --- |
| 15 kHz | AWGN | 0.41 |
|  | TDLA-30ns | 0.74 |
|  | TDLB-100ns | 0.53 |
|  | TDLC-300ns | 0.59 |
| 30 kHz | AWGN | 0.41 |
|  | TDLA-30ns | 0.73 |
|  | TDLB-100ns | 0.53 |
|  | TDLC-300ns | 0.50 |
| 120 kHz | AWGN | 0.42 |
|  | TDLA-30ns | 0.70 |
| 240 kHz | AWGN | 0.41 |
|  | TDLA-30ns | 0.61 |

According to our simulation results, the accuracy degradation due to reducing the number of RX branches is up to 0.74 dB in FR1 and 0.70 dB in FR2.

For SSB-based RRM measurements, relax the accuracy level by 1 dB in both FR1 and FR2 for RedCap UEs with 1 RX.

Conclusion

- According to our simulation results, the successful acquisition of NR PSS/SSS for a RedCap UE with 1 Rx requires at most: 2 samples in FR1 and FR2, which is the same as in Rel-15.

- The simulations show that 4 samples are required for time index detection delay in RedCap with 1 Rx for FR1 at -6dB.

- The simulations show that the time index detection delay should be extended by 2 samples in RedCap with 1 Rx for FR1 at -6dB compared to legacy UE with 2 Rx.

- The simulations show that 7 samples are required for time index detection delay in RedCap with 1 Rx for FR1 at -8dB.

- The simulations show that the time index detection delay should be extended by 4 samples in RedCap with 1 Rx for FR1 at -8dB compared to legacy UE with 2 Rx.

- Time period for PSS/SSS detection, TPSS/SSS_sync_intra and TPSS/SSS_sync_inter, for RedCap UEs with 1 RX is extended by 1 sample in FR1.

- Do not extend the lower bound for PSS/SSS detection delay in FR1 and FR2

- Define the time index detection delay in FR1 for RedCap UEs based on the side condition of -6dB

- Define the time index detection delay in FR1 for RedCap UEs for the worst-case scenario (within the proposed channel profiles)

- Extend the time index detection delay in FR1 by 2 samples at for RedCap UE with 1 RX

- For the requirements of SSB based measurements for RedCap UEs with 1 RX, keep the same measurement period as in Rel-15.

- For the requirements of SSB based measurements for RedCap UEs with 1 RX, do not extend the lower bound.

- According to our simulation results, the accuracy degradation due to reducing the number of RX branches is up to 0.74 dB in FR1 and 0.70 dB in FR2.

- For SSB-based RRM measurements, relax the accuracy level by 1 dB in both FR1 and FR2 for RedCap UEs with 1 RX.

- References

- RP-211574, “Revised WID on support of reduced capability NR devices”, Ericsson

- R4-2115360, “Simulation assumptions for RedCap SSB based RRM measurement performance”, Huawei, HiSilicon

- R4-2125362. “Simulaton assumptions for RedCap L1 RSRP Measurement”, Ericsson

- R4-2115361, “Simulation assumptions for RedCap RLM and BFD performance”, Ericsson

- R4-2202670, “WF on RedCap RRM requirements”, Ericsson, RAN4 #101bis-e

- R4-2120418, “WF on RedCap RRM requirements”, Ericsson, RAN4 #101-e

- R4-2200869, “Discussion on measurement procedures for RedCap”, Nokia, Nokia Shanghai Bell, RAN4 #101bis-e

- Appendix A

Table A1: General parameters for the L3 measurements simulations

| Simulation parameters | Comments/values |  |
| --- | --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | 4 GHz | 30 GHz |
| System bandwidth | 5 MHz; 10 MHz | 50 MHz; 100 MHz |
| Prior knowledge of Cell 1 / Cell 2 by the UE | No / Yes | No / Yes |
| DRX | No | No |
| BS transmit antennas for SS blocks | 1 tx or single layer transmissions | 1 tx or single layer transmissions |
| UE receive antennas | 1RX | 1RX |
| Data and control channel subcarrier spacing | The same as SS block subcarrier spacing | The same as SS block subcarrier spacing |
| Measurement period (in number of measurement samples) | 5, 6, 7, 8 | 5, 6, 7, 8 |
| SS burst set configuration in Cell 1 and Cell 2: |  |  |
| - Subcarrier spacing | 15 kHz; 30 kHz | 120 kHz; 240 kHz |
| - Number of SS blocks per SS burst set | 1 | 1 |
| - SS burst periodicity | 5 ms | 5 ms |
| - Number of transmit antenna ports | 1 (the same port for NR-SSS, NR-PSS, NR-PBCH) | 1 (the same port for NR-SSS, NR-PSS, NR-PBCH) |
| - SS block BW | 20 PRBs | 20 PRBs |
| - Actual SS block transmissions | always transmitted | always transmitted |

Table A2: Cell-specific parameters for the L3 measurements simulations

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| E-UTRA RF Channel number | - | Channel 1 | Channel 1 |
| NR-PSS and NR-SSS sequences | - | To be indicated by companies | To be indicated by companies |
| PBCH and DMRS power offset with respect to NR-PSS and NR-SSS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS and NR-SSS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | Ms | 0 | 3 ms |
| SNR | dB | 6 dB | 1 dB |
| Es/IoT (calculated from SNR) | dB | N/A | -5.97 dB |
| Propagation conditions | - | FR1: AWGN TDL-A 30ns TDL-B 100ns TDL-C 300ns FR2: AWGN TDL-A 30ns |  |
| UE speed |  | FR1: 30 km/h FR2: 3 km/h |  |
| NOTE: the companies are encouraged to state channel model parameters together with the results, the parameters are to be further discussed and aligned |  |  |  |

## PBCH-DMRS time index detection

Table A5: Parameters for PBCH-DMRS time index detection

| Parameters | Value |
| --- | --- |
|  | FR1 |
| Carrier frequency | 4GHz |
| Subcarrier Spacing for SSB | 15kHz, 30kHz |
| SSB Periodicity | 20ms |
| Transmit antennas | 1 |
| Receive antennas | 1 |
| Propagation conditions | AWGN TDL-A 30ns TDL-C 300ns |
| UE speed | 30 km/h |
| CP type | Normal CP |
| PBCH-DMRS power offset | 0 dB |
| PBCH SNR | [-8 -7 -6 -5 -4 -3] dB |
| Detection method | One shot detection (without combining) |
| Performance metrics | Detection rate = 99% False alarm rate = 1% |