3GPP TSG-RAN WG4 Meeting # 102-e                                                        R4-2204750			R4-22xxxxx

# Electronic Meeting, February 21 – March 3, 2022

Source:	ZTE Corporation

Title:	TP for TR 37.717-41-11: DC_1A-7A-20A-38A_n78A

Agenda Item:	9.17.2

Document for:	Approval

- Introduction

This contribution provides a text proposal on DC_1A-3A-7A-20A_n38A  for TR37.717-11-21[1] .

- Reference

- TR 37.717-41-11  Rel-17 DC combinations LTE 4DL and one NR band v0.4.0, v0.5.0

- Text Proposal

----- Start of TP -----

    - 5.x.2	DC_1-7-20-38_n78

      - 5.x.2.1	Configuration for EN-DC

Table 5.x.2.1-1: Inter-band EN-DC configurations (five bands)

| EN-DC band configuration | UL EN-DC configuration(s) |
| --- | --- |
| DC_1A-7A-20A-38A_n78A | DC_1A_n78A DC_20A_n78A |

      - 5.x.2.2	∆TIB and ∆RIB values

Table 5.x.2.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-20-38_n78 | 1 | 0.6 |
|  | 7 | 0.7 |
|  | 20 | 0.6 |
|  | 38 | 0 |
|  | n78 | 0.8 |

Table 5.x.2.2.-2: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-7-20-38_n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.2 |
|  | 38 | 0.4 |
|  | n78 | 0.8 |

      - 5.x.2.3	Reference sensitivity exceptions

There are no additional MSD requirements for this band combination.

----- End of TP -----