3GPP TSG-WG4 Meeting #102-e	R4-2204977

# Electronic Meeting, February 21 – March 3, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | XXX | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on PC3 intra-band non-contiguous UL CA requirements for 2LO case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, Huawei, Skyworks |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-Core |  |  |  |  |  | Date: |  |  | 2022-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the discussion in R17 UE FR1 enhancement WI, for 2LO architecture, single carrier MPR requirments should be applied to 1CC allocation for NC CA case as indicated in WF R4-2202340. In current Release 16 requirements, 1 CC allocation was not specficially seperated. It was agreed to also update Rel-16 requirments with this latest agreement to keep spec consistency and clearnance. Considering this is for 2LO case, UE impletation is not be impacted. There are a number of minor issues need to update, such as: - Hanging paragraphs in 6.2A.2.2; - No clear definition and reference of MPRIM3; - Duplicate definition of LCRB_alloc, 1 and 2; - Other duplicate descriptions in MPR definion logic - Redudant defintion of B in multiple clauses |  |  |  |  |  |  |  |  |
| Summary of change: |  | Revise the MPR applicability to single carrier requirements for 1 CC allocation for 2LO and power class 3. Other revisions: - Adding clause titile 6.2A.2.2.0 to aviod hanging paragraphs. - Revise MPRIM3 To MPR which is more consistent to the rest of the spec; - Revise the LCRB_alloc, 1/2 to LCRB1/2 which is consistent to most of the spec; - Delete the duplicate part of MPR definition logic; - Move definition of B in general section. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 1 CC requirements for 2LO power class 3 would be not aligned with Release 17. A number of other issues would be stayed in Rel-16 spec.. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isoloation Impact analysis: This revision should have no UE implementation impact. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

6.2A.2.2	UE maximum output power reduction for Intra-band non-contiguous CA

6.2A.2.2.0	General

For intra-band non-contiguous CA, the allowed Maximum Power Reduction (MPR) for the maximum output power is specified into 2 types: MPR to meet -30dBm/MHz and -13dBm/MHz. The UE determins the MPR type as follows:

If OR( LCRB1 = 0, LCRB2 = 0 )

MPR defined in Table 6.2.2-1 for PC3

Else If AND( FIM3,low_block,low > SEM-13,low ,  FIM3,high_block,high < SEM-13,high )

MPR defined in Clause 6.2A.2.2.2

Else

MPR defined in Clause 6.2A.2.2.1

where

-	LCRB1 is for CC1 which is the component carrier with lower frequency

-	LCRB2 is for CC2 which is the component carrier with higher frequency

-	B =  (LCRB1* 12* SCS1 + LCRB2 * 12 * SCS2)/1,000,000

-	FIM3,high_block,high = (2 * Fhigh_alloc,high_edge ) – Flow_alloc,low_edge

-	FIM3,low_block,low = (2 * Flow_alloc,low_edge) – Fhigh_alloc,high_edge

-	Flow_alloc,low_edge is the lowermost frequency of the lower transmission bandwidth allocation.

-	Flow_alloc,high_edge is the uppermost frequency of the lower transmission bandwidth allocation.

-	Fhigh_alloc,low_edge is the lowermost frequency of the upper transmission bandwidth allocation.

-	Fhigh_alloc,high_edge is the uppermost frequency of the upper transmission bandwidth allocation.

-	SEM-13,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.2.2.

-	SEM-13,high = Threshold frequency where upper spectral emission mask above the upper channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.2.2.

6.2A.2.2.1	MPR to meet -30dBm/MHz

MPR in this clause is for intra-band non-contiguous CA power class 3 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MAWhere MA is defined as follows

MA = 	15; 	0 ≤ B < 1.08

14.5; 	1.08 ≤ B < 2.16

13.5; 	2.16 ≤ B < 3.24

12.5;       3.24 ≤ B < 5.04

11.5; 	5.04≤ B < 10.08

10.5; 	10.08 ≤ B < 16.38

10;        16.38 ≤ B < 21.78

9; 	      21.78 ≤ B

6.2A.2.2.2	MPR to meet -13dBm/MHz

MPR in this clause is for intra-band non-contiguous CA power class 3 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MA

Where MA is defined as follows

MA = 	9	;	 0 ≤ B < 0.54

8	;	 0.54 ≤ B < 1.08

7	; 	1.08 ≤ B < 2.16

6.5	; 	2.16 ≤ B < 3.24

5.5	; 	3.24 ≤ B < 5.4

4	; 	5.4 ≤ B

<End of Change>