3GPP TSG-RAN WG4 Meeting #102-e		R4-2204990
Electronic Meeting, 21st Feb. – 03rd Mar., 2022

Agenda item:	10.2.3.2

Source:	OPPO

Title:	TP to TS 38.161 on Annex A: Test methodology

Document for:	Approval

1	Introduction

As previously discussed [1], the Annex A: Test methodology of TS 38.161 will capture the normative information. This paper provides the text proposal to TS 38.161 Annex A Test methodology which are captured from TR 38.834.

References

[1] R4-2120727 Summary_326_2nd_round

[2] 3GPP TR 38.834 v0.3.0

Text Proposal

<TP for TS 38.161>

Annex A (normative):
Test methodology

<Editor’s note: normative information of test methods, e.g., test configuration, minimum range length, antenna setting. Detailed structure of the subclause is TBD >

A.1 Test configuration

A.1.1 TRP Test configuration

For Standalone, the NR System Simulator (SS) and DUT shall be configured per TS 38.521-1 [5], section 6.2 (Transmitter Power) using the default settings specified in TS 38.521-1 [5] and TS 38.508-1 [7] as applicable. The measurement should be carried out based on the detailed test parameters for each band, as defined in TR 38.834 Table 4.3-1.

For EN-DC, the SS and DUT shall be configured per TS 38.521-3 [6], Section 6.2B.1 (UE Maximum Output Power for EN-DC) using the default settings specified in TS 38.521-3 [6] and TS 38.508 [7] as applicable. The measurement should be carried out based on the detailed test parameters for each band, as defined in TR 38.834 Table 4.3-3. The UL output power of LTE carrier should be set as a constant power of 10dBm, while measuring NR at maximum output power, i.e., with fixed p-MaxEUTRA-r15=10 dBm, and p-NR-FR1 not configured.

A.1.2 TRS Test configuration

For Standalone, the NR System Simulator (SS) and DUT shall be configured per section 7.3 (Reference Sensitivity) of TS 38.521-1 [5] using the defaults specified in TS 38.521-1 [5] and TS 38.508-1 [7] as applicable. The measurement should be carried out based on the detailed test parameters for each band, as defined in TR 38.834 Table 4.3-2.

For EN-DC, the SS and DUT shall be configured per TS 38.521-3 [6], Section 6.2B.1 (UE Maximum Output Power for EN-DC) using the default settings specified in TS 38.521-3 [6] and TS 38.508 [7] as applicable. The measurement should be carried out based on the detailed test parameters for each band, as defined in TR 38.834 Table 4.3-3. The UL output power of LTE carrier should be set as a constant power of 10dBm, while measuring NR at maximum output power, i.e., with fixed p-MaxEUTRA-r15=10 dBm, and p-NR-FR1 not configured.

<End of TP >