3GPP TSG-RAN4 Meeting #102-e	R4-2205044

# Electronic meeting, February 21 – March 3, 2022

Source:		Ericsson

Title:			NTN – Additional simulation results

Agenda item:		10.13.2.1

Document for:		Discussion

# Introduction

The NTN WI ([1]) has been approved in RAN#88e meeting to specify requirements for the support of NTN. It has been revised in last RAN#94-e meeting [2].

In last RAN4 meeting, the simulation effort was almost concluded, excepting case 6, and outcomes have been captured in the TR 38.863. But no company provided any result for NB-IoT coexistence, which would be needed to complete this coexistence study.

In this contribution, we are sharing our simulation results for some of the scenarios, focusing on the ones which have been identified as the most stringent for NTN-TN NR coexistence.

# Discussion

## Case 1 – DL-DL - TN Urban macro – Victim: NTN GEO

Figure 1: Throughput Loss - Aggressor: NTN GEO – Victim: TN Urban macro – isolation area

| Required ACIR [dB] |  | 31 | 32 | 33 | 34 | 35 |
| --- | --- | --- | --- | --- | --- | --- |
| Throughput loss | Mean | 3.0 | 2.6 | 2.2 | 1.9 | 1.6 |
|  | 5%-tile | 7.5 | 6.6 | 5.7 | 5.0 | 4.5 |

## Case 2 – UL-UL - Aggressor: TN Urban macro – Victim: NTN GEO

Figure 2: Throughput Loss - Aggressor: TN Urban macro – Victim: NTN GEO

| Required ACIR [dB] |  | 27 | 28 | 29 | 30 | 31 | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput loss | Mean | 10.9 | 9.3 | 7.9 | 6.7 | 5.5 | 4.4 |
|  | 5%-tile | NA | NA | NA | NA | NA | NA |

## Case 3 – DL-DL - Aggressor: NTN LEO600 – Victim: TN Rural

TBD.

Note that rural scenario was not considered for NB-IoT study.

Some assumptions have not been discussed in this meeting.

## Case 4 – UL-UL - NTN GEO – Victim: TN Urban macro

Figure 3: SINR Loss - Aggressor: NTN GEO – Victim: TN Urban macro

| Required ACIR [dB] |  | 18 | 19 | 20 | 21 | 22 |
| --- | --- | --- | --- | --- | --- | --- |
| SINR loss | Mean | 1.0 | 0.9 | 0.8 | 0.7 | 0.6 |
|  | 5%-tile | 1.3 | 1.2 | 1.0 | 0.6 | 0.6 |

## Results Summary

|  |  | Case | TN = NB-IoT |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Aggressor | Victim |  | ACIR |
| DL-DL | Urban | 1 | TN NB-IoT | NTN GEO | Isol 1500m | 34 |
| UL-UL | Urban | 2 | TN NB-IoT | NTN GEO |  | 31.5 |
| DL-DL | Rural | 3 | NTN LEO600 | TN NB-IoT |  | TBD |
| UL-UL | Urban | 4 | NTN GEO | TN NB-IoT |  | 20 |

## ACLR and ACS needed for coexistence with NB-IoT

Based on our previous initial results, the needed NTN SAN/UE ACLR/ACS would be according to the following table.

|  | ACLR | ACS |
| --- | --- | --- |
| SAN | TBD | 37 |
| NTN UE | 23 | 31 |

Those values are based on our initial simulation results only and don’t consider cases 5 and 6. Also, we haven’t had time to simulate case 3 scenario, this would have to be done as well but some assumptions have first to be agreed.

It looks like that:

- The agreed NTN UE ACLR (30dB) would be sufficient.

- SAN and NTN UE ACS would also be sufficient, but there is no much margin. Additional simulations would be needed to confirm any conclusion.

Even if very late, we would then encourage companies to further investigate coexistence with NB-IoT.

# Conclusion

In this contribution, we have shared our preliminary results for coexistence with NB-IoT considering only the worst case scenarios identified during the NTN-TN coexistence study.

# References

- RP-201256, Solutions for NR to support non-terrestrial networks (NTN), Thales

- RP-213691, Solutions for NR to support non-terrestrial networks (NTN), Thales