3GPP TSG-RAN4 Meeting #102-e	R4-2205046

# Electronic meeting, February 21 – March 3, 2022

Source:		Ericsson

Title:			NTN: Satellite Access node RF Tx conducted

Agenda item:		10.13.3.1

Document for:		Approval

# Introduction

The NTN WI ([1]) has been approved in RAN#88e meeting to specify requirements for the support of NTN. It has been revised in last RAN#94-e meeting [2].

In last RAN4 meeting, a Way Forward related to BS RF requirements ([3]) was agreed. This contribution is further discussing Tx spurious and band specific requirements for satellite access node.

# Discussion

## Tx spurious

It was agreed in last RAN4#101-e meeting to specify satellite access node spurious emission based on ERC 74-01 for space stations, i.e. ITU-R Radio Regulations Annex 3.

Adapted to RAN4 type of requirement, the spurious requirement should then be as proposed below and capture in [4].

Proposal1: Following Tx spurious limits shall be considered for NTN satellite access node:

| Spurious frequency range | Basic limit | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 9 kHz – 150 kHz |  | 1 kHz | Note 1 |
| 150 kHz – 30 MHz |  | 10 kHz | Note 1 |
| 30 MHz – 1 GHz |  | 100 kHz | Note 1 |
| 1 GHz 12.75 GHz | -13dBm | 1 MHz | Note 1, Note 2 |
| 12.75 GHz – 5th harmonic of the upper frequency edge of the DL operating band in GHz |  | 1 MHz | Note 1, Note 2, Note 3 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [2], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [2], s2.5 table 1. NOTE 3: This spurious frequency range applies only for operating bands for which the 5th harmonic of the upper frequency edge of the DL operating band is reaching beyond 12.75 GHz. |  |  |  |

## Additional requirements

In TS 38.104, additional operating band unwanted emission requirement specific to band n24 has been specified. Similar requirement should then be specified for n255.

Proposal2 : Specify additional operating band unwanted emissions limits for Band n24 (subclause 6.4.2.5.6 in TS 38.104) for satellite band n255 as well.

# Conclusion

In this contribution, we further discussed the satellite access node RF Tx requirements. We made the following proposal:

Proposal1: Following Tx spurious limits shall be considered for NTN satellite access node:

| Spurious frequency range | Basic limit | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 9 kHz – 150 kHz |  | 1 kHz | Note 1 |
| 150 kHz – 30 MHz |  | 10 kHz | Note 1 |
| 30 MHz – 1 GHz |  | 100 kHz | Note 1 |
| 1 GHz 12.75 GHz | -13dBm | 1 MHz | Note 1, Note 2 |
| 12.75 GHz – 5th harmonic of the upper frequency edge of the DL operating band in GHz |  | 1 MHz | Note 1, Note 2, Note 3 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [2], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [2], s2.5 table 1. NOTE 3: This spurious frequency range applies only for operating bands for which the 5th harmonic of the upper frequency edge of the DL operating band is reaching beyond 12.75 GHz. |  |  |  |

Proposal2 : Specify additional operating band unwanted emissions limits for Band n24 (subclause .6.4.2.5.6 in TS 38.104) for satellite band n255 as well.

# References

- RP-201256, Solutions for NR to support non-terrestrial networks (NTN), Thales

- RP-213691, Solutions for NR to support non-terrestrial networks (NTN), Thales

- R4-2202033, WF on RF requirement for SAN type 1-H, CATT

- R4-2205055, CR to TS 38.108 - Transmitter spurious emissions, Ericsson