3GPP TSG-RAN WG4 Meeting #102-e	R4-2205087

Electronic Meeting, 21 February – 3 March 2022

Agenda Item:	10.6.5.2

Source:	Ericsson

Title:	UE demodulation requirements for DL 1024QAM

Document for:	Discussion

# 1	Introduction

RAN4#101-bis-e agreed with the way forward on the UE demodulation and CQI reporting requirements for DL 1024QAM in FR1 [1]. This contribution discusses the open issues based on the simulation results.

# 2	Discussion

RAN4#101-bis-e agreed the assumed Tx EVM is 2.5% and MCS is 23, but there are still two options for the propagation channel model: TDLA30-10 or TDLD30-5. To decide the propagation channel model, we show our simulation results in Figure 1 according to the simulation assumption [1]. Table 1 also summarizes the required SNR to achieve 70% of the maximum throughput. Comparing the TDLA30 and TDLD30 regarding the requires SNR to achieve 70% of maximum throughput, the SNR with TDLA30 is about 1.5dB higher than TDLD30. In RAN4#101-bis-e, some companies proposed to consider the extra RF margin due to the very high SNR test point. To avoid the extra RF margin, we propose to use TDLD30-5 to keep the required SNR test points low.

| (a) FDD 2Rx, MCS23 Rank 1, Tx EVM 2.5% | (b) FDD 4Rx MCS23 Rank 1, Tx EVM 2.5% |
| --- | --- |

Figure 1	PDSCH simulation results for DL 1024QAM.

Table 1	SNR to achieve 70% of the maximum throughput.

|  |  | FDD 2Rx | FDD 4Rx | TDD 2Rx | TDD 4Rx |
| --- | --- | --- | --- | --- | --- |
| Tx EVM 2.5% | TDLA30-10 | 27.5 | 23.4 | 28.1 | 23.6 |
|  | TDLD30-5 | 25.9 | 22.7 | 26.1 | 22.9 |

Proposal: Set TDLD30-5 for PDSCH demodulation requirements with 1024QAM.

# 4	Summary

Proposal: Set TDLD30-5 for PDSCH demodulation requirements with 1024QAM.

# References

- R4-2203044, “Way forward for NR DL1024QAM demodulation and CQI reporting requirements”, Ericsson.

# Appendix: MCS table with 1024QAM entries

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 193 | 0.3770 |
| 2 | 2 | 449 | 0.8770 |
| 3 | 4 | 378 | 1.4766 |
| 4 | 4 | 490 | 1.9141 |
| 5 | 4 | 616 | 2.4063 |
| 6 | 6 | 466 | 2.7305 |
| 7 | 6 | 517 | 3.0293 |
| 8 | 6 | 567 | 3.3223 |
| 9 | 6 | 616 | 3.6094 |
| 10 | 6 | 666 | 3.9023 |
| 11 | 6 | 719 | 4.2129 |
| 12 | 6 | 772 | 4.5234 |
| 13 | 6 | 822 | 4.8164 |
| 14 | 6 | 873 | 5.1152 |
| 15 | 8 | 682.5 | 5.3320 |
| 16 | 8 | 711 | 5.5547 |
| 17 | 8 | 754 | 5.8906 |
| 18 | 8 | 797 | 6.2266 |
| 19 | 8 | 841 | 6.5703 |
| 20 | 8 | 885 | 6.9141 |
| 21 | 8 | 916.5 | 7.1602 |
| 22 | 8 | 948 | 7.4063 |
| 23 | 10 | 805.5 | 7.8662 |
| 24 | 10 | 853 | 8.3301 |
| 25 | 10 | 900.5 | 8.7939 |
| 26 | 10 | 948 | 9.2578 |
| 27 | 2 | reserved |  |
| 28 | 4 | reserved |  |
| 29 | 6 | reserved |  |
| 30 | 8 | reserved |  |
| 31 | 10 | reserved |  |