3GPP TSG-RAN WG4 Meeting #102-e	R4-2205092

Electronic Meeting, 21 February – 3 March 2022

Agenda Item:	12.9.6.2.2

Source:	Ericsson

Title:	BS demodulation requirements for Rel-17 NB-IoT

Document for:	Discussion

# 1	Introduction

RAN4#101-bis-e agreed the way forward on BS demodulation requirements for Rel-17 NB-IoT [1], where RAN4 agreed to define NPUSCH format 1 demodulation requirements with 16QAM and single-TB scheduling.

The remaining open issue is the defined number of test cases including allocated tones, number of RUs, and TBS. We show our view on the options.

# 2	Discussion

## 2.1	Number of allocated tones

- Option 1: 3,6,12 tones - Option 2: 12 tones

In Rel-13, RAN4 defined NPUSCH format 1 demodulation requirements with 3, 6, 12 tones with repetition levels 2, 16, and 64 (See TS36.104 Table 8.5.1.1.1-3). Since RAN4 have already defined all the possible multi tone configuration with QPSK, we don’t think RAN4 need to define all the multi-tone configurations to verify 16QAM. For example, when 3GPP introduced multi-TB transmission for NPUSCH format 1 in Rel-15, RAN4 has defined the demodulation requirements with 12 tones only.

Proposal 1: Define NPUSCH format 1 demodulation requirements with 12 tones for 16QAM.

## 2.2	TBS and number of resource units (RUs)

- Option 1: (IRU, ITBS) = (0,15), TBS=280 bits - Option 2: (IRU, ITBS) = (19,3), TBS=1736 bits - Option 3: (IRU, ITBS) = (0,14), TBS=256 bits - Other options are not precluded

Table 1 shows the TBS table agreed in RAN1 to support NPUSCH with 16QAM. According to RAN1 agreements, NPUSCH format 1 with 16QAM is scheduled with 14 ≤ ITBS ≤ 21.

Table 1	TBS table applicable for NPUSCH format 1 with 16QAM

|  | IRU |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ITBS | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | Code rate with 16QAM |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |  |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |  |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |  |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |  |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |  |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |  |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1000 |  |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 | 1000 | 1224 |  |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 1096 | 1384 |  |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1256 | 1544 |  |
| 10 | 144 | 328 | 504 | 680 | 872 | 1000 | 1384 | 1736 |  |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 | 1608 | 2024 |  |
| 12 | 208 | 440 | 680 | 1000 | 1128 | 1352 | 1800 | 2280 |  |
| 13 | 224 | 488 | 744 | 1032 | 1256 | 1544 | 2024 | 2536 |  |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 |  | ~0.5 |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 |  | ~0.54 |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | 2536 |  | ~0.56 |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 |  |  | ~0.62 |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 |  |  | ~0.70 |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2536 |  |  | ~0.75 |
| 20 | 440 | 904 | 1384 | 1864 | 2344 |  |  |  | ~0.80 |
| 21 | 488 | 1000 | 1480 | 1992 | 2536 |  |  |  | ~0.88 |

One concern on option 2 is relatively higher code rate around 0.75, although RAN4 usually sets the target code rate around 0.5 for 16QAM. Since the highest code rate set in RAN4 NPUSCH format 1 demodulation requirements are 0.56 which is configured in A16-5 with (IRU, ITBS) = (0, 9), our preference is to set (IRU, ITBS) = (0, 15) so that the target code rate is close to A16-5. However we are also fine with setting (IRU, ITBS) = (0, 14).

Proposal 2: Set (IRU, ITBS) = (0, 15) and TBS=280 bits for NPUSCH format 1 demodulation requirements with 16QAM.

## 2.3	Simulation assumption

We propose to define NPUSCH format 1 demodulation requirements with 16QAM with the following setup.

- Number of allocated subcarriers: 12

- TBS: 280bits / IRU: 0 (ITBS =15, target CR=0.53, according to Table 1)

Proposal 3: RAN4 assumes the simulation assumption below for NPUSCH format 1 with 16QAM.

| Test number | Subcarrier spacing | Number of allocated subcarriers | Propagation condition | TBS / ITBS | IRU | Repetition number | Antenna configuration |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15kHz | 12 | ETU 1Hz low | 280 bits / 15 | 0 | 1 | 1x2 |

# 3	Summary

Proposal 1: Define NPUSCH format 1 demodulation requirements with 12 tones for 16QAM.

Proposal 2: Set (IRU, ITBS) = (0, 15) and TBS=280 bits for NPUSCH format 1 demodulation requirements with 16QAM.

Proposal 3: RAN4 assumes the simulation assumption below for NPUSCH format 1 with 16QAM.

| Test number | Subcarrier spacing | Number of allocated subcarriers | Propagation condition | TBS / ITBS | IRU | Repetition number | Antenna configuration |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15kHz | 12 | ETU 1Hz low | 280 bits / 15 | 0 | 1 | 1x2 |

# References

- R4-2203041, “Way forward for performance requirements of Rel-17 NB-IOT and eMTC”, Huawei, HiSilicon.