3GPP TSG-RAN WG4 Meeting #102-e	R4-2205095

Electronic Meeting, 21 February – 3 March 2022

Agenda Item:	10.20.4.2.1

Source:	Ericsson

Title:	PDSCH demodulation requirements for RedCap

Document for:	Discussion

# 1	Introduction

RAN4#101-bis-e agreed on RedCap demodulation and CQI reporting requirements [1]. This contribution discusses the open issues on PDSCH demodulation and SDR requirements for RedCap.

# 2	PDSCH demodulation

RAN4 agreed to define PDSCH demodulation requirements for RedCap 1Rx UE and 2Rx UE. We understand the motivation to minimize the work effort for defining the performance requirements. However we also need to ensure the test coverage including MCS, rank (for 2Rx UE), fading channel model (low/high delay spread and high/low Doppler spread) and TDD pattern. We also think it is important to apply the same test configuration regardless of FDD/TDD and FR1/FR2. Therefore we propose to define PDSCH with the following configuration for 1Rx/2Rx UEs. We are also fine to define PDSCH with 256QAM in FR1 with optional capability.

For FR2, as we discussed in our companion paper [2], RAN4 RF session agreed to exclude 1Rx from FR2, but it is FFS whether the supporting 2 MIMO layers is mandatory or not. We therefore propose to define the requirements with 2Rx only for FR2.

Proposal 1: Define the following PDSCH demodulation requirements for RedCap UE.

FR1 FDD

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 Table | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB400-400 | 2x1 low | 70% of peak rate | 5.2.2.1.1-3 Test 1-1 | New |
| 1Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x1 low | 70% of peak rate | 5.2.2.1.1-3 Test 1-4 | New |
| 1Rx UE | 10MHz / 15kHz | 64QAM 0.50 Rank 1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.1.1-4 Test 2-1 | New |
| 1Rx UE (Optional with UE capability) | 10 MHz / 15kHz | 256QAM 0.82 Rank 1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.1.1-3 Test 1-3 | New |
| 2Rx UE | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB400-400 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-3 Test 1-1 | Reuse |
| 2Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-3 Test 1-4 | New |
| 2Rx UE | 10MHz / 15kHz | 64QAM 0.5 Rank 2 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-4 Test 2-1 | Reuse |
| 2Rx UE (Optional with UE capability) | 10 MHz / 15kHz | 256QAM 0.82 Rank 1 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-3 Test 1-3 | Reuse |

FR1 TDD

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 Table | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20MHz / 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB400-400 | 2x1 low | 70% of peak rate | 5.2.2.2.1-3 Test 1-1 | New |
| 1Rx UE | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x1 low | 70% of peak rate | 5.2.2.2.1-3 Test 1-4 | New |
| 1Rx UE | 20MHz / 30kHz | 64QAM 0.50 Rank 1 | FR1.30-1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.2.1-4 Test 2-1 | New |
| 1Rx UE (Optional with UE capability) | 20MHz / 30kHz | 256QAM 0.82 Rank 1 | FR1.30-1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.2.1-3 Test 1-3 | New |
| 2Rx UE | 20MHz / 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB400-400 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-3 Test 1-1 | New |
| 2Rx UE | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-3 Test 1-4 | New |
| 2Rx UE | 20MHz / 30kHz | 64QAM 0.50 Rank 2 | FR1.30-1 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-4 Test 2-1 | New |
| 2Rx UE (Optional with UE capability) | 20MHz / 30kHz | 256QAM 0.82 Rank 1 | FR1.30-1 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-3 Test 1-3 | New |

FR2 TDD

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 100MHz / 120kHz | QPSK 1/3 Rank 1 | FR2.120-1A | TDLC60-300 | 2x1 low | 70% of peak rate | 7.2.2.2.1-3 Test 1-1 | New |
| 1Rx UE | 100MHz / 120kHz | 16QAM 0.48 Rank 1 | FR2.120-1 | TDLA30-300 | 2x1 low | 70% of peak rate | 7.2.2.2.1-4 Test 2-2 | New |
| 1Rx UE | 100MHz / 120kHz | 64QAM 0.43 Rank 1 | FR2.120-2 | TDLA30-75 | 2x1 low | 70% of peak rate | 7.2.2.2.1-4 Test 2-6 | New |
| 2Rx UE | 100MHz / 120kHz | QPSK 1/3 Rank 1 | FR2.120-1A | TDLC60-300 | 2x2 ULA low | 70% of peak rate | 7.2.2.2.1-3 Test 1-1 | Reuse |
| 2Rx UE | 100MHz / 120kHz | 16QAM 0.48 Rank 2 | FR2.120-1 | TDLA30-300 | 2x2 ULA low | 70% of peak rate | 7.2.2.2.1-4 Test 2-2 | Reuse |
| 2Rx UE | 100MHz / 120kHz | 64QAM 0.46 Rank 2 | FR2.120-2 | TDLA30-75 | 2x2 ULA low | 70% of peak rate | 7.2.2.2.1-4 Test 2-6 | Reuse |

# 3	SDR requirements

- Option 1: Existing SDR test methodology can be reused for RedCap 1Rx/2Rx UE. - Discuss whether to update MCS configuration and common test parameters for 1 Rx RedCap UE testing - Option 2: Not introduce SDR tests for RedCap 1Rx/2Rx UE

The purpose of the SDR requirements is to verify that the Layer 1 and Layer 2 correctly process in a sustained manner the received packets corresponding to the maximum data rate indicated by UE capabilities. Although RedCap targets low complexity/cost UE implementation, it also requires DL peak rate of 150Mbps according to WID [3]. Even for FR1, RedCap UE can support the maximum channel bandwidth of 20MHz with 64QAM rank 1. This is the same capability as Rel-14 LTE Cat-1bis device, and RAN4 defined SDR test for Cat-1bis as shown in TS36.101 Table 8.7.1-4.

We therefore propose to define SDR test for RedCap 1Rx/2Rx UEs. As we discussed in RAN4#101-bis-e, RAN4 need to update the common test parameters in TS 38.101-4 Table 5.5A-1 for FR1 and Table 7.5A.1-1 for FR2, as shown in Table 1 and Table 2.

Proposal 2: Define SDR test for RedCap UE assuming 1Rx for FR1/FR2 and 2Rx for FR1.

Table 1	Possible update of TS 38.101-4 Table 5.5A-1 for RedCap in FR1

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Antenna configuration for non RedCap UE | 1 layer CCs |  | 1x2 or 1x4 |
|  | 2 layers CCs |  | 2x2 or 2x4 |
|  | 4 layers CCs |  | 4x4 |
| Antenna configuration for RedCap UE | 1 layer |  | 1x1 or 1x2 |
|  | 2 layers |  | 2x2 |
| Physical signals, channels mapping and precoding |  |  | As specified in Annex B.4.1 |

Table 2	Possible update of TS 38.101-4 Table 7.5A.1-1 for RedCap in FR2.

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Antenna configuration for non RedCap UE | 1 layer CCs |  | 1x2 or 1x4 |
|  | 2 layers CCs |  | 2x2 or 2x4 |
| Antenna configuration for RedCap UE | 1 layer |  | 1x2 |
|  | 2 layers |  | 2x2 |
| Physical signals, channels mapping and precoding |  |  | As specified in Annex B.4.1 |

# 4	Summary

Proposal 1: Define the following PDSCH demodulation requirements for RedCap UE.

FR1 FDD

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 Table | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB400-400 | 2x1 low | 70% of peak rate | 5.2.2.1.1-3 Test 1-1 | New |
| 1Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x1 low | 70% of peak rate | 5.2.2.1.1-3 Test 1-4 | New |
| 1Rx UE | 10MHz / 15kHz | 64QAM 0.50 Rank 1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.1.1-4 Test 2-1 | New |
| 1Rx UE (Optional with UE capability) | 10 MHz / 15kHz | 256QAM 0.82 Rank 1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.1.1-3 Test 1-3 | New |
| 2Rx UE | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB400-400 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-3 Test 1-1 | Reuse |
| 2Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-3 Test 1-4 | New |
| 2Rx UE | 10MHz / 15kHz | 64QAM 0.5 Rank 2 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-4 Test 2-1 | Reuse |
| 2Rx UE (Optional with UE capability) | 10 MHz / 15kHz | 256QAM 0.82 Rank 1 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-3 Test 1-3 | Reuse |

FR1 TDD

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 Table | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20MHz / 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB400-400 | 2x1 low | 70% of peak rate | 5.2.2.2.1-3 Test 1-1 | New |
| 1Rx UE | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x1 low | 70% of peak rate | 5.2.2.2.1-3 Test 1-4 | New |
| 1Rx UE | 20MHz / 30kHz | 64QAM 0.50 Rank 1 | FR1.30-1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.2.1-4 Test 2-1 | New |
| 1Rx UE (Optional with UE capability) | 20MHz / 30kHz | 256QAM 0.82 Rank 1 | FR1.30-1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.2.1-3 Test 1-3 | New |
| 2Rx UE | 20MHz / 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB400-400 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-3 Test 1-1 | New |
| 2Rx UE | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-3 Test 1-4 | New |
| 2Rx UE | 20MHz / 30kHz | 64QAM 0.50 Rank 2 | FR1.30-1 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-4 Test 2-1 | New |
| 2Rx UE (Optional with UE capability) | 20MHz / 30kHz | 256QAM 0.82 Rank 1 | FR1.30-1 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-3 Test 1-3 | New |

FR2 TDD

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 100MHz / 120kHz | QPSK 1/3 Rank 1 | FR2.120-1A | TDLC60-300 | 2x1 low | 70% of peak rate | 7.2.2.2.1-3 Test 1-1 | New |
| 1Rx UE | 100MHz / 120kHz | 16QAM 0.48 Rank 1 | FR2.120-1 | TDLA30-300 | 2x1 low | 70% of peak rate | 7.2.2.2.1-4 Test 2-2 | New |
| 1Rx UE | 100MHz / 120kHz | 64QAM 0.43 Rank 1 | FR2.120-2 | TDLA30-75 | 2x1 low | 70% of peak rate | 7.2.2.2.1-4 Test 2-6 | New |
| 2Rx UE | 100MHz / 120kHz | QPSK 1/3 Rank 1 | FR2.120-1A | TDLC60-300 | 2x2 ULA low | 70% of peak rate | 7.2.2.2.1-3 Test 1-1 | Reuse |
| 2Rx UE | 100MHz / 120kHz | 16QAM 0.48 Rank 2 | FR2.120-1 | TDLA30-300 | 2x2 ULA low | 70% of peak rate | 7.2.2.2.1-4 Test 2-2 | Reuse |
| 2Rx UE | 100MHz / 120kHz | 64QAM 0.46 Rank 2 | FR2.120-2 | TDLA30-75 | 2x2 ULA low | 70% of peak rate | 7.2.2.2.1-4 Test 2-6 | Reuse |

Proposal 2: Define SDR test for RedCap UE assuming 1Rx for FR1/FR2 and 2Rx for FR1.

# References

- R4-2203032 “WF on RedCap demodulation and CQI reporting requirements”, Ericsson.

- R4-2205094, “General issues on UE demodulation and CSI requirements for RedCap”, Ericsson.

- RP-211574, “Revised WID on support of reduced capability NR devices”, Ericsson.