3GPP TSG-RAN WG4 Meeting #102-e	R4-2205139

# Electronic meeting, Feb 21st – March 3rd , 2022

Source:		UIC, Ericsson, Nokia

Title:			TP 1900MHz RMR band – BS RF

Agenda item:		9.5.3

Document for:	Approval

# Introduction

The 1900MHz NR band for European RMR WI has been approved (revisions in RAN#92e meeting) to enable the use of RMR 1900MHz band with 3GPP 5G NR technology.

Following the discussion in RAN4#101-e/RAN4#101-bis-e meeting, this contribution is addressing the subject of maximum gain, encompassing the consideration of feeder loss, and further coupling losses impacting the BS RF conducted requirements applicable for the use of RMR1900 spectrum. The scope of references will be enlarged by ECC Report 318 [2] and CEPT Report 76 [3]. Both reports are used as the basis for ECC Decision (20)02 [1].

Infrastructure managers considers to re-use present site infrastructure used for GSM-R. To not consider feeder and other internal losses will impact ISD and can significantly increase the amount of investments especially in large countries. Such investments are then made by taxpayers.

Hence, certain flexibility need to be envisaged for potential deployment scenarios for railway lines. The use of fixed antenna gain may cause confusions using such wording.

# Discussion

Infrastructure managers in Europe will consider various antenna types depending on the deployment scenario and corresponding losses. For the examination of e.g. ECC Decision OBUE E.I.R.P limits and the use of  BS Category B applicable to Europe, a maximum gain needs to be considered rather than a fixed antenna gain.

The antenna gain of 18dBi corresponds to ECC Report 318 table 5 but internal losses in ECC Report 318 are considered and agreed as 4dB.

The resulting maximum gain applicable for RMR1900 yields to 14 dB.

Observation 1: For BS operating in n101, the BS rated output power Prated,c,AC  should not exceed (65 – GmaxRMR1900) dBm / 10 MHz, (assuming certain antenna gain and losses).

Observation 2: The maximum gain may vary depending on the deployment scenario e.g. flat surface, hilly terrain or tunnel. In this context, the assumption of the maximum gain requires further clarification.

# Conclusion

In this contribution, the context of maximum gain based on ECC Report 318 are further studied which will require revision of the corresponding clauses in 3GPP TR 38.852.

The following proposals are made :

Proposal 1: To allow the necessary flexibility for deployment along railway lines it is proposed to use the term of “maximum gain” (defined as antenna gain and losses) instead of “antenna gain”.

Proposal 2: Accordingly the approach in TR 38.853 need to be revised.

Proposal 3: Capture the revisions in CRs applicable for band n101 in 3GPP TS 38.104.

# References

- ECC Decision(20)02,  Harmonised use of the paired frequency bands 874.4-880.0 MHz and 919.4-925.0 MHz and of the unpaired frequency band 1900-1910 MHz for Railway Mobile Radio (RMR), 20 November 2020

- ECC Report 318, Compatibility between RMR and MFCN in the 900 MHz range, the 1900-1920 MHz band and the 2290-2300 MHz band

- CEPT Report 76, Report from CEPT to the European Commission in response to the Mandate on spectrum for the future railway mobile communications system, Report B: EU-harmonised technical conditions for the future railway mobile radio communications system (Task 5)

# Annex TP for changing maximum gain

#### ***********************************************1st Change******************************************

### 7.1.1	General

According to ECC Decision (20)02 [1], the BS RF work for RMR1900 is limited to the Wide Area Base Stations,  determined by the use along railway lines and their corresponding characteristics defined by 3GPP TS 38.104 [4].

In accordance to ECC Decision 20(02) [1], the related studies were done for the non-AAS BS architectures,  defined in 3GPP TS 38.104 [4]. Therefore the RAN4 requirements derivation was limited to the BS type 1-C requirements, only.

As the EIRP limits were defined in the ECC Decision (20)02 [1], consideration of non-AAS BS architecture required to convert those EIRP limits into the conducted requirements assuming a maximum gain that can vary depending on the used antenna and related antenna gain as well as the potential losses, e.g., feeder. In the present consideration an antenna gain of 18 dBi and 0 dB losses are assumed resulting to a maximum gain value of GmaxRMR1900 = 18 dBi. Other assumptions may lead to other maximum gain values and conducted requirements.

#### ***********************************************2nd Change******************************************

#### 7.1.2.1	BS maximum output power

Based on ECC Decision (20)02 [1], the BS maximum output power for BS operating in band n101 applicable for [uncoordinated deployment], should not exceed the Prated,c,AC derived based on table 4.3.1-1 EIRP limits, with the assumption of a maximum value of the BS GmaxRMR1900 as:

(65 – GmaxRMR1900) dBm / 10 MHz = 47 dBm / 10 MHz,

where GmaxRMR1900 is the maximum gain (antenna gain and losses) of the RMR1900 BS consideration.

#### ***********************************************3rd Change******************************************

##### 7.1.2.2.2	Tx spurious emissions

Additional Tx spurious emission requirements are considered for band n100 in 3GPP TS 38.104 [4], based on conversion of the EIRP limits from ECC Decision (20)02 [1], as summarized in table 7.1.2.2.2-1. The requirement is derived based on table 4.3.1-5, with the assumption of maximum gain value of the BS  GmaxRMR1900:

- Table 7.1.2.2.2-1: Additional Tx spurious emissions requirement derivation for n101

| - Spurious frequency range | - Basic limit | - Measurement bandwidth |
| --- | --- | --- |
| 1920 MHz – 1980 MHz | - 43 - GmaxRMR1900 = -61 dBm (NOTE) | 5 MHz |
| NOTE: Assuming a 18dB maximum gain(antenna gain and losses) for GtmaxRMR1900. |  |  |

#### *********************************************End of Changes******************************************