3GPP TSG-RAN WG4 #102-e	R4-2205315

e-Meeting, Feb 21 – Mar 3, 2022

Agenda item:	10.16.2

Source: 	Qualcomm Incorporated

Title: 	60GHz system parameters

Document for:	Approval

# Mandatory channel bandwidths

In RAN4 allowable bandwidths for each SCS have been agreed, however an open question is which CCBWs are mandatory, and which are optional.

As described in the WID, one of the goals is to leverage FR2-1 implementations to enable adoption and rollout of 60 GHz.

In FR2-1 CCBWs up to 200 MHz are the maximum mandated CCBWs. 400 MHz and above are optional. FR2-2 UEs for this new 60 GHz band will be based heavily on FR2-1 implementations. Practical UE analog signal paths have limitations in the bandwidths they can support. To enable UE implementations, we propose to require 400 MHz as the mandatory CCBW for all 480 and 960 SCS.

Proposal: 120 kHz SCS 100 MHz(mandatory), 400 MHz (optional)

Proposal: 480 kHz SCS: 400 MHz(mandatory): other CCBWs optional

Proposal: 960 kHz SCS: 400 MHz(mandatory): other CCBWs optional

# Conclusions

Proposal: 120 kHz SCS 100 MHz(mandatory), 400 MHz (optional)

Proposal: 480 kHz SCS: 400 MHz(mandatory): other CCBWs optional

Proposal: 960 kHz SCS: 400 MHz(mandatory): other CCBWs optional