3GPP TSG-RAN WG4 Meeting #102-e	R4-2205328

# Online, Feb 21 – Mar 3, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on applicability rules for FR2 inter-band CA with CBM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh2-Core |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For CBM based FR2 inter-band CA, the applicability rules needs to be introduced for the requirements in RRC connnected state. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To introduce the applicability rules for CBM based FR2 inter-band CA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There may have misunderstanding on the requirements applied in CBM based FR2 inter-band CA operation. . |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.6.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of change 1>

### 3.6.11	Applicability for FR2 inter-band CA with CBM

For the requirements in RRC connected state specified in this version of the specification, UE capable of common beam management on a FR2 band pair can assume that the transmitted signals from all the serving cells in this FR2 band pair should have the same QCL assumption on one OFDM symbol.

When inter-band carrier aggregation in FR2 is performed, the UE capable of common beam management is required to perform RLM/BFD/CBD only on SpCell and to perform L1-RSRP/L1-SINR measurements only on the serving cell(s) in the same band as SpCell.

### <End of change 1>