3GPP TSG-RAN WG4 Meeting #102-e	R4-2205359

Electronic Meeting, 21 February – 3 March, 2022

Title: 	Discussion on remaining issue in CSI-RS core requirements

Source: 	Huawei, HiSilicon

Agenda item:	5.1.4

Document for:	Discussion

Introduction

In RAN4#101-e, the following issue in RRM core requirements for CSI-RS measurement was discussed without agreement:

- UE behavior when the timing offset exceeds the threshold with single FFT assumption

In this paper we will provide our views on this remaining issue for CSI-RS measurement requirements.

Discussion

- Option 1: - UE is not required to report CSI-RS based L3 measurements. If UE reports CSI-RS based L3 measurement, then the UE shall meet CSI-RS based L3 measurement reporting requirements in TS 38.133 section 9.10.2.4 and 9.10.3.4 based on the accuracy requirements for the case when the timing offset is below the threshold - Do not define test cases to verify UE behavior for the case when the timing offset is above the threshold - Option 2: - UE is not required to report CSI-RS based L3 measurements. If UE reports CSI-RS based L3 measurement, then the UE may not meet CSI-RS based L3 measurement reporting requirements in TS 38.133 section 9.10.2.4 and 9.10.3.4 based on the accuracy requirements for the case when the timing offset is below the threshold - Do not define test cases to verify UE behavior for the case when the timing offset is above the threshold - Option 3: - UE is not required to report CSI-RS based L3 measurements. - Option 4: - When the timing offset exceeds the threshold X - X = CP + Y, where Y is the extra margin. Y is FFS - UE is not required to report CSI-RS based L3 measurements. - If UE reports CSI-RS based L3 measurement, then the UE shall meet CSI-RS based L3 measurement reporting requirements in TS 38.133 section 9.10.2.4 and 9.10.3.4 based on the accuracy requirements for the case when the timing offset is below the threshold - Do not define test cases to verify UE behavior for the case when the timing offset is above the threshold

Among the 4 options discussed in RAN4#101-e, we support option 2 or option 3.

In our view, the timing offset smaller than a threshold is a side condition which is similar to other side conditions like Es/Iot larger than -6dB. For the other side conditions, RAN4 has not defined such requirements that the reported measurement results shall meet the accuracy requirements even when the side condition is not met, and we see no reason to mandate such requirement as in option 1 or 2.

Technically, option 1 or option 4 require UE to determine whether a certain side condition is met or not. This is however not always feasible, especially for UE to do it accurately. They would lead to conservative UE implementation that would only measure cells that meets the side condition, because if it tries to measure a cell that does not meet the side condition, it would risk non-compliance with the requirements. We think this is undesirable, especially considering that the timing offset condition is stringent from NW perspective.

Therefore, we suggest to leave it to UE implementation whether to measure and report a neighbor cell whose time offset is larger than the threshold, and apply no accuracy requirement for such cells even UE measures and reports for them. This is the same way how UE handles the measurement and reporting for cells that do not meet the Es/Iot side condition, and it is noted that we have not seen any issue from the field since LTE.

Proposal: If timing offset exceeds the single FFT threshold, UE is not required to report CSI-RS based L3 measurements.

- If UE reports CSI-RS based L3 measurement, then the UE may not meet CSI-RS based L3 measurement reporting requirements in TS 38.133 section 9.10.2.4 and 9.10.3.4 based on the accuracy requirements for the case when the timing offset is below the threshold with single FFT assumption.

Conclusions

In this paper we provided our views on the remaining issue for CSI-RS measurement requirements.

Proposal: If timing offset exceeds the single FFT threshold, UE is not required to report CSI-RS based L3 measurements.

- If UE reports CSI-RS based L3 measurement, then the UE may not meet CSI-RS based L3 measurement reporting requirements in TS 38.133 section 9.10.2.4 and 9.10.3.4 based on the accuracy requirements for the case when the timing offset is below the threshold with single FFT assumption.

Reference