3GPP TSG-RAN WG4 Meeting #102-e	R4-2205701

Electronic Meeting, 21 February – 03 March 2022

Source:	Ericsson, Bell Mobility

Title:	TP for TR 37.717-11-11: Including band combinations DC_2_n25

Agenda item:			9.6.1

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.717-11-11 to add DC_2A_n25A.

# 2. Text Proposal

---Start of changes---

## 5.1.x	DC_2_n25

### 5.1.x.1	Operating bands for EN-DC

Table 5.1.X.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2_n25 | 2 | n25 | Yes |

### 5.1.x.2	Configuration for DC

Table 5.1.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A_n25A10, 11, 13 | n25A | 2A | n25A |
| NOTE 10: Void. The combination is not used alone as fallback mode of other band combinations in which UL in Band 2 is not used. NOTE 11: The minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 13: The minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec. The requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. |  |  |  |

### 5.1.x.3	Maximum output power for DC

Table 6.4.6.3-1: Maximum output power for Inter-band EN-DC

| DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_2A_n25A2 |  |  | 23 | +2/-3 |
| NOTE 2: Only single switched UL is supported |  |  |  |  |

### 5.1.x.4	Spurious emission band UE co-existence for DC

Only Single Tx Switched UL mode is supported for this combination, therefore no spurious emission band UE co-existence requirement is needed

### 5.1.x.5	MSD analysis for DC

Since only Single Tx Switched UL mode is supported for this combination, MSD requirement is not needed.

### 5.1.x.6	∆RIBNC values

The delta R IBNC values for EN-DC DC_2_n25 are shown in table below. The values are reused from LTE combination CA_25A-25A as defined in TS 36.101.

Table 5.1.x.6-1: EN-DC with one uplink configuration on NR for reference sensitivity

| DC configuration | Aggregated bandwidth |  | Wgap / (MHz) | UL NR allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
|  | NR | E-UTRA |  |  |  |  |
| DC_2A_n25A | 5MHz | 5MHz | 30.0 < Wgap ≤ 55.0 | 101 | 5.0 | FDD |
|  |  |  | 0.0 < Wgap ≤ 30.0 | 251 | 0 |  |
|  | 5MHz | 10MHz | 25.0 < Wgap ≤ 50.0 | 101 | 4.5 |  |
|  |  |  | 0.0 < Wgap ≤ 25.0 | 251 | 0 |  |
|  | 5MHz | 15MHz | 20.0 < Wgap ≤ 45.0 | 101 | 4.3 |  |
|  |  |  | 0.0 < Wgap ≤ 20.0 | 251 | 0 |  |
|  | 5MHz | 20MHz | 15.0 < Wgap ≤ 40.0 | 101 | 4.1 |  |
|  |  |  | 0.0 < Wgap ≤ 15.0 | 251 | 0 |  |
|  | 10MHz | 5MHz | 15.0 < Wgap ≤ 50.0 | 10 (RBstart = 33) | 5.5 |  |
|  |  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 10MHz | 10MHz | 10.0 < Wgap ≤ 45.0 | 10 (RBstart = 33) | 5.0 |  |
|  |  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 10MHz | 15MHz | 5.0 < Wgap ≤ 40.0 | 10 (RBstart = 33) | 4.5 |  |
|  |  |  | 0.0 < Wgap ≤ 5.0 | 321 | 0 |  |
|  | 10MHz | 20MHz | 0.0 < Wgap ≤ 35.0 | 10 (RBstart = 33) | 4.2 |  |
|  | 15MHz | 5MHz | 10.0 < Wgap ≤ 45.0 | 10 (RBstart = 44) | 7.6 |  |
|  |  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 15MHz | 10MHz | 5.0 < Wgap ≤ 40.0 | 10 (RBstart = 44) | 6.7 |  |
|  |  |  | 0.0 < Wgap ≤ 5.0 | 321 | 0 |  |
|  | 15MHz | 15MHz | 0.0 < Wgap ≤ 35.0 | 10 (RBstart = 44) | 5.6 |  |
|  | 15MHz | 20MHz | 0.0 < Wgap ≤ 30.0 | 10 (RBstart = 44) | 4.8 |  |
|  | 20MHz | 5MHz | 0.0 < Wgap ≤ 40.0 | 12 (RBstart = 62) | 8.0 |  |
|  | 20MHz | 10MHz | 0.0 < Wgap ≤ 35.0 | 12 (RBstart = 62) | 6.7 |  |
|  | 20MHz | 15MHz | 0.0 < Wgap ≤ 30.0 | 12 (RBstart = 62) | 6.1 |  |
|  | 20MHz | 20MHz | 0.0 < Wgap ≤ 25.0 | 12 (RBstart = 62) | 5.7 |  |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. |  |  |  |  |  |  |

### 5.1.x.7	MPR, AMPR requirements

Since only Single Tx Switched UL mode is supported for this combination, no MPR or A-MPR requirement is needed

---End of changes---

# Reference

[1]		RP-202613, Dual Connectivity (DC) of 2 bands LTE inter-band CA  (2DL/1UL) and 1 NR band (1DL/1UL), Huawei, HiSilicon