3GPP TSG-RAN WG4 Meeting #102-e	R4-2205748

# Electronic Meeting, 21st Feb – 3rd Mar, 2022

Title: 	Discussion and simulation results on 1024QAM PDSCH

Source: 	Huawei, HiSilicon

Agenda item:	10.6.5.2

Document for:	Discussion

Background

During RAN4#101bis-e meeting, Way forward [1] for NR DL1024QAM demodulation and CQI reporting requirements was approved. In this contribution, we share our views about the 1024QAM PDSCH demodulation requirements.

Discussion

PDSCH demodulation requirements

The simulation assumption is derived as following Table 2.1-1 and the updated simulation results is shown in Figure 2.1-1 and Table 2.1-2.

Table 2.1-1 Simulation assumption for 1024QAM PDSCH

| Parameters | FDD | TDD |
| --- | --- | --- |
| SCS/CBW | 15kHz and 10MHz | 30kHz and 40MHz |
| TDD UL/DL configuration | N/A | 7D1S2U, with S=6:4:4 Schedule PDSCH in the special slots |
| MCS (MCS index table 4) | MCS23 |  |
| Rank | 1 |  |
| Antenna configuration | 2x2 with ULA Low 2x4 with ULA Low |  |
| Propagation channel | Option 1: TDLD30-5 Option 2: TDLA30-10 |  |
| PDSCH configuration | Type A mapping, Start symbol 2, Duration 12 |  |
| PDSCH DMRS configuration | Type 1, Single symbol, additional DMRS: pos1 |  |
| Tx EVM assumed for simulation | 2.5% |  |
| SSB configuration | Periodicity 20 ms, Allocated in first slot within 20ms |  |
| TRS configuration | 20 ms periodicity, 2 slots, Offset 10 ms |  |
| Number of HARQ processes | 4 for FDD 15kHz, 8 for TDD 30kHz |  |
| Transform precoding | CP-OFDM |  |
| Allocated RBs | Full BWP |  |
| PRB bundling | 2 |  |
| Precoding model | Random Precoding, per slot and per RB bundling size granularity (codebook configuration: Single panel Type 1) |  |
| Receiver type | MMSE-IRC |  |
| Test metric | 70% of maximum throughput |  |

Figure 2.1-1 Ideal simulation results for 1024QAM PDSCH

Table 2.1-2 Ideal simulation results for 1024QAM PDSCH

| SNR@70%maximum throughput |  | TDLA30-10 | TDLD30-5 |
| --- | --- | --- | --- |
| FDD | 2Rx | 26.38 | 25.56 |
|  | 4Rx | 22.18 | 22.05 |
| TDD | 2Rx | 26.19 | 25.46 |
|  | 4Rx | 22.23 | 21.98 |

Propagation channel

For the propagation channel, considering the stationary wireless scenario with sufficiently high DL SINR and better channel conditions (e.g., LOS or LOS-like channels), with no mobility or very low mobility environment, TDLD30-5 is more consistent with the application scenarios. Also TDLD30-5 can provide lower SNR that is more feasible for the test, we propose to use TDLD30-5 as the propagation condition for 1024QAM requirements.

Select TDLD30-5 for 1024QAM PDSCH requirements.

Impairment margin

We have the following formula about the logarithmic SNR and linear EVM:

SNRequivalent=-10log10EVM2+10-SNRextra/10

Where, SNRequivalent is the equivalent SNR at the UE side, and SNRextra is the extra SNR added by channel simulator. The curve about two different SNR can be shown in the following Figure 2.1-3.

Figure 2.1-3 The curve about equivalent SNR and extra SNR

We can see that the equivalent SNR at the UE side is much less than the extra SNR when high extra SNR is added due to EVM impact. For the existing lower MCS point case with lower SNR, companies usually add about 1~3dB margin for impairment. To achieve the same effect under 1024QAM, the SNR to be added for impairment should be greatly increased. The following Figure 2.1-4 show the function about the SNR point for ideal simulation results and the SNR to be added for impairment to achieve different equivalent impairment values.

Figure 2.1-4 The SNR point for ideal simulation results and the SNR to be added for impairment

As per simulation results, for example, by selecting about 22dB for and 26dB as SNR point for ideal simulation results corresponding to the 4Rx and 2Rx requirements respectively in above Table 2.1-2, if companies want to add 2 dB impairment margin, there is negligible (<0.5dB) impact for 4Rx and about 1dB extra impairment for 2Rx. To ensure proper impairment margin, we propose to add extra 1dB margin for 2Rx requirements besides the impairment margin added by companies.

Add extra 1dB margin for 2Rx requirements besides the impairment margin added by companies.

Proposals

In this contribution, we discuss on UE PDSCH demodulation requirements for 1024QAM. Our observations and proposals are:

- Select TDLD30-5 for 1024QAM PDSCH requirements.

Add extra 1dB margin for 2Rx requirements besides the impairment margin added by companies.

Reference

R4-2203044, Way forward for NR DL1024QAM demodulation and CQI reporting requirements, RAN4#101bis-e, Ericsson