3GPP TSG-RAN WG4 Meeting#102-e                	R4-2205874

E-meeting, 21st Feb – 3rd Mar, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on the number of serving carriers for IBM inter-band FR2 UL CA

Agenda item:	10.4.6.2

Document for:	Approval

Introduction

In meeting RAN4#101bis-e, number of UL carriers for FR2 inter-band UL CA for IBM UEs has been discussed and captured in the agreed WF [1]. In this paper we will discuss and provide our view on it.

Discussion

In last RAN4 meeting, the supported number of UL carriers for FR2 inter-band UL CA for IBM UEs was discussed. During the discussions following options were proposed in agreed WF [1] as below:

Issue 2-1-2 number of UL carriers to be supported for FR2 inter-band UL CA for IBM UEs Candidate options: - Option 1: Number of serving carriers capabilities of clause 3.6.2.1 for FR2 inter-band UL CA shall be revised to following: - up to 1 UL (or 2 UL if SUL is configured) in PCell and up to 2 UL (or 3 UL if SUL is configured) in SCell. - SUL may be configured together with one of the UL. - Option 2: The revision is not needed.

In RF side, it was discussed that two NR bands for inter-band UL CA for IBM UEs and there is 1 UL CC per band. It was captured in CR [3] in RF side as below:

- inter-band uplink CA with two NR bands, and each UL band is configured with a single CC.

The supported number of serving carriers for UL defined in current specification 38.133 section 3.6.2 numbers of serving carries is as below:

with 1 UL (or 2 UL if SUL is configured) in PCell and up to 1 UL (or 2 UL if SUL is configured) in SCell.

-	SUL may be configured together with one of the UL

The current specification can fulfil the supported numbers of serving carriers for inter-band FR2 UL CA for IBM UEs defined in RF side.

- Proposal: The supported numbers of serving carriers for inter-band FR2 UL CA for IBM UEs is up to 2 CCs which each UL band is configured with a single CC. Current specification as below can fulfil the requirements.

with 1 UL (or 2 UL if SUL is configured) in PCell and up to 1 UL (or 2 UL if SUL is configured) in SCell.

-	SUL may be configured together with one of the UL

Conclusion

- In this paper, we discussed the supported number of UL carriers for FR2 inter-band UL CA for IBM UEs in RRM specification, and our proposal is as below:

- Proposal: The supported numbers of serving carriers for inter-band FR2 UL CA for IBM UEs is up to 2 CCs which each UL band is configured with a single CC. Current specification as below can fulfil the requirements.

with 1 UL (or 2 UL if SUL is configured) in PCell and up to 1 UL (or 2 UL if SUL is configured) in SCell.

-	SUL may be configured together with one of the UL

References

- RP-213666, Revised WID on further NR RF Enhancements for FR2, Nokia, Nokia Shanghai Bell, RAN#94

- R4-2202581, WF on RRM requirements for FR2 Inter-band DL CA and UL CA, Nokia, Nokia Shanghai Bell, RAN4#101e-bis

- R4-2206057, Draft CR to 38.101-2 FR2+FR2 ULCA, Qualcomm, Nokia, Verizon, Docomo