3GPP TSG-RAN WG4 # 102-e	R4-2205905

Electronic Meeting, February 21 – March 3, 2022

Agenda item:	10.6.5.2

Source:	MediaTek Inc.

Title:	Discussion on the PDSCH requirements for 1024QAM

Document for:	Discussion

- Introduction

In RAN4#101-bis-e, a way forward on DL1024QAM was agreed [1]. In this contribution, we present simulation results based on the agreed simulation assumptions in appendix A.

- Discussion

We provide our simulation results in Table 2 based on the simulation assumptions in Table 1.

Table 1	Simulation parameters for PDSCH demodulation for DL 1024QAM in FR1.

| Parameters | FDD | TDD |
| --- | --- | --- |
| SCS/CBW | 15kHz and 10MHz | 30kHz and 40MHz |
| TDD UL/DL configuration | N/A | 7D1S2U, with S=6:4:4 Schedule PDSCH in the special slots |
| MCS (MCS index table 4) | MCS23 |  |
| Rank | 1 |  |
| Antenna configuration | 2x2 with ULA Low 2x4 with ULA Low |  |
| Propagation channel | Option 1: TDLD30-5 Option 2: TDLA30-10 |  |
| PDSCH configuration | Type A mapping, Start symbol 2, Duration 12 |  |
| PDSCH DMRS configuration | Type 1, Single symbol, additional DMRS: pos1 |  |
| Tx EVM assumed for simulation | 2.5% |  |
| SSB configuration | Periodicity 20 ms, Allocated in first slot within 20ms |  |
| TRS configuration | 20 ms periodicity, 2 slots, Offset 10 ms |  |
| Number of HARQ processes | 4 for FDD 15kHz, 8 for TDD 30kHz |  |
| Transform precoding | CP-OFDM |  |
| Allocated RBs | Full BWP |  |
| PRB bundling | 2 |  |
| Precoding model | Random Precoding, per slot and per RB bundling size granularity (codebook configuration: Single panel Type 1) |  |
| Receiver type | MMSE-IRC |  |
| Test metric | 70% of maximum throughput |  |

Table 1.	Summary for simulation results

| Antenna configuration and propagation channel | SNR for 70% of max T-put |
| --- | --- |
| 2x2 ULA, TDLA 30-10 | 27.5 dB |
| 2x4 ULA, TDLA 30-10 | 23.6 dB |
| 2x2 ULA, TDLD 30-5 | 25.5 dB |
| 2x4 ULA, TDLD 30-5 | 22.7 dB |

In the last meeting, it was agreed to define test cases with EVM 2.5%. There is an outstanding issue about the propagation channel and two candidate options are TDLA 30-10 and TDLD 30-5. We think TDLD (with LOS channel) provides a suitable propagation condition for 1024QAM. Also, compare the simulation results between TDLA and TDLD, TDL-D provide lower SNR which is more feasible for the test. Hence, we prefer to use TDLD 30-5 to define test cases for 1024QAM PDSCH requirements.

Proposal 1: Consider TDLD 30-5 to define PDSCH requirements for 1024QAM.

- Conclusion

In this contribution, we provide simulation results for 1024QAM with the agreed simulation assumption in [1]. The proposals are summarized as below:

Proposal 1: Consider TDLD 30-5 to define PDSCH requirements for 1024QAM.

- Reference

- R4-2203044, “Way forward for NR DL1024QAM demodulation and CQI reporting requirements”, Ericsson