3GPP TSG-RAN WG4 Meeting #102 -e	R4-2205963

Electronic Meeting, February 21 – March 3, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	HST FR2 PUSCH UL TA Impairment Simulation Results

Agenda item:	10.9.4.3.2 PUSCH with UL timing adjustment requirements

Document for:	Discussion

Introduction

In the previous RAN4#101-bis e-meeting [1], the last open issues regarding the HST FR2 PUSCH UL timing testing setup were resolved. It was agreed that the same applicability rules and manufacture declarations with PUSCH will be used.

In this contribution, following the previous agreements achieved, we are providing imapirment simulation results that will be used for the alignment of requirements.

Discussion

We are following the simulation assumptions for UL timing adjustment requirements that were summarized in [3] to parametrize link-level simulations:

Table 1: Simulation assumptions for UL timing adjustment requirements.

| Parameters | Value |
| --- | --- |
| Channel Model | Stationary UE: AWGN, Moving UE: AWGN |
| UE speed | 350km/h |
| Waveform | CP-OFDM |
| CP length | Normal |
| A | AWGN |
| Δω | 1T2R |
| CBW | 50MHz and 200MHz |
| DM-RS type | Type 1 |
| Mapping type | B |
| Length of PUSCH data | 10 |
| RS configuration | 1DMRS+PT-RS(L=1,K=2), 2DMRS+PT-RS(L=1,K=2), 3DMRS+PT-RS(L=1,K=2) |
| Number of DM-RS CDM group(s) without data | 2 |
| Ratio of PUSCH EPRE to DM-RS EPRE | -3dB |
| DM-RS port | {0} |
| DM-RS sequence generation | NID0=0, nSCID =0 for moving UE NID0=1, nSCID =1 for stationary UE |
| PUSCH resource allocation | 50MHz: 0 to 15 RB for moving UE, 16 to 31 for stationary UE (if 100MHz 200MHz: 0 to 65RB for moving UE, 66 to 131 for stationary UE |
| SRS resource allocation | 50MHz CBW (32RBs)~C_SRS =9, B_SRS =0 200MHz CBW (132RBs)~ C_SRS=33, B_SRS=0 |

Additionally, we take into account the agreements from RAN4#100-e [1]:

- RS configuration - Align RS configuration for UL timing adjustment requirement and PUSCH requirement - SRS transmission - The transmission of SRS in UL timing adjustment requirement is optional - MCS: Only with MC16

and adopt the parameters of the channel model from TS 38.104 to 120 kHz SCS as it was agreed at RAN4#101-e [3]:

- On test parameters for timing offset: - Use A=1.25us, Δw= 1.04s-1 corresponding to 120KHz SCS for HST FR2 UL timing adjustment requirements - On Timing different between moving UE and stationary UE: - Apply the timing difference between moving UE and stationary UE as Δτ-(TA-31)x16*8Tc for UL timing adjustment requirement - On SRS transmission configuration: - The SRS transmission location is the last symbol in slot#3 in radio frame with TDD pattern as DDDSU, S=10:2:2

The results of link-level simulations are summarized in Table 2 below.

Table 2: PUSCH UL Timing Adjustment demodulation performance results.

| SCS and BW | MCS | Channel | DM-RS mapping | Length of data | RS configuration | SNR, dB, Ideal | SNR, dB, Impairment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 120kHz, 50MHz | MCS16 | AWGN | B | 10 | DMRS 1+0 | 6.75 | 9.85 |
| 120kHz, 50MHz | MCS16 | AWGN | B | 10 | DMRS 1+1 | 6.46 | 9.56 |
| 120kHz, 50MHz | MCS16 | AWGN | B | 10 | DMRS 1+1+1 | 6.73 | 9.83 |
| 120kHz, 200MHz | MCS16 | AWGN | B | 10 | DMRS 1+0 | 6.60 | 9.7 |
| 120kHz, 200MHz | MCS16 | AWGN | B | 10 | DMRS 1+1 | 6.47 | 9.57 |
| 120kHz, 200MHz | MCS16 | AWGN | B | 10 | DMRS 1+1+1 | 6.47 | 9.57 |

Use the impairment results in the table above for the alignment of PUSCH UL Timing Adjustment demodulation performance requirements.

Conclusion

In the contribution, we are proviing impairment simulation results for the alignemnt. The following proposal is made:

| SCS and BW | MCS | Channel | DM-RS mapping | Length of data | RS configuration | SNR, dB, Ideal | SNR, dB, Impairment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 120kHz, 50MHz | MCS16 | AWGN | B | 10 | DMRS 1+0 | 6.75 | 9.85 |
| 120kHz, 50MHz | MCS16 | AWGN | B | 10 | DMRS 1+1 | 6.46 | 9.56 |
| 120kHz, 50MHz | MCS16 | AWGN | B | 10 | DMRS 1+1+1 | 6.73 | 9.83 |
| 120kHz, 200MHz | MCS16 | AWGN | B | 10 | DMRS 1+0 | 6.60 | 9.7 |
| 120kHz, 200MHz | MCS16 | AWGN | B | 10 | DMRS 1+1 | 6.47 | 9.57 |
| 120kHz, 200MHz | MCS16 | AWGN | B | 10 | DMRS 1+1+1 | 6.47 | 9.57 |

- Use the impairment results in the table above for the alignment of PUSCH UL Timing Adjustment demodulation performance requirements.

- References

- R4-2203006, WF on BS demodulation requirement for FR2 HST, Nokia, RAN4#101-bis-e.

- R4-2115726, WF on FR2 HST demodulation, Samsung, RAN4#100-e.

- R4-2120703, WF on BS demodulation requirement for FR2 HST, Nokia, Nokia Shanghai Bell, Samsung, RAN4#101-e.