3GPP TSG-RAN WG4 Meeting #102-e		R4- 2206004

E-meeting, February 21 – March 03, 2022

Agenda item:	10.13.6.3

Source:	Intel Corporation

Title:	Discussion on UE demodulation requirements for NR NTN

Document for:	Discussion

# Introduction

In this paper we provide our views on the open issues identified during the RAN4 101-bis-e meeting and captured in the WF [1]

# Discussion

Scope of requirements

PDSCH/PDCCH/PBCH requirements

As it was agreed during RAN4 101-bis-e meeting legacy TN UE Rel-15/Rel-16 demodulation requirements shall also apply for NTN UE supporting NTN, so only incremental requirements should be defined. There are no algorithm enhancements for NTN PDSCH/PDCCH/PBCH. The only change is propagation conditions. In order to check correct operation under such propagation conditions it’s enough to have PDSCH tests.

Proposal 1: RAN4 to define new requirements only for PDSCH

Enhancement on HARQ

In real systems there will be no scenarios with totally disabled HARQ since there always are transmissions that require feedback. We don’t need to define tests for the scenario which will never happen in real systems. We prefer to define PDSCH performance requirements with only some portion of HARQ processes disabled. It is ok to consider half HARQ processes disabled during the tests.

Proposal 2: For PDSCH requirements with disabled HARQ processes RAN4 to consider half of HARQ processes disabled and half HARQ processes enabled.

General assumptions

Channel model

NTN channel models for link level simulations are introduced in TR38.811: two TDL models, namely NTN-TDL-A and NTN-TDL-B are constructed to represent two different channel profiles for NLOS, while NTN-TDL-C and NTN-TDL-D are constructed for LOS.

During RAN4 101-bis-e meeting it was agreed to use all four cahannel models as the starting point. In Rel-15/16 the PDSCH requirements are defined for different combinations of TDL channels and FRC. We propose to have the same approach for NR NTN. Different combinations of NTN-TDL channels and FRC can be defined as it is done for Rel-15/16. The corresponding proposal is made for the whole Table 2-2 below

Delay spread

Study on the delay spread in NTN systems can be found in [2] and [3]. However, there is no DS values recommended for link-level simulation as in TR38.901. In [2] and [3] the DS distributions for different elevation angles and different scenarios are present. We propose not to consider different scenarios and elevations but define short, nominal and long delay spread instead.

Proposal 3: RAN4 will not consider different scenarios and elevations for delay spread but define short, nominal and long delay spread instead.

Based on the above-mentioned distributions we propose the following values for delay spread:

Table 2-1 Scaling parameters for TDL channel model

| Model | DS |
| --- | --- |
| Short delay spread | 10 ns |
| Nominal delay spread | 50 ns |
| Long delay spread | 150 ns |

Proposal 4: RAN4 to consider the DS values for requirements definition as shown in Table 2-1

Doppler shift

RAN1 made a conclusion that DL frequency compensation by gNB for the service link Doppler is not supported in Release 17. That means that unlike UL here we need to consider significant Doppler shift due to satellite motion. Moreover, the angle between the velocity vector and the direction of propagation of the signal is not constant which leads to Doppler shift evolution. Similar issue was studied during for High Speed Train WI, where to address the Doppler variation, special HST Doppler model was designed.

Proposal 5: RAN4 to decide whether special Doppler model need to be designed to be used for NTN PDSCH requirements definition.

PDSCH requirements

Based on the above discussion the test parameters from Table 2-2 are proposed as the starting point for PDSCH performance requirements definition.

Table 2-2 Minimum performance for NTN PDSCH

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDL-C [+NTN Doppler model] | 2x2, ULA Low | 70 | [TBD] |
| 1-2 | R.PDSCH.2-2.1 FDD | 20 / 30 | 16QAM, 0.48 | NTN-TDL-A [+NTN Doppler model] | 2x2, ULA Low | 70 | [TBD] |

Proposal 6: The test parameters from Table 2-2 are proposed as the starting point for PDSCH performance requirements definition.

# Conclusion

In this paper we discussed the identified open issues for the UE performance requirements. The following proposals were made:

Proposal 1: RAN4 to define new requirements only for PDSCH

Proposal 2: For PDSCH requirements with disabled HARQ processes RAN4 to consider half of HARQ processes disabled and half HARQ processes enabled.

Proposal 3: RAN4 will not consider different scenarios and elevations for delay spread but define short, nominal and long delay spread instead.

Proposal 4: RAN4 to consider the DS values for requirements definition as shown in Table 2-1

Table 2-1 Scaling parameters for TDL channel model

| Model | DS |
| --- | --- |
| Short delay spread | 10 ns |
| Nominal delay spread | 50 ns |
| Long delay spread | 150 ns |

Proposal 5: RAN4 to decide whether special Doppler model need to be designed to be used for NTN PDSCH requirements definition.

Proposal 6: The test parameters from Table 2-2 are proposed as the starting point for PDSCH performance requirements definition.

Table 2-2 Minimum performance for NTN PDSCH

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDL-C [+NTN Doppler model] | 2x2, ULA Low | 70 | [TBD] |
| 1-2 | R.PDSCH.2-2.1 FDD | 20 / 30 | 16QAM, 0.48 | NTN-TDL-A [+NTN Doppler model] | 2x2, ULA Low | 70 | [TBD] |

# References

- R4-2203042, WF on general and NTN UE demodulation requirements, Qualcomm Inc., RAN4 #101-bis-e, January 17th – 25th, 2022

- R1-1807164, NR-NTN Channel Modeling - Flat fading criteria, Thales RAN1 #93 May 21st  –  25th, 2018

- 3GPP TR 38.811