3GPP TSG-RAN WG4 Meeting #102-e 												R4-2206368

Electronic Meeting, 21 February – 3 March 2022

Agenda item:			9.2

Source:	Apple Inc.

Title:	WF on introduction of the full unlicensed band

Document for:	Information

# Introduction

3GPP Rel-16 NR-U WI specified how the NR technology can be used on the unlicensed spectrum thus offering more resources in frequency bands, such as 5GHz and 6GHz.  5GHz is a well-known band for the unlicensed operation, but 6GHz is a relative new band usage of which was approved recently in different regulatory regions. While the 6GHz band for the US is already part of the Rel-16 core functionality, current 3GPP specifications do not support it for other countries, such as South Korea and Canada, which have finalised their regulatory requirements only recently. Thus, RAN#92 meeting approved a new WI aim of which is to enable support of the 6GHz unlicensed band for those countries and regions that have finalised recently the corresponding regulatory requirements.

This document presents a way forward based on the agreements made during the [102-e][106] NR_6GHz_unlic_full email discussion (RAN4#102 meeting).

# Way forward

## Summary of the NS values

No updates during this meeting.

## Band n96 and its applicability

As per the agreement from the RAN4#100 meeting, band n96 can be used not only in US, but also in other countries where the full frequency band is used for unlicensed operation. Thus, it was suggested to revise the existing band n96 NOTE so that it does not limit band n96 applicability only to US. As a related aspect, it was also discussed starting from which release the corresponding NOTE should be changed.

The following agreement was reached during the RAN4#102 meeting.

Agreements:

-	The NOTE 14 is kept but the wording is changed to "This band is applicable only in countries/regions designating this band for shared-spectrum access use subject to country-specific conditions ".

-	The same change is applied to band n96 in Rel-16 and Rel-17.

MODERATOR NOTE:

-	Same wording applies also the lower 6GHz unlicensed band n102.

-	This WI will change only Rel-17 NOTE of band n96, while the corresponding change for Rel-16 should be done with a separate CR.

## UE RF aspects

Agreements:

-	For LPI:

-	Content of the running CR is agreed (R4-2206370)

-	For VLP:

-	PC5 VLP A-MPR values for South Korea are agreed to TR 38.849 (R4-2206369)

## BS RF aspects